# 3GPP TSG-RAN WG4 Meeting #114	R4-2501354

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:			NTN Ku bands –WI update and related impacts

Agenda item:		7.11.1

Document for:		Approval

# Introduction

In last RAN#106, the NTN KU-band WI was revised ([1]) and captured the agreement that NTN Ku-bands shall be defined for both FR1-NTN and FR2-NTN.

This contribution is analyzing the impact of those updates.

# Discussion

## WI updates

### Scope confirmation

The last WI update clarified that US should be considered as well:

Observation1: FCC regulation should be studied to check if any regulatory requirement should be added to TS 38.108 for the NTN Ku-bands.

### Additional channel bandwidths

The following FR1-NTN channel bandwidths were already listed in the first WI: 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz.

The following channel bandwidths have been added in the last WI update: 10 MHz and 25 MHz.

The current version of the SAN RF specification only supports the following channel bandwidths: 5 MHz, 10 MHz, 15 MHz, 20 MHz. 30 MHz was only added in the “operating bands and channel arrangement” subclauses, but no RF requirement was specified for this channel bandwidth.

Proposal1: According to the last WI, system and RF requirements should be added for the following channel bandwidths: 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz, all new channel bandwidths supporting 30kHz SCS only.

We made proposals for the corresponding system parameters in [2] and discussed the consequences on SAN RF requirements and NTN UE RF requirements in [3] and [4] respectively.

### FR1-NTN and FR2-NTN

#### Background

Another major update of the WI is the fact that NTN Ku-bands shall be specified for both numerologies, FR1-NTN and FR2-NTN

Observation2: RAN confirmed that any new NTN Ku-band shall be specified in both FR1-NTN and FR2-NTN.

#### RF requirements

The following contributions provide a detailed analysis of those impacts:

- System impacts are described in [2].

- SAN RF impacts are described in [3].

- NTN VSAT RF impacts are described in [4].

### Mobile VSAT and NGSO

The support for mobile VSAT served by NGSO satellite has been added to the scope of the updated WI:

It won’t be possible to simply update the definition of NTN VSAT types 4 and 5 by adding NGSO. We are making some proposals in our contribution ([4]) related to NTN VSAT, further discussion will be needed to converge on a way forward.

Consequently, new ECC Decisions and ETSI EN specifications would need to be analyzed when introducing the NTN VSAT RF requirements for the NTN Ku-bands, more details could be found in our other contribution [4].

Also, this WI update impacts the scope of the coexistence, adding new scenarios, see our contribution on coexistence [5].

Observation3: Aligned with the last WI updates, Mobile VSAT operating with NGSO satellite is now included in the scope of the specification of the NTN Ku-bands.

### NTN VSAT types 1, 2 and 3

According to the updated WI, NTN VSAT types 1, 2 and 3 are not anymore in the scope, meaning that those NTN types won’t be supported for NTN Ku-bands in TS 38.101-5:

Observation4: Aligned with the last WI updates, NTN VSAT types 1, 2 and 3 (fixed VSAT) will not be supported in TS 38.110-5 for the NTN Ku-bands.

Therefore, some ECC Decisions and ETSI EN specifications don’t need to be further analyzed when introducing the NTN VSAT RF requirements for the NTN Ku-bands, more details could be found in our other contribution [4].

Another consequence is that scenarios with fixed VSAT should be removed from the scope of the coexistence study, see [5].

Finally, the initially agreed bands definition would require further discussion if only NTN Mobile VSAT will be supported by TS 38.108 and TS 38.101-5. See our other contribution ([2]) on system parameters for more details.

# Conclusion

In this contribution, we analyzed the impacts of the latest updates of the NTN Ku-bands WI. We made the following observations:

Observation1: FCC regulation should be studied to check if any regulatory requirement should be added to TS 38.108 for the NTN Ku-bands.

Proposal1: According to the last WI, system and RF requirements should be added for the following channel bandwidths: 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz, all new channel bandwidths supporting 30kHz SCS only.

Observation2: RAN confirmed that any new NTN Ku-band shall be specified in both FR1-NTN and FR2-NTN.

Observation3: Aligned with the last WI updates, Mobile VSAT operating with NGSO satellite is now included in the scope of the specification of the NTN Ku-bands.

Observation4: Aligned with the last WI updates, NTN VSAT types 1, 2 and 3 (fixed VSAT) will not be supported in TS 38.110-5 for the NTN Ku-bands.

# References

- RP-243314, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2501355, NTN Ku bands – System parameters, Ericsson

- R4-2501357, NTN Ku bands – SAN RF, Ericsson

- R4-2501356, NTN Ku bands – UE RF requirements, Ericsson

- R4-2501398, NTN Ku bands – On Co-existence simulation assumptions for NR NTN Ku band, Ericsson