# 3GPP TSG-RAN WG4 Meeting #114	R4-2501357

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:			NTN Ku bands SAN RF

Agenda item:		7.11.5

Document for:		Approval

# Introduction

In last RAN#106, the NTN KU-band WI was revised ([1]) and captured the agreement that NTN Ku-bands shall be defined for both FR1-NTN and FR2-NTN.

Based on our other contribution analyzing the WI update impacts [2], this contribution is studying the relevant system parameters for the NTN Ku-bands considering previous RAN4 agreements as well.

# Discussion

## Additional channel bandwidths

As highlighted in our other contribution evaluating the last update of the WI and its impacts ([2]), 5 new channel bandwidths shall be specified in FR1-NTN (25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz).

In the following table, we are analyzing the related impacts on FR1-NTN requirements.

Proposal1: Update TS 38.108 according to Table 1 when adding the new channel bandwidths (30kHz SCS only) mentioned in the NTN Ku-bands WI.

Table 1: TS 38.108 impacts when adding the new NTN Ku-bands channel bandwidths

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 6. Conducted transmitter requirements |  |  |
| 6.1 General | No |  |
| 6.2 Base station output power | No |  |
| 6.3 Output power dynamics | Yes | Requirements for the new channel BWs and 30kHz SCS shall be added for the total power dynamic range |
| 6.4 Transmit ON/OFF | NA |  |
| 6.5 Transmitted signal quality | No |  |
| 6.6 Unwanted emissions |  |  |
| 6.6.1 General | No |  |
| 6.6.2 Occupied bandwidth | No |  |
| 6.6.3 ACLR | Yes | The new channel BWs shall be added to the ACLR tables |
| 6.6.4 Operating Band Unwanted Emissions | No |  |
| 6.6.5 Transmitter spurious emission | No |  |
| 6.7 Transmitter intermodulation | NA |  |
| 7. Conducted receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Receiver sensitivity level | Yes | Requirements for the new channel BWs and 30kHz SCS shall be added |
| 7.3 Dynamic range | Yes | Requirements for the new channel BWs and 30kHz SCS shall be added (if relevant to be specified for the NTN Ku-bands) |
| 7.4 In-band selectivity and blocking | Yes | The new channel BWs shall be added to the ACS tables |
| 7.5 Out of band blocking | No |  |
| 7.6 Receiver spurious emissions | NA |  |
| 7.7 Receiver intermodulation | NA |  |
| 7.8 In channel selectivity | Yes | Requirements for the new channel BWs and 30kHz SCS shall be added |
| 9. Radiated transmitter requirements |  |  |
| 9.1 General | No |  |
| 9.2 Radiated transmit power | No |  |
| 9.3 OTA SAN output power | No |  |
| 9.4 OTA output power dynamics | No | FR1-NTN requirements are referencing to the conducted requirements. No new channel BW is added to FR2-NTN. |
| 9.5 OTA transmit ON/OFF power | NA |  |
| 9.6 OTA transmitted signal quality | No |  |
| 9.7 OTA unwanted emissions |  |  |
| 9.7.1 General | No |  |
| 9.7.2 OTA occupied bandwidth | No |  |
| 9.7.3 OTA ACLR | No | FR1-NTN requirements are referencing to the conducted requirements. No new channel BW is added to FR2-NTN. |
| 9.7.4 OTA out-of-band emissions | No |  |
| 9.7.5 OTA transmitter spurious emissions | No |  |
| 9.8 OTA transmitter intermodulation | NA |  |
| 10.1 General | No |  |
| 10.2 OTA sensitivity | No |  |
| 10.3 OTA reference sensitivity level | Yes | Requirements for the new FR1-NTN channel BWs and 30kHz SCS shall be added. |
| 10.4 OTA dynamic range | Yes | Requirements for the new FR1-NTN channel BWs and 30kHz SCS shall be added (if relevant to be specified for the NTN Ku-bands). |
| 10.5 OTA in-band selectivity and blocking | Yes | The FR1-NTN new channel BWs shall be added to the ACS tables |
| 10.6 OTA out-of-band blocking | No |  |
| 10.7 OTA receiver spurious emissions | NA |  |
| 10.8 OTA receiver intermodulation | NA |  |
| 10.9 OTA in-channel selectivity | Yes | The FR1-NTN new channel BWs shall be added to the ACS tables |

## SAN OTA requirements, FR1-NTN and FR2-NTN

Note that the related system impacts are described in our other contribution [2].

In the following Table 2, answering the WI objective to “if possible, specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN)”, we compare the FR1-NTN requirement with the FR2-NTN one for each OTA RF requirement. We made the following recommendations to align as much as possible FR1-NTN and FR2-NTN requirements for the NTN Ku-bands.

Proposal2: The tolerance for the radiated transmit power and SAN output power shall be the same for both FR1-NTN and FR2-NTN.

Observation1: The same ACLR requirement for NTN Ku-bands in FR1-NTN and FR2-NTN will be specified based on the conclusion of the coexistence study.

Proposal3: The spurious frequency range for NTN Ku-bands shall be identical when the bands belong to FR1-NTN and FR2-NTN.

Proposal4: RAN4 should discuss how to align the specification of REFSENS, ACS (based on coexistence study conclusion), out of band blocking and in-channel selectivity requirements.

Also, considering that RE power control and Rx dynamic range requirements are not specified for FR2-NTN cand that OTA:

Proposal5: RAN4 should discuss if RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands.

Table 2: Comparison of 1-O and 2-O requirements in the scope of the NTN Ku-band

| Requirements | Same FR1-NTN and FR2-NTN | Comments |
| --- | --- | --- |
| 9. Radiated transmitter requirements |  |  |
| 9.1 General |  |  |
| 9.2 Radiated transmit power | No | The tolerance on EIRP level declaration is not the same, it should be aligned for the NTN Ku-band. |
| 9.3 OTA SAN output power | No | The tolerance on EIRP level declaration is not the same, it should be aligned for the NTN Ku-band. |
| 9.4 OTA output power dynamics |  |  |
| 9.4.1 General |  |  |
| 9.4.2 OTA RE power control dynamic range | No | The requirement doesn’t exist for FR2-NTN. |
| 9.4.3 OTA total power dynamic range | No | The requirement is channel BW and SCS specific. No alignment is foreseen for this requirement. |
| 9.5 OTA transmit ON/OFF power | NA |  |
| 9.6 OTA transmitted signal quality |  |  |
| 9.6.1 OTA frequency error | Yes |  |
| 9.6.2 OTA modulation quality | Yes |  |
| 9.6.3 OTA alignment error | NA |  |
| 9.7 OTA unwanted emissions |  |  |
| 9.7.1 General |  |  |
| 9.7.2 OTA occupied bandwidth | Yes |  |
| 9.7.3 OTA ACLR | No | Based on the coexistence study, the same ACLR requirement shall be specified for all NTN Ku-bands |
| 9.7.4 OTA out-of-band emissions | Yes |  |
| 9.7.5 OTA transmitter spurious emissions | No | The spurious frequency range is different, but it shall be the same for all NTN Ku-bands. |
| 9.8 OTA transmitter intermodulation | NA |  |
| 10. Radiated receiver characteristics |  |  |
| 10.1 General |  |  |
| 10.2 OTA sensitivity | No | There is no OTA sensitivity for SAN type 2-O. |
| 10.3 OTA reference sensitivity level | No | The requirement is channel BW and SCS specific. Still, as the approach is different for 1-O and 2-O, further discussion might be needed to be consistent. |
| 10.4 OTA dynamic range | No | There is no OTA sensitivity for SAN type 2-O. |
| 10.5 OTA in-band selectivity and blocking | No | The requirement is channel BW and SCS specific. Still, as the approach is different for 1-O and 2-O, further discussion might be needed to be consistent. |
| 10.6 OTA out-of-band blocking | No |  |
| 10.7 OTA receiver spurious emissions | NA |  |
| 10.8 OTA receiver intermodulation | NA |  |
| 10.9 OTA in-channel selectivity | No | The requirement is channel BW and SCS specific. Still, as the approach is different for 1-O and 2-O, further discussion might be needed to be consistent. |

## Regulations

### FCC

47 CFR 25.208 specifies PFD limits in the bands 10.95-11.2 GHz and 11.45-11.7 GHz for GSO and 10.7-11.7 GHz for NGSO

Also, in the band 12.2-12.7 GHz, additional pfd limit is specified for NGSO, at the specified low angle.

Observation2: FCC specified pfd limits in the considered NTN Ku-bands.

Proposal5: RAN4 should discuss how to consider the pfd limits specified by FCC (47 CFR 25.208) for the Ku-band.

# Conclusion

In this contribution, we analyzed the SAN RF specification TS 38.108 for the new NTN Ku-bands considering the last update of the WID. We made the following observations and proposals:

Proposal1: Update TS 38.108 according to Table 1 when adding the new channel bandwidths (30kHz SCS only) mentioned in the NTN Ku-bands WI.

Proposal2: The tolerance for the radiated transmit power and SAN output power shall be the same for both FR1-NTN and FR2-NTN.

Observation1: The same ACLR requirement for NTN Ku-bands in FR1-NTN and FR2-NTN will be specified based on the conclusion of the coexistence study.

Proposal3: The spurious frequency range for NTN Ku-bands shall be identical when the bands belong to FR1-NTN and FR2-NTN.

Proposal4: RAN4 should discuss how to align the specification of REFSENS, ACS (based on coexistence study conclusion), out of band blocking and in-channel selectivity requirements.

Proposal5: RAN4 should discuss if RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands.

Observation2: FCC specified pfd limits in the considered NTN Ku-bands.

Proposal5: RAN4 should discuss how to consider the pfd limits specified by FCC (47 CFR 25.208) for the Ku-band.

# References

- RP-243314, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2501354, NTN Ku bands - WI update impacts, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat