3GPP TSG-RAN WG4 Meeting #114	R4-2501377

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	7.26.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

# Topic #1: General and system parameters ### Sub-topic 1-1 General Issue 1-1-3: General issue on how to introduce LP-WUS feature into core specs 38.101-x Agreements: - Add a new suffix “M” as “LP-WUS/WUR” Issue 1-1-4: views on which requirements should be updated in core spec Agreements: - The part should be updated for LP-WUS/WUR in spec. - The additional requirements are not precluded, if needed. Sections or requirements; Needed or not?. Operation bands; FFS. UE channel bandwidth; FFS. Channel raster; FFS. REFSENS; YES. Maximum input level; YES. ACS; YES. ASCS; YES. In-band blocking; YES. Out-of-band blocking; YES. Narrow band blocking; YES. Spurious response; FFS. Wide band Intermodulation; FFS. Spurious emissions; FFS. Annex A Measurement channels; YES. ### Sub-topic 1-2 System parameters Issue 1-2-1: Channel raster for LP-WUR Agreements: - No new channel raster is introduced dedicated for LP-WUS/WUR. - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

Discussion

Channel raster for LP-WUR

In the last meeting, it was agreed that no new channel raster will be introduced for LP-WUS/WUR. However, there still was a question that whether there is a need for a limited set of frequencies supported by LP-WUS in order to reduce the complexity. In our opinion this is not required. LP-WUS and LP-SS will be configured on common resource grid based on the SCS applied. The LP-WUR will be told the exact frequency resource where LP-WUS message will be transmitted by the gNB. LP-WUR does not have to do any frequency search, and thus it is not clear that limiting the set of frequencies in a band would reduce the LP-WUR complexity and cost. Further, limiting the possible location of LP-WUS will bring scheduling restrictions, which we deem to be unwarranted for.

LP-WUR will know the frequency resource over which LP-WUR will be transmitted by the gNB.

Restricting the location of LP-WUS will bring scheduling restrictions for gNB.

No need for limited set of frequencies in a band to be supported by LP-WUS.

No changes required in the channel raster section of the UE RF specifications.

Channel bandwidth

In previous meeting [2], it was agreed that there is no need to define the BW for LP-WUR (agreement reproduced for clarification). Given the RAN1 design for LP-WUS is 11 PRBs irrespective of SCS, there is no need of reproducing that in the RAN4 specifications. Thus, we propose that there is no need to update the UE channel bandwidth section in the UE RF specification.

Issue 1-2-5: Whether need to define CBW for LP-WUR? Agreements: - No need to define BW for LP-WUR

No changes required in the channel bandwidth section of the UE RF specification.

Conclusion

The following Observations and Proposals were made:

Observation 1: LP-WUR will know the frequency resource over which LP-WUR will be transmitted by the gNB.

Observation 2: Restricting the location of LP-WUS will bring scheduling restrictions for gNB.

Proposal 1: No need for limited set of frequencies in a band to be supported by LP-WUS.

Proposal 2: No changes required in the channel raster section of the UE RF specifications.

Proposal 3: No changes required in the channel bandwidth section of the UE RF specification.

- References

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R4-2414309, WF on LP-WUS UE RF requirements, Vivo, RAN4#112, Maastricht, Netherlands, 19th – 23nd Aug, 2024.