3GPP TSG-RAN WG4 Meeting #114	R4-2501396

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia, Anterix

Title:	Support of 3MHz channel bandwidth for (e)RedCap devices

Agenda item:	5.33.1

Document for:	Discussion

Introduction

In the last RAN plenary meeting, Anterix raised the request for supporting 3 MHz channel bandwidth for (e)RedCap devices. In their contribution, they outlined the justification for the request [1]. In RAN#106 no new WI for this request was agreed. However, it was discussed that this work may be pursued in the TEI path.

In this document, we present our views on what technical changes in various specifications are required and the feasibility of this work in the TEI path.

Discussion

Additional channel bandwidth

The basic assumption is that 3 MHz will be the additional channel bandwidth supported by the (e)RedCap devices. In the current specification, for bandwidth restricted eRedCap devices, requirements are based on 5MHz channel bandwidth and that shall not change by the addition of 3 MHz channel bandwidth.

3 MHz channel bandwidth shall be introduced as an additional and optional channel bandwidth for (e)RedCap devices.

Current requirements for bandwidth restricted eRedCap devices will still utilize a maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH.

Required UE RF changes

Channel bandwidth for RedCap devices

In Section 5.3I, the specification explicitly states that 3MHz channel bandwidth shall not be supported in Rel18. This has to be modified to support 3 MHz.

One line deletion is required in section 5.3I.

Reference sensitivity for RedCap devices

RedCap UEs with two Rx antenna ports, utilize the REFSENS requirements defined for a normal UEs in section Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. For RedCap UEs with one Rx antenna port, RedCap specific ΔR1R in Table 7.3I.2-1 for the applicable operating bands. However, if the scope of this work is restricted to (e)RedCap devices with two Rx antenna port, then no modification will be required to Table 7.3I.2-1.

RedCap devices support HD-FDD mode. For HD-FDD operation, there is a separate per band REFSENS requirement and configuration. However, if in this case operation is restricted to FDD mode only, then no modification will be required

With scope limited to two Rx antenna ports and no HD-FDD mode, no changes to REFSENS section will be required.

In case of eRedCap devices with bandwidth restrictions, current requirements are defined for only channel bandwidth equal to and larger than 5 MHz. In case of 3 MHz, NRB is 15, since NRB_PR3 is defined as 25 RBs for 15 KHz SCS, this will yield RBlow as negative. To fix this, the easiest solution is to just add an additional line stating NRB_PR3 will be 15 in case of 3 MHz channel bandwidth.

Update the value to NRB_PR3 will be 15 in case of bandwidth limited eRedCap device.

Fixed reference channels

Due to bandwidth restrictions for one category of eRedCap devices, new FRCs were defined. For 3 MHz FRCs, the allocated PRBs and the data rate are within limits of what the reduced bandwidth eRedCap devices support, so no new eRedCap specific 3 MHz FRC is required. Further, as the desired band is n106, which is a FDD band, TDD FRCs are not needed.

No changes to FDD FRCs are required, existing NR 3MHz FRC can be used.

No new FRCs are required as existing NR 3MHz FRC can be used.

Required BS RF changes

The BS specification already supports 3 MHz channel bandwidth in the desired n106 band. Further, there is nothing specific to (e)RedCap devices in the BS specifications. Thus, with work scope limited to n106, there will not be any impact on the BS specifications from RF perspectives.

No changes are required to the RF requirements in BS specification.

Scope

The basic premise of this work is to bring support for 3 MHz channel bandwidth for low cost (e)RedCap devices in n106. Currently, (e)RedCap devices may support 1 RX and HD-FDD mode. While discussing the support for these capabilities, 3 MHz channel bandwidth was not taken into consideration. As explained, it may not be feasible to introduce support of 3 MHz channel bandwidth with 1 RX and HD-FDD under TEI. Therefore, a new WI should be considered for that. However, with a limited scope, the work is feasible under TEI.

With the scope restricted to band n106 with (e)RedCap devices with 2 RX only and no HD-FDD support, work can be finished under Rel-19 TEI.

Introduce 3 MHz channel bandwidth support for n106 for (e)RedCap devices with 2 RX only, and no HD-FDD support.

Agree to the accompanying UE RF CR (R4-2501395) [2].

(e)RedCap devices support of 3 MHz channel bandwidth with 1 RX and HD-FDD can be introduced in Rel-20.

Conclusion

The following Observations and Proposals were made:

Proposal 1: 3 MHz channel bandwidth shall be introduced as an additional and optional channel bandwidth for (e)RedCap devices.

Proposal 2: Current requirements for bandwidth restricted eRedCap devices will still utilize a maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH.

Observation 1: One line deletion is required in section 5.3I.

Observation 2: With scope limited to two Rx antenna ports and no HD-FDD mode, no changes to REFSENS section will be required.

Observation 3: Update the value to NRB_PR3 will be 15 in case of bandwidth limited eRedCap device.

Observation 4: No new FRCs are required as existing NR 3MHz FRC can be used.

Observation 5: No changes are required to the RF requirements in BS specification.

Observation 6: With the scope restricted to band n106 with (e)RedCap devices with 2 RX only and no HD-FDD support, work can be finished under Rel-19 TEI.

Proposal 3: Introduce 3 MHz channel bandwidth support for n106 for (e)RedCap devices with 2 RX only, and no HD-FDD support.

Proposal 4: Agree to the accompanying UE RF CR (R4-2501395) [2].

Observation 7: (e)RedCap devices support of 3 MHz channel bandwidth with 1 RX and HD-FDD can be introduced in Rel-20.

- References

- RP-242812, Motivation for new WI on less than 5 Mhz (e)RedCap Support in Rel-19, Anterix, RAN#106, Madrid, Spain, Dec 9 – Dec 12, 2024.

- R4-2501395, Support of 3MHz channel bandwidth for RedCap devices, RAN4#114, Athens, Greece, Feb 17 – Feb 21, 2025.