3GPP TSG-RAN WG4 Meeting #114	R4-2501405

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On NR MIMO Phase 5 UEIBM requirements

Agenda item:	7.22.3.1

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the UE-initiated beam management feature.

Issue 1-4: Reporting delay requirements

<Way forward >

- Option 1: (Qualcomm)

  - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0;

- Option 2: (Nokia)

  - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

- Option 3: (Xiaomi)

  - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

- Option 4: (Samsung)

  - Need further RAN1 progress for RSs relationship between current beam and new beams.

- Option 5: (CMCC)

  - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on

- Option 6: (Apple)

  - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay

- Option 7: ( MediaTek)

  - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS

  - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period

- Option 8: (Ericsson)

  - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel

  - RAN4 to discuss components and criteria for defining TTime to first UL channel.

Discussion on reporting delay requirements

In the last RAN4 meeting, some companies have proposed to exclude the time the UE waits for the UL resources to send the first PUCCH. It may be recalled that the starting and ending point of the delay as well as the applicability of the requirements to both modes A and B and all supported events was agreed in RAN4# 112bis. We see no need to revisit this agreement.

However, we agree that it will be challenging to quantify the delay uncertainty between the first PUCCH and the 2nd  uplink channel. Hence, we propose that the reporting delay shall in totality be the time from when an event that triggers the measurement report occurs to the time instance when the UE sends the first PUCCH at the first available PUCCH occasion after the event that triggered the measurement report has occurred. The UE may hence send the PUCCH at any occasion from the moment the reporting condition becomes fulfilled and the first PUCCH after the event evaluation delay component, which may be defined based on the existing L1 measurement delay.

The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. the maximum time it can take the UE to realize the condition is fulfilled).

Regarding the definition of the event evaluation delay for event 2, RAN4 spent a lot of time discussing whether this delay needs to consider only the candidate beam or both current (serving) and candidate beams. The related agreement is given below.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

As the condition may become fulfilled at any point while the UE is measuring, the UE may realize the event 2 condition is met based on the next candidate beam measurement, based on the next serving beam measurement, or in the worst case, the UE would need to measure both beams to be able to realize the condition is met. Since RAN4 is defining minimum requirements, the worst case scenario needs to be taken into account. Hence, the maximum time it may take for the UE to realize the condition for UEIBM reporting is met is the maximum of L1 measurement delay for serving and candidate beam the report considers.

For events involving serving and candidate beam, maximum L1 event evaluation delay is the maximum of L1 measurement period for serving beam and candidate beam that the event considers.

Discussion on TCI switching requirements

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

  - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Option 2: (vivo)

  - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- Option 3: (ZTE)

  - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Samsung)

  - No unknown case is applied if the TCI states switching is based on UEIBM.

- Option 3: (ZTE)

  - Need to decide whether to define L1-RSRP requirements for unknown cell

In the last RAN4 meeting, the following was left as open issue for discussion [1]:

In case of a TCI state being considered as known or unknown, with reference to the RAN1 agreements in UEIBM so far, we see no impact on the existing conditions which determine if a TCI state can be considered as known or unknown.

Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

In the last RAN1#119 meeting various alternatives about beam application latency reduction after a UEI beam report is sent were discussed and agreed to be studied.

Agreement (RAN1 #119 Orlando)

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

- after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

- update TCI state(s) with the reported new beam(s)

- activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

- Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

- FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

- In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Other alternatives are not precluded

If some of these alternatives are agreed in RAN1, these could have a potential impact on the RAN4 TCI state switching delay requirements. Hence, we propose to wait for RAN1 progress on this issue before defining the TCI state switching delay requirements in RAN4.

RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay.

RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining TCI state switching delay requirements.

Discussion on other measurement requirements

Issue 1-5: Whether to update L1 report which is measured in L1 report?

<Way forward >

- Option 1: (Xiaomi)

  - It needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Option 2: (vivo)

  - RAN4 to specify the requirements to consider the condition which will trigger the event is remained fixed during the time UE performs L1 measurements.

  - Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

Issue 1-6: Conditions when UE performs L1 measurements for evaluating the event?

<Way forward >

- Option 1: (vivo)

  - No change on the indicated TCI state

  - No change on the to-be-measured RS of the new beam.

In the last RAN4 meeting, the following was left as open issue for discussion regarding L1 report [1]:

The issue about UE behaviour while waiting to send the UEI report after sending the 1st PUCCH when the conditions that originally caused the UEI event changes, could be a likely scenario in the network and needs to be discussed. However, we believe that this needs to be discussed and decided in RAN1.

UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1.

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM

There are currently no baseline RAN4 requirements defined for inter-cell CSI-RS based L1-RSRP measurements for beam management. Hence, unless a clear use case is identified we propose not to introduce CSI-RS based L1-RSRP requirements for inter-cell beam management.

No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management within the Rel-19 WI.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

RAN4 has defined measurement period requirements to compute to compute L1-RSRP to be reported in the baseline periodic/semi-persistent/aperiodic reporting. In the case of FR2, beam sweeping factor is one of the parameters which is taken into account while calculating the total measurements period for L1-RSRP.

In the FR2-1 standard, the baseline beam sweeping factor (N) is set to 8. The main objective of UE-initiated beam management (UEIBM) is to enable faster beam switching. However, when defining RAN4 RRM requirements for UEIBM, if N is still assumed to be 8, the resulting measurement delay will be significant, which as a consequence will not fulfill one of objectives of the UEIBM feature which is to reduce latency

Rel-18 RRM requirements for multi-Rx support smaller beam sweeping factors (N=2, 4, 6) based on UE capabilities, leading to shorter measurement delays. Therefore, it's logical to allow UEs with faster beam sweeping capabilities to utilize this capability for L1-RSRP measurements during UEIBM. Hence, when defining requirements for event-triggered L1 beam reporting, the beam sweeping factor (N) should be determined based on the UE's capabilities.

Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Apple)

  - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 3: (Ericsson)

  - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP.

- Option 4: (MediaTek)

  - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Issue 1-12: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Samsung, Apple)

  - Do not combine capabilities across different L1 measurements.

- Option 3: (Ericsson)

  - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

- Option 4: (MediaTek)

  - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria? And Issue 1-12: Compatibility of capabilities with LTM

RAN1 is currently discussing the UE capability for the number of events to be supported for UEIBM. RAN4 should wait for RAN1 agreements before deciding how to define related requirements. Considering coordination between MIMO UEIBM and LTM mobility L1 events, we think it is better to keep the discussion and requirements independent, as these are two separate parallel work items.

RAN1 is discussing UE capability about the number of events supported for UEIBM.

Do not combine capabilities across different L1 measurements (MIMO and LTM).

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

Currently, L1-RSRP has been agreed as the baseline quality metric for trigger events, although other quality metrics like L1-SINR are under discussion in RAN1. However, such measurements can be performed by the UE as done in legacy, so there is no need for RAN4 to define new measurement accuracy requirements specific for UEIBM.

The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

Conclusion

In this paper, the following observations and proposals were made:

Proposal 1: The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (i.e. the maximum time it can take the UE to realize the condition is fulfilled).

Proposal 2: For events involving serving and candidate beam, maximum L1 event evaluation delay is the maximum of L1 measurement period for serving beam and candidate beam that the event considers.

Proposal 3: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Observation 1: RAN1 is discussing various alternatives to reduce the beam application latency, some of which may have an impact on the TCI state switch delay.

Proposal 4: RAN4 to wait for RAN1 progress on discussion of beam application latency reduction after a UEI beam report is sent before defining TCI state switching delay requirements.

Proposal 5: UE behaviour in the interim between sending the 1st PUCCH and waiting for UL resources to send the UEI report is up to RAN1.

Observation 2: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 6: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management within the Rel-19 WI.

Proposal 7: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

Observation 3: RAN1 is discussing UE capability about the number of events supported for UEIBM.

Proposal 8: Do not combine capabilities across different L1 measurements (MIMO and LTM).

Proposal 9: The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- References

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4#112bis, Orlando, US, 18th ‒ 22nd November 2024

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management