3GPP TSG-RAN WG4 Meeting # 114															R4-2401409

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.22.2

Source:	Samsung

Title:	Progress on UE RF requirements of NR MIMO Phase 5

Document for:	Discussion

1	Introduction

RAN4 has discussed UE RF aspects of Rel-19 NR MIMO Phase 5 focusing on the objective of 3-antenna-port transmissions in the WID [1]. Thanks to the companies who have contributed until the last meeting, RAN4 has achieved considerable progress as captured in the latest WF [2]. RAN4 also received a reply LS from RAN1, which would help to resolve some remaining issues on the table [3]. In this contribution, we would like to check what we have agreed so far and see if more potential solutions for the rest of the issues.

2	Discussion

2.1	Power class

The applicable power class for 3Tx, legacy power classes such as PC3/2/1.5 were discussed, and RAN4 has done most of power class by allowing PC3 and PC2 except PC1.5 for full power mode 0. It is also necessary to discuss PC1.5 further for power scaling mode (3-layer) with its applicable UE type as below [2].

<Agreement>: PC for 3Tx requirement - 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19 <Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

Observation 1:	RAN4 agreed to introduce PC3 and PC2 for 3Tx, but not for PC1.5 in case of full power mode 0.

As discussed in past meetings, PC1.5 cannot be supported at current level of maximum single PA output, e.g., 26dBm, in consideration of the mode 0. This is because, in our understanding, mode 0 also means to support a target power class with a single full rated port, which is also confirmed by the reply LS from RAN1 [3]. Moreover, as a common understanding agreed in multiple WFs before, the WI does not consider Tx diversity, which would mean that the power scaling mode with 3-port antenna transmission for PC1.5 could not be in the scope of the WI.

Observation 2:	WI does not consider Tx diversity, which would mean that the power scaling mode with 3-port antenna transmission for PC1.5 could not be in the scope of the WI.

Therefore, given the two limitation such as full power mode 0 and Tx diversity, RAN4 should focus on PC3 and PC2, but exclude PC1.5 from Rel-19 3Tx discussion.

Proposal 1:		RAN4 should focus on PC3 and PC2, but exclude PC1.5 from Rel-19 3Tx discussion.

2.2	MPR and PA configurations

Regarding maximum power reduction (MPR) for 3Tx, RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2 as copied in Table 1, Table 2 and Table 3 although it is not yet clear which MPR can be referred to for 3Tx PC2 between Table 2 (1Tx) and Table 3 (2Tx). On the other hand, for PC1.5, since it still has a dependency on the previous power class discussion, RAN4 could not have an explicit way forward yet. The PC1.5 MPR could be determined if it use the same MPR as PC1.5 with 2Tx in case PC1.5 can be introduced for 3Tx as below [2].

<Agreement>: PC3 for 3Tx - Use the same MPR as MPR for PC3 single carrier. <Agreement>: PC2 for 3Tx - Use the same MPR as MPR for PC2 single carrier. - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied <Way forward>: FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx.

Observation 3:	RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2.

Observation 4:	PC1.5 MPR could be determined if it use the same MPR as PC1.5 with 2Tx in case PC1.5 can be introduced for 3Tx.

Table 1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostTSRel18] and if the IE [powerBoostPi2BPSKRel18] is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostTSRel18] and if the IE [powerBoostQPSKRel18] is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 2: Maximum power reduction (MPR) for power class 2 (1Tx)

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 3: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |
| NOTE 1: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost NOTE 2: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost |  |  |  |  |

In comparison between 1Tx (Table 2) and 2Tx (Table 3) MPR for PC2 above, it is shown that the gap is not that considerable even for the higher modulation. In that case, our preference for 3Tx PC2 MPR would be to have Table 3 based on PC2 with 2Tx as multi-Tx MPR should be more reasonable for the 3Tx scenario.

Proposal 2:		3Tx PC2 MPR would be to have Table 3 based on PC2 with 2Tx.

2.3	MIIMO layers

For the MIMO layer discussion for full power mode 0, the agreement was reached to use 1-layer only as the performance verification purpose for 3Tx in the last meeting.

<Agreement>: Supported layer for mode 0 - For the performance verification of 3Tx, use 1-layer only for mode 0.

Observation 5:	For 3Tx full power mode 0, only 1-layer is agreed for the performance verification purpose.

With that, RAN4 is able to add one more row for 3-port transmission into the full power mode 0 of PUSCH configuration in Table 6.2D.1-3 of TS 38.101-1 with W=13100,13010,13001 as below.

Table 4: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  |  | 1 | 3 | [TBD] |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |

Proposal 3:		Table 4 should be considered for PUSCH Configuration for uplink full power transmission with 3-port transmission in TS 38.101-1.

2.4	Asymmetric TRPs

For the asymmetric DL sTRP/UL mTRP, RAN4 also had discussed whether it has impact on the UE RF requirement in Rel-19, and found the scenario that UL only TRP can transmit SSB is not in the scope of the WID. Although RAN4 sent the same question to RAN1, the answer was not changed from our previous understanding as below [3].

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Observation 6:	RAN1 confirms the scenario that UL only TRP can transmit SSB is not in the scope of the WID.

Since the basic motivation of the objective is that there is neither DL control information nor DL RS including pathloss RS (PL RS) from UL TRP hence scheduling/control information for UL TRP could be transmitted from TRP1 (i.e., both DL/UL capable TRP), the pathloss offset (PL offset) configuration should be considered to calculate PL RS from TRP1, which is defined as difference between two pathloss values where the 1st pathloss is between TRP1 and UE, and the 2nd pathloss is between TRP2 and UE, respectively.

If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE. Therefore, it is proposed for RAN4 not to discuss the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios in UE RF discussion.

Observation 7:	If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE.

Proposal 4:		It is proposed for RAN4 not to discuss the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios in UE RF discussion.

3	Conclusion

This contribution focuses on remaining open issues from the outcome of the last meeting [2]. Following observations and proposals were derived for consideration in this meeting.

Observation 1:	RAN4 agreed to introduce PC3 and PC2 for 3Tx, but not for PC1.5 in case of full power mode 0.

Observation 2:	WI does not consider Tx diversity, which would mean that the power scaling mode with 3-port antenna transmission for PC1.5 could not be in the scope of the WI.

Proposal 1:		RAN4 should focus on PC3 and PC2, but exclude PC1.5 from Rel-19 3Tx discussion.

Observation 3:	RAN4 has reached a consensus to reuse the single carrier MPR for each power class of 3Tx for PC3 and PC2.

Observation 4:	PC1.5 MPR could be determined if it use the same MPR as PC1.5 with 2Tx in case PC1.5 can be introduced for 3Tx.

Proposal 2:		3Tx PC2 MPR would be to have Table 3 based on PC2 with 2Tx.

Observation 5:	For 3Tx full power mode 0, only 1-layer is agreed for the performance verification purpose.

Proposal 3:		Table 4 should be considered for PUSCH Configuration for uplink full power transmission with 3-port transmission in TS 38.101-1.

Observation 6:	RAN1 confirms the scenario that UL only TRP can transmit SSB is not in the scope of the WID.

Observation 7:	If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE.

Proposal 4:		It is proposed for RAN4 not to discuss the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios in UE RF discussion.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2420400,	WF on UE RF requirements for MIMO enhancement, RAN4#113

[3] R4-2500005,	Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN4#114