3GPP TSG-RAN WG4 Meeting # 114	R4-2501449

# Athens, Greece, 17th – 21st February, 2025

Source:	Huawei, HiSilicon

Title:	TP for TR 37.862 on introduction of new channel bandwidths in band n80 and n84

Agenda item:	6.13.2

Document for:	Approval

1 Background

This contribution provides text proposal on the new channel bandwidth for SUL bands n80 and n84.

2 Text Proposal

---Start of changes---

1	Scope

This clause shall start on a new page.

The present document is a technical report on adding channel bandwidth in existing NR bands under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address the addition of channel bandwidth support in existing NR bands as listed in Table 1-1.

Table 1-1: Added channel bandwidth(s) in NR bands during Rel-19 time frame

| Band | Channel bandwidth and SCS |
| --- | --- |
| n48 | 50 MHz (15, 30 and 60kHz SCS) 60, 70, 80, 90, 100 MHz (30 and 60 kHz SCS) |
| n80 | 35, 45, 50 MHz (15, 30 and 60kHz SCS) |
| n84 | 45 MHz (15, 30 and 60kHz SCS) |

---Next of changes---

5.X	Band n80 – 35, 45, 50 MHz channel BWs.

5.X.1	System parameters

The 35, 45 and 50 MHz (15, 30 and 60kHz SCS) channel bandwidths are proposed for SUL band n80.

5.X.2	UE A-MPR

There is no bandwidth-specific AMPR requirements for SUL band n80.

5.X.3	UE REFSENS

No need to specify UE REFSENS for single SUL band.

5.X.4	Other

None

5.Y	Band n84 –45 MHz channel BW.

5.Y.1	System parameters

The 45 MHz (15, 30 and 60kHz SCS) channel bandwidth is proposed for SUL band n84.

5.Y.2	UE A-MPR

The bandwidth-specific AMPR requirements for band n1 can be reused for SUL band n84.

5.Y.3	UE REFSENS

No need to specify UE REFSENS for single SUL band.

5.Y.4	Other

None

---End of changes---

Reference

[1]	RP-241684, “New WID: Basket WID on adding channel bandwidth support to existing NR bands”, Ericsson