# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501453

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the Rel-19 PC2 FDD RedCap UE

Agenda Item:	7.7.1

Document for:	Approval

Introduction

In the RAN106 meeting, the WI [1] for NR PC2 RedCap UE was revised to include the FDD PC2 feature. There are two aspects highlighted here. Firstly, RAN4 may need to discuss the band agnostic way for FDD PC2 RedCap UE. Secondly, it’s noted that wearable device doesn’t support HPUE in FDD bands.

- For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item

In this paper, we’d like to discuss the spec impacts when RAN4 introduce FDD PC2 for RedCap UE.

Discussion

For the FDD PC2 RedCap UE, the following characteristics are listed below.

- Maximum channel bandwidth 20MHz.

- 1 Tx architecture with 26dBm PA

- 1 Rx or 2 Rx is allowed to be implemented.

Observation 1: Compared with FDD PC2 non-RedCap UE in Rel-17, only 1Tx 26dBm PA is assumed for transmitter characteristics and 1 Rx is allowed for receiver characteristics apart from 2 Rx.

Thus, the FDD PC2 RedCap UE can be implemented by using the same PA/duplexer of 1 Tx FDD PC2 non-RedCap. According to the TR 38.861, PA linearity and Duplexer allowed maximum power rating should be improved at least 3dB compared to the existing component. From standard perspective, the same PA/duplexer of 1 Tx FDD PC2 non-RedCap can accelerate the industrial ecosystem of FDD PC2 RedCap UE. Consequently, the 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused for the FDD PC2 RedCap UE.

Proposal 1: the 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused for the FDD PC2 RedCap UE.

Logically, since FDD PC2 RedCap UE have to follow the component ecosystem of 1 Tx FDD PC2 non-RedCap, the supported PC2 FDD bands for RedCap UE should be same as FDD PC2 non-RedCap UE.

Proposal 2: the supported PC2 FDD bands for RedCap UE should be same as FDD PC2 non-RedCap UE.

For 1Tx/2Rx architecture of FD-FDD PC2 RedCap, the RSD requirements less than or equal to 20 MHz should be same as the table 7.3.2-1c specified in TS 38.101-1 for UE not supporting Tx diversity in FDD bands.

Proposal 3: For 1Tx/2Rx architecture of FD-FDD PC2 RedCap, the RSD requirements less than or equal to 20 MHz should be same as the table 7.3.2-1c specified in TS 38.101-1 for UE not supporting Tx diversity in FDD bands.

For 1Tx/1Rx architecture of FD-FDD PC2 RedCap, more discussion is needed since it’s different from the RF architecture of FDD PC2 non-RedCap UE.

We summarize the different REFSENS for band n3 5/20MHz.

|  | 1Tx (23dBm) /2Rx | 1Tx (23dBm) /1Rx (RedCap) | 1Tx (26dBm) /2Rx | 2Tx (23dBm) /2Rx | 1Tx (26dBm) /1Rx |
| --- | --- | --- | --- | --- | --- |
| band n3 20MHz REFSENS | -90.8 | -87.8 (+3dB ΔR1R) | -90.3 (+0.5dB 1Tx RSD) | -89.3 (+1.5dB 2Tx RSD) | -87.8 ~ (-90.3 + ΔR1R ) |
| band n3 5MHz REFSENS | -97 | -94.5 (+2.5dB ΔR1R) | -96.5 (+0.5dB 1Tx RSD) | -95.6 (+1.4dB 2Tx RSD) | -94.5 ~ (-96.5 + ΔR1R) |

Firstly, RAN4 should discuss the ΔR1R for FDD PC2 RedCap. Generally, the ΔR1R can’t be higher than 3dB. It’s observed that the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R

Observation 2: the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R

Considering the worst case, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD. In other words, REFSENS (1Tx (26dBm) /2Rx) + ΔR1R is equal to REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD. And the logical diagram for band n3 20MHz REFSENS as an example can be found below.

Figure 1 the logical diagram about how to specify the REFSENS for 1Tx/1Rx architecture of FD-FDD PC2 RedCap

Proposal 4: For 1Tx/1Rx architecture of FD-FDD PC2 RedCap, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD.

Summary

Observation 1: Compared with FDD PC2 non-RedCap UE in Rel-17, only 1Tx 26dBm PA is assumed for transmitter characteristics and 1 Rx is allowed for receiver characteristics apart from 2 Rx.

Proposal 1: the 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused for the FDD PC2 RedCap UE.

Proposal 2: the supported PC2 FDD bands for RedCap UE should be same as FDD PC2 non-RedCap UE.

Proposal 3: For 1Tx/2Rx architecture of FD-FDD PC2 RedCap, the RSD requirements less than or equal to 20 MHz should be same as the table 7.3.2-1c specified in TS 38.101-1 for UE not supporting Tx diversity in FDD bands.

Observation 2: the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R

Proposal 4: For 1Tx/1Rx architecture of FD-FDD PC2 RedCap, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD.

- References

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] TR 38.861, Study on high power UE (power class 2) for one NR FDD band