# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501457

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN HPUE

Agenda Item:	7.10.2.1

Document for:	Approval

Introduction

In the RAN4#113 meeting, the initial coexistence results for NTN HPUE were captured into the collection [1] including our contribution [2]. And the required ACLR for NR NTN/IoT-NTN HPUE were summarized in the WF [3].

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective: For NR-NTN PC2: - Less than 22 dB with 1.5km isolation distance assumed (baseline assumption) - [26.5 dB] with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1.5: - Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption) - [30.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1: - Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption) - [28.3/32.7] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC2: - Less than 21.1 dB with 1.5km isolation distance assumed; - Less than [20.3] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC1: - Less than 21.9 dB with 1.5km isolation distance assumed; - Less than [24.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case

However, our results for NR NTN were not properly captured under the reasonable isolation distance assumption. In this paper, we’d like to further clarify our simulation results for NR NTN and further discuss the scenario for IoT NTN coexistence study.

Discussion on the coexistence for NR NTN HPUE

Referring to our contribution [2] in last RAN4 meeting, we clarify our results about whether isolation distance is used or not.

We consider the scenario that NTN UE was dropped into the cluster of terrestrial network, the following red area can be considered with 1.5km above radius as the worst case. However, this scenario is much stricter than the real deployment.

Observation 1: Our simulation results were submitted based on zero meter isolation distance considering the worst scenario that NTN UE was dropped into the cluster of terrestrial network.

In the NTN HPUE coexistence summary [1], our results were captured based on 1.5km isolation distance. If possible, it’s better to update the results based on the real simulation assumption.

Simulation results

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC2 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.931 | 35.886 | 35.947 | 36.014 | 35.938 | 35.868 | 35.91 |
| Average throughput with adjacent interference [Mbps] | 35.642 | 35.66 | 35.736 | 35.85 | 35.789 | 35.758 | 35.82 |
| Average throughput loss | 0.80% | 0.63% | 0.59% | 0.46% | 0.41% | 0.31% | 0.25% |
| 5%-tile throughput baseline [Mbps] | 15.069 | 14.989 | 15.163 | 15.455 | 15.313 | 15.103 | 15.287 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.97 | 14.102 | 14.428 | 14.827 | 14.765 | 14.707 | 14.911 |
| 5%-tile throughput loss | 7.29% | 5.92% | 4.85% | 4.06% | 3.58% | 2.62% | 2.46% |

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC1.5 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.895 | 35.954 | 35.949 | 35.91 | 35.879 | 35.918 | 35.913 |
| Average throughput with adjacent interference [Mbps] | 35.537 | 35.64 | 35.673 | 35.71 | 35.704 | 35.773 | 35.782 |
| Average throughput loss | 1.00% | 0.87% | 0.77% | 0.56% | 0.49% | 0.40% | 0.36% |
| 5%-tile throughput baseline [Mbps] | 15.347 | 15.268 | 15.156 | 15.325 | 15.344 | 15.01 | 15.425 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.869 | 14.043 | 14.095 | 14.54 | 14.714 | 14.469 | 14.915 |
| 5%-tile throughput loss | 9.63% | 8.02% | 7.00% | 5.12% | 4.11% | 3.60% | 3.31% |

For LEO1200-Urban Macro scenario, the results for 2GHz scenario 4 at PC1 are shown below.

| Sate UE ACLR1 [dB] | 20 | 22 | 24 | 26 | 28 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput baseline [Mbps] | 35.887 | 35.922 | 35.87 | 35.91 | 35.966 | 35.915 | 35.885 |
| Average throughput with adjacent interference [Mbps] | 35.496 | 35.571 | 35.583 | 35.681 | 35.758 | 35.731 | 35.741 |
| Average throughput loss | 1.09% | 0.98% | 0.80% | 0.64% | 0.58% | 0.51% | 0.40% |
| 5%-tile throughput baseline [Mbps] | 15.085 | 15.043 | 15.03 | 14.973 | 15.349 | 15.194 | 15.086 |
| 5%-tile throughput with adjacent interference [Mbps] | 13.604 | 13.597 | 13.847 | 14.173 | 14.618 | 14.561 | 14.565 |
| 5%-tile throughput loss | 9.82% | 9.61% | 7.87% | 5.34% | 4.76% | 4.17% | 3.45% |

Observation 2: Based on the simulation results for 2GHz scenario 4 (UL UEtoBS) LEO1200-Urban Macro (zero meter isolation distance), 24dB ACLR for PC2, 27dB ACLR for PC1.5 and 28dB ACLR for PC1 can be observed.

Discussion on coexistence study for NB-IoT NTN

Referring to the R18 WF for NB-IoT NTN coexistence, the following proposed frequency and bandwidth for coexistence study were captured.

The frequency and bandwidth are listed in table 6.1-3. Table 3.1-3. Proposed frequency and bandwidth for co-existence study Frequency; Bandwidth; Duplex mode; Frequency reuse factor. TN Rural; 2 GHz; 20MHz; FDD, TDD; 1. TN Urban macro; 2 GHz; 20MHz; FDD, TDD; 1. GEO; 2 GHz; 0.18MHz NB-IoT; FDD; 1. LEO; 2 GHz; 0.18MHz NB-IoT; FDD; 1.

However, in the end, RAN4 just reuse the TN IoT NTN ACLR requirements for coexistence with GSM and UTRA.

Table 6.5B.3.4-1: category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

In last RAN4 meeting, the WF [4] on IoT NTN HPUE was approved with the following agreements for ACLR requirements.

Issue 1-1-1: For NB-Iot Based Iot-NTN, whether and how to define GSM_ACLR. - Proposals - Option 1: Remove this requirement, [at least for HPUE] - Option 2: Keep this requirement and reusing the PC3 requirement for HPUE. - Option 3: Others Agreement: - Remove GSM_ACLR for NB-IoT based NTN HPUE. Issue 1-1-2: PC1/2 ACLR should not below PC3 requirements, based on the assumption that PC3 requirement can be relaxed if necessary. - Proposals - Option 1: Yes. - Option 2: No - Option 3: Others Agreement: - HPUE ACLR should not be relaxed than that for PC3 for NB-IoT NTN.

Based on the agreements above, RAN4 has reached the consensus that GSM ACLR requirements will be removed and there is no discussion on whether UTRA ACLR will be kept or not.

Observation 3: For NB-IoT NTN HPUE, RAN4 has reached the consensus that GSM ACLR requirements will be removed and there is no discussion on whether UTRA ACLR will be kept or not.

In addition, some companies provided the coexistence studies for NB-IoT NTN in last meeting and results were summarized below.

For IoT-NTN PC2: - Less than 21.1 dB with 1.5km isolation distance assumed; - Less than [20.3] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC1: - Less than 21.9 dB with 1.5km isolation distance assumed; - Less than [24.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case

Observation 4: Based on the initial simulation results, the NB-IoT NTN HPUE ACLR requirements coexisting with NR 20MHz can be specified below.

|  | NRACLR |
| --- | --- |
| ACLR | 20 ~ 24 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±10 MHz |
| Adjacent channel measurement bandwidth | 15kHz*(12*106RB+1)/1000 |
| Measurement filter | Rectangular |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular |

As the adjacent channel measurement bandwidths between UTRA and NR are different, it’s better to clarify how to specify the new ACLR requirements for NB-IoT NTN HPUE in order not to relax the NB-IoT NTN HPUE ACLR requirements compared to PC3.

Proposal 1: As the adjacent channel measurement bandwidths between UTRA and NR are different, it’s better to clarify how to specify the new ACLR requirements for NB-IoT NTN HPUE in order not to relax the NB-IoT NTN HPUE ACLR requirements compared to PC3.

Summary

Observation 1: Our simulation results were submitted based on zero meter isolation distance considering the worst scenario that NTN UE was dropped into the cluster of terrestrial network.

Observation 2: Based on the simulation results for 2GHz scenario 4 (UL UEtoBS) LEO1200-Urban Macro (zero meter isolation distance), 24dB ACLR for PC2, 27dB ACLR for PC1.5 and 28dB ACLR for PC1 can be observed.

Observation 3: For NB-IoT NTN HPUE, RAN4 has reached the consensus that GSM ACLR requirements will be removed and there is no discussion on whether UTRA ACLR will be kept or not.

Observation 4: Based on the initial simulation results, the NB-IoT NTN HPUE ACLR requirements coexisting with NR 20MHz can be specified below.

Proposal 1: As the adjacent channel measurement bandwidths between UTRA and NR are different, it’s better to clarify how to specify the new ACLR requirements for NB-IoT NTN HPUE in order not to relax the NB-IoT NTN HPUE ACLR requirements compared to PC3.

- References

[1] R4-2420406	Collected NTN HPUE co-existence results, Samsung

[2] R4-2419071	Discussion on coexistence study for NR NTN HPUE, Huawei, HiSilicon

[3] R4-2420403, WF on NR NTN HPUE, Samsung

[4] R4-2420404, WF on HPUE for IoT NTN, vivo