# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501458

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	7.10.2.2.1

Document for:	Approval

Introduction

In the RAN4#112 meeting, the WF [1] on UE RF requirements for NTN HPUE was approved with the following agreements.

Issue 3-1-1: General considerations - Agreement - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements. Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - For PC2 IoT NTN, at least 1Tx shall be considered - For other PCs (both IoT NTN and NR NTN), FFS Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type. Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes.

In the RAN4#112bis meeting, we have the following way forward [2].

Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE Agreement: - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - .... Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE. Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information. Issue 2-8: SEM Agreement: - To re-use existing SEM requirements from PC3 to other PCs as starting point. Issue 2-9: MOP tolerance Agreement: - Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE

In the RAN4#113 meeting, we have the following way forward [3].

Issue 2-1: MOP Agreement: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE Issue 2-2: MPR/A-MPR Agreement: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. Issue 2-3: SEM Way forward: - Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point. - Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes. Issue 2-4: ACLR Agreement in #112bis meeting: Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.. Way forward: - Further discuss the ACLR requirements for NR-NTN HPUE - Take into account the co-ex study outcomes in this #113 meeting with previous agreements.

In this paper, we’d like to share our views on parts of Tx RF requirements, feasibility and RF architecture. In addition, we’d like to share our views about SEM requirements.

Feasibility and RF architecture for different power classes

In last meeting, RAN4 reached the consensus that both handheld and non-handheld devices can support PC2. However, there is no consensus on whether the other power classes can be applicable for handheld device. We’d like to share the following aspects to discuss the feasibility for different UE power classes.

Demands: Based on the rough evaluation, there are around 10~15dB gaps for handheld UE coupling loss between LEO scenario and GEO scenario. Due to large pathloss, it’s very hard to apply wideband communication technology for GEO scenario. Thus, GEO scenario, is not friendly to wideband communication technology, i.e. NR NTN. It’s not technically reasonable to force handheld UE to support other higher power classes for NR NTN as PC2 can meet the demands of LEO scenario.

UE implementation/cost: currently, PC3 is widely supported for handheld UE in TN FDD bands. Although PC2 for TN FDD bands was introduced in Rel-17 with both 1Tx and 2Tx RF architecture, this PC2 feature in FDD bands is not widely supported or implemented in handheld UE due to the difficulties of some RF components, e.g. FDD band duplexer and so on. If we assume handheld UE to support PC1.5/PC1 by using 1 Tx chain, all of RF components have to be redesign in order to tolerate the higher output power, which are not realistic. And this RF architecture is not assumed for TN PC1.5 TDD bands. If we want to reuse the current industry ecosystem as much as possible, the only way to implement PC1.5 is to use 2Tx architecture with two PC2 PAs or 4Tx architecture with four PC3 PAs. However, since the handheld UE has the limited form factor and can’t tolerate the large power consumption, currently it’s infeasible to implement PC1.5/PC1 for FDD bands in handheld UE.

Standard perspective: In TN UE specification, only TDD bands can support PC1.5 with two Tx RF architecture assumption as it is very mature that current industry can support 26dBm with one Tx chain in TDD bands. However, in FDD bands, the situation seems different that current industry can’t widely support 26dBm with one Tx chain in FDD bands. Thus, current TN UE specification doesn’t support PC1.5 in FDD bands. Generally, NTN UE need to reuse the ecosystem of TN RF components. From standard perspective, it is premature to specify PC1.5/PC1 for FDD bands in handheld UE.

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19. It’s unnecessary to further discuss the handheld UE supports for PC1.

Proposal 2: For NR NTN, it’s proposed not to support / specify PC1.5 for handheld UE in Rel-19.

As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, logically it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

Proposal 3: As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

In the last meeting, RAN4 reached the way forward to further discuss the 2Tx and 3Tx architecture for PC1 non-handheld UE.

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

We think it’s also feasible to achieve 4Tx for PC1 non-handheld UE. PC1 non-handheld UE can utilize four 26dBm PAs to implement such architecture to reduce the costs comparing to the single 31dBm PA.

Proposal 4: 4Tx RF architecture for PC1 non-handheld UE can be specified.

Antenna gain assumption for handheld UE

In the replied LS [4], RAN4 thinks that a realistic UE antenna gain value would be -5.5 dBi. Thus, 26dBm NR NTN UE is only applicable to the LEO scenario. In the ETSI EN 301 681, it’s only applicable to the Geostationary satellite scenario.

Observation 1: for some regulatory discussion (e.g. ETSI EN 301 681), it’s very hard to consider 3GPP handheld UE with antenna gain larger than -5.5dBi, even if 15dBw maximum EIRP is allowed.

SAR

- Agreement: - Further discuss the proposed schemes related to duty cycle related SAR scheme. - Other schemes or no duty cycle related scheme are not precluded.

In the last meeting, RAN4 has no consensus on how to address the SAR issue and which kind of schemes can be used for SAR issue. Some companies proposed to consider both P-MPR and duty cycle method [5], but it’s unclear what is the assumed period for the duty cycle. For FDD NTN bands, there is no TDD-like UL/DL semi-static configurations. In addition, it’s also unclear whether it’s allowed for network to ignore the reports of UE duty-cycle and configure UL slots based on traffic demands as what we specify for TN TDD duty cycle solution. Many companies have commented that the current TN TDD duty cycle solution doesn’t provide the expected benefits or performance in the real field, so it’s also unclear whether FDD NTN duty cycle solution has the similar issue.

Some companies proposed to investigate the serving time of each beam or the beam service time ratio of the UE [6]. This is a good way forward. But GSO scenario may be different from LEO scenario. For LEO scenario, satellite may provide limited serving time at each beam footprint when a lot of NTN UE need to be served in the specific area. However, for GSO scenario, satellite usually use one fixed beam to cover very large area. In addition, the targeted UE power class are different for different scenarios. Thus, it’s better to discuss the targeted scenario and goal for SAR issue firstly. And then RAN4 can further discuss which kind of scheme can be used or not.

Proposal 5: for NTN HPUE SAR issue, RAN4 can further discuss whether the following typical scenarios are assumed.

|  | Targeted Satellite orbit | Targeted UE PC | Duplex mode | Satellite beam hopping / area | The minimum duty cycle to comply SAR without power reduction. | UL traffic mode |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT NTN | GSO | PC1 or PC2? | HD-FDD | No beam hopping, large area service | <=12.5% for PC1 <=50% for PC2 | FFS? |
| NR NTN | LEO | PC2 | FD-FDD | Support beam hopping. Beam serving time is unclear | <=50% for PC2 | FFS? |
| Common characteristics: 1) There is no TDD-like UL/DL semi-static configurations for NR/NB IoT NTN FDD bands. 2) Generally, SAR compliance is verified every 6 minutes. |  |  |  |  |  |  |

Generally, current NB-IoT NTN RAT can only support some message service, it’s unclear whether such small package UL traffic mode need a standardized SAR solution. For NR NTN, it’s targeted to apply for the satellite internet service, but it’s unclear how long the UL traffic package and how often UL package request arrive. It’s better to get more information about NR NTN UL traffic mode.

Proposal 6: RAN4 can investigate the UL traffic mode for NB-IoT NTN and NR NTN respectively and decide whether there is a SAR issue to be addressed.

SEM

In the TS 38.101-5, the spectrum emission mask was specified referring to the LTE/NR SEM as below.

- 6.5.2.2 Spectrum emission mask The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable. NOTE: For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band. The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth. Table 6.5.2.2-1: General NR spectrum emission mask ΔfOOB (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5; 10, 15, 20. ± 0-1; -13; -13; 1 % of channel BW. ± 1-5; -10; -10; 1 MHz. ± 5-6; -13. ± 6-10; -25. ± 5-BWChannel; -13. ± BWChannel-(BWChannel+5); -25.

According to the clause 6.6.2 of TR 36.803, current LTE/NR/NR NTN spectrum emission mask was derived based on partial FCC terrestrial IMT requirements of band 2 and band 7 (41 2.6GHz).

However, it’s unclear for NR NTN UE to comply with the similar FCC terrestrial IMT requirements of band 2 and band 7.

Observation 2: According to the clause 6.6.2 of TR 36.803, current general NR NTN UE spectrum emission mask in TS 38.101-1 was derived based on partial FCC terrestrial IMT requirements of band 2 and band 7 (41 2.6GHz).

In the paper [7], companies provided the existing regulatory information for satellite communication. We summarized some observations below.

- ETSI EN 301 681 for L band 255 and 253

Observation: It seems that the SEM requirements in ETSI EN 301 681 are only for some narrow bandwidth system, not sure whether NR NTN can use or refer to this regulation.

- ETSI EN 302 574 Part 3 for S band 256

Observation: It seems that the SEM requirements in ETSI EN 302 574 Part 3 are only for some narrow bandwidth system less than 1MHz BW operating in GSO, not sure whether NR NTN can use or refer to this regulation.

- ETSI EN 302 574 Part 2 for S band n256

Observation: It seems that the SEM requirements in ETSI EN 302 574 Part 2 were referring to the SEM of TN UTRA for the assigned channel bandwidth of 1MHz or greater (but nor larger than 5MHz). The satellite component is based on GSO.

- FCC 47 CFR part 25 specification in clause 202

The applicable frequency range ( band 255, 254, 252):

This requirement can be translated to the following table.

| Center Frequency of assigned channel to out-of-band region | Attenuation [dB] | Measurement BW [kHz] | NOTE |
| --- | --- | --- | --- |
| 0.5B - B | 25 | 4 |  |
| B – 2.5B | 35 | 4 |  |
| >=2.5B | 43+10log10(W) | 4 | -13dBm/4kHz Same SE as specified in ITU SM.329 |

- Annex 5 of ITU-R SM.1541-6

In the Annex 5 of ITU-R SM.1541-6, the requirements were specified below.

- 3 OoB masks for mobile-satellite service (MSS) earth and space stations The masks contained in Recommendation ITU-R M.1480 can be used for mobile earth stations of geostationary-satellite orbit (GSO) MSS systems implementing the GMPCS memorandum of understanding in parts of the frequency band 1-3 GHz. The masks contained in Recommendation ITU-R M.1343, representing non-GSO mobile earth stations in the band 1-3 GHz can form one input for the mobile earth station data. For earth stations not covered in the above-mentioned Recommendations and for all space stations, the following generic OoB mask, considered as an upper bound for MSS systems, is to be used: Attenuation of OoB emissions in the reference bandwidth of 4 kHz for MSS systems below 15 GHz (otherwise in reference bandwidth of 1 MHz for MSS systems above 15 GHz) is: where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth, which will range from 0% to the spurious boundary (which is usually 200%). The above-proposed mask may not be applicable in detailed examination of adjacent band compatibility.

The requirements include three parts:

| Regulation | Application | NOTE |
| --- | --- | --- |
| ITU R M.1480 | GSO MSS systems implementing the GMPCS memorandum in frequency band 1-3 GHz. | The requirements are similar to the ETSI EN 301 681 only for some narrow bandwidth system, not sure whether NR NTN can use or refer to this regulation. |
| ITU R M.1343 | Representing non-GSO mobile earth stations in the band 1-3 GHz | The requirements are only applicable to the NGSO MSS system in the bands 1-3 GHz using TDMA and CDMA. Not sure whether NR NTN using OFDMA can use or refer to this regulation. |
| ITU-R SM.1541-6 | earth stations not covered in the above mentioned Recommendations | General |

In total, we summarize all the current regulation for satellite communication below.

| Regulation | Application | NOTE |
| --- | --- | --- |
| ETSI EN 301 681 for L band 255 and 253 | narrow bandwidth system | NR NTN can’t use or refer to this regulation. |
| ETSI EN 302 574 Part 3 for S band 256 | some narrow bandwidth system less than 1MHz BW operating in GSO | NR NTN can’t use or refer to this regulation. |
| ETSI EN 302 574 Part 2 for S band n256 | referring to the SEM of TN UTRA for the assigned channel bandwidth of 1MHz or greater (but nor larger than 5MHz). The satellite component is based on GSO. | NR NTN can’t use or refer to this regulation. |
| FCC 47 CFR part 25 specification in clause 202 | The applicable frequency range ( band 255, 254, 252): | General Attenuation Method |
| ITU R M.1480 | GSO MSS systems implementing the GMPCS memorandum in frequency band 1-3 GHz. | The requirements are similar to the ETSI EN 301 681 only for some narrow bandwidth system, not sure whether NR NTN can use or refer to this regulation. |
| ITU R M.1343 | Representing non-GSO mobile earth stations in the band 1-3 GHz | The requirements are only applicable to the NGSO MSS system in the bands 1-3 GHz using TDMA and CDMA. Not sure whether NR NTN using OFDMA can use or refer to this regulation. |
| ITU-R SM.1541-6 | earth stations not covered in the above mentioned Recommendations | General Attenuation Method Attenuation of OoB emissions in the reference bandwidth of 4 kHz for MSS systems below 15 GHz (otherwise in reference bandwidth of 1 MHz for MSS systems above 15 GHz) is: where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth, which will range from 0% to the spurious boundary (which is usually 200%). |

Based on the analysis above, the general attenuation methods based on FCC 47 CFR part 25 specification in clause 202 and Annex 5 of ITU-R SM.1541-6 can be referred to update the NR NTN SEM definition.

Proposal 7: the general attenuation methods based on FCC 47 CFR part 25 specification in clause 202 and Annex 5 of ITU-R SM.1541-6 can be referred to update the NR NTN SEM definition.

Summary

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19. It’s unnecessary to further discuss the handheld UE supports for PC1.

Proposal 2: For NR NTN, it’s proposed not to support / specify PC1.5 for handheld UE in Rel-19.

Proposal 3: As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

Proposal 4: 4Tx RF architecture for PC1 non-handheld UE can be specified.

Observation 1: for some regulatory discussion (e.g. ETSI EN 301 681), it’s very hard to consider 3GPP handheld UE with antenna gain larger than -5.5dBi, even if 15dBw maximum EIRP is allowed.

Proposal 5: for NTN HPUE SAR issue, RAN4 can further discuss whether the following typical scenarios are assumed.

Proposal 6: RAN4 can investigate the UL traffic mode for NB-IoT NTN and NR NTN respectively and decide whether there is a SAR issue to be addressed.

Observation 2: According to the clause 6.6.2 of TR 36.803, current general NR NTN UE spectrum emission mask in TS 38.101-1 was derived based on partial FCC terrestrial IMT requirements of band 2 and band 7 (41 2.6GHz).

Proposal 7: the general attenuation methods based on FCC 47 CFR part 25 specification in clause 202 or Annex 5 of ITU-R SM.1541-6 can be referred to update the NR NTN UE SEM definition.

- References

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[3] R4-2420403, WF on NR NTN HPUE, Samsung

[4] R4-2214968, LS Reply on UE antenna gain for NR NTN coverage enhancement, RAN WG4

[5] R4-2420404, WF on HPUE for IoT NTN, vivo

[6] R4-2419645, Discussion on IoT-NTN HPUE Tx requirements and SAR, Samsung

[7] R4-2416396, Key aspects on HPUE for NTN, Inmarsat, Viasat