3GPP TSG-RAN WG4 Meeting #114		R4-2501463

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.22.3.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last RAN4 meeting, we analyzed the RRM requirements/impacts of objectives#2/3/4/5 and achieved some agreements in [2].

There are no open issues in objective #2 and objective #4. The only potential RRM impacts are related to CJTC reporting and asymmetric DL sTRP/UL mTRP scenarios.

In this contribution, we continue to discuss the RRM requirements for “CJTC reporting” and “asymmetric DL sTRP/UL mTRP scenarios” in Rel-19 MIMO phase 5.

- Discussion

2.1 UE reporting enhancement for CJT calibration

In RAN4#113 meeting, RAN4 agreed to perform further simulation for CJTC reporting.

Issue 2-1: On the simulation assumption for CJT calibration Agreement is captured in R4-2420209. Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define accuracy requirements. - Option 2: FFS. Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

According to the agreed simulation assumption in [3], we simulated for frequency estimation for two TRPs.

The initial simulation results are shown as:

| One shot measurement | SNR=-3dB | SNR=0dB | SNR=3dB |
| --- | --- | --- | --- |
| Frequency offset between TRP = 350Hz | Delta_freq_error = not valid | Delta_freq_error ~ 65Hz | Delta_freq_error ~ 17Hz |
| Frequency offset between TRP = 700Hz | Delta_freq_error ~ 63Hz | Delta_freq_error ~ 18Hz | Delta_freq_error <10Hz |

Observation 1: If SNR = -3dB, one shot measurement is not valid.

Observation 2: If SNR = 3dB, one shot measurement is enough for frequency offset reporting.

For frequency offset reporting, the reporting type is aperiodic measurement report.

In last meeting, companies what to collect more simulation results to verify whether one sample or 3 samples of measurement can be used to define the measurement period for aperiodic reporting.

Proposal 1: RAN4 to check whether use SNR 3dB to verify one shot measurement is accurate for frequency offset reporting in most cases.

In last meeting, the simulation assumption is approved. However, the metric of accuracy was not discussed and approved for fair alignment.

We want to propose to collect the simulation results by CDF curves of ∆f and ∆t for the estimated frequency offset reporting and delay offset reporting.

Proposal 2: Use simulation results by CDF curves of ∆f and ∆t for the estimated frequency offset reporting and delay offset reporting.

2.3 Asymmetric DL sTRP/UL mTRP

In RAN4#113 meeting, it is agreed to update the RRM requirement for 2TA enhancement for asymmetric DL sTRP/UL mTRP in [2]:

Issue 4-1: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? In the last meeting (RAN4#112-bis) RAN4 agreed to update the RRM requirements for 2TA enhancement: Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.. Way forward: - Alt 1: Define requirement for 2TA following the legacy principle. - Alt 2: Wait for RAN1.

The detailed requirements are still not defined.

In RAN1#118bis meeting, RAN1 agreement for 2TA extension are shown as:

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Firstly, it removed the restriction of that coresetPoolIndex needs to be configured for the 2TA feature.

In Rel-18, two TAs for m-DCI and m-TRP are introduced. RAN4 define such RRM requirements including: MRTD/MTTD, DL reference timing, other RRM requitements which have impacts such as L1-RSRP and TCI state switching delay requitements.

The association between coresetPoolIndex and TCI state are shown as:

38.821 The Unified TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size consisting of following fields: - CORESET Pool ID: This field indicates that the mapping between the activated TCI states and the codepoint of the DCI Transmission Configuration Indication set by field TCI state ID is specific to the ControlResourceSetId configured with CORESET Pool ID as specified in TS 38.331 [5]. This field set to 1 indicates that the TCI states are specified to CORESET pool ID equal to 1, otherwise the TCI states are specified to CORESET pool ID equal to 0. If no more than one value for the coresetPoolIndex is configured for any CORESET in the BWP, the R bit is present instead;

For one unified TCI state, it is associated with CORESET Pool ID.

For joint/UL TCI state, it is associated with either TAG1 or TAG2, as shown in:

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] } TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ..., [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

If follow the principle of that in Rel-18, the uplink shall be timing advanced before a downlink reference point.

In last RAN1#119 meeting, RAN1 sent reply LS to RAN4 in [4]:

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

In Rel-19 asymmetric DL sTRP/UL mTRP, the scenarios are: UE can receive DL transmission and may transmit UL transmission from the TRP#1 while transmit UL transmission from TRP#2 (TRP#2 might be a UL-TRP with PL-RS offset configured).

From this reply from RAN1, it can be observed: if UE is configured with PL offset in joint/UL TCI states, UE only receive DL signal from one TRP.

In Rel-18 two TA mechanism, two uplink transmission timing are related to the reception of the downlink reference point for each TAG, which means for each TAG, there can be dedicated downlink reference point.

Proposal 3: In Rel-19 asymmetric DL sTRP/UL mTRP with two TAs, if UE is configured with PL offset in joint/UL TCI states, for both TAGs, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. The reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState.

Where NTA and NTA offset are commanded by the network independently for both TAGs. only one n-TimingAdvanceOffset is configured, otherwise it is NTA offset defined in table 7.1.2-2.

For the TCI state switching delay requitements of Rel-19 asymmetric DL sTRP/UL mTRP scenarios, it is still open and the options are listed in [2] as below:

Issue 4-2: Whether to define UL TCI state activation requirement based on SRS? Way forward: - Option 1: Not define requirement (Samsung, HW, vivo) - Option 2: RAN4 to define UL TCI state activation requirement based on SRS. (Xiaomi, Ericsson, Nokia) - Option 3: Further discuss based on the feasibility of defining requirement and testing (QC, Apple, MTK)

In last meeting, we analyzed the UL TCI state switching based on SRS. Companies still want to check whether there is feasibility to define the requirements and the test. Let’s emphasize our analysis again.

Firstly, for FR2, it seems the common understanding in group is that the UL TCI state can be associated to SRS only.

According to RAN1 agreement, joint TCI states is not supported in FR2. Only separate TCI UL state can be supported in FR2.

If the UL TCI state switching requirements are needed to be defined for SRS source, let’s discuss what can be defined.

If the target TCI state is associated with SRS, there is no time for measurement and UL beam refinement. In other word, if there is the delay, the delay is corresponding to the MAC-CE decoding time.

The other thing is the known condition. For uplink TCI state, the only thing is the spatial info of the beams.

In uplink TCI, there are two types of RS. One if referenceSignal which we called source RS (SRS in FR2 above). The other is pathlossReferenceRS. In UL-TRP, the power control is using PL-RS offset + PL-RS in DL/UL TRP.

If the known condition of target TCI is defined, it can contain: known condition of source RS as SRS. known condition of PL-RS in another TRP and relationship with PL-RS offset.

In legacy Rel-17 feMIMO discussion, the similar discussion on the known condition of source RS as SRS were discussed and finally, the conclusion is it is hard to define the known condition of SRS.

Compare the difference between Rel-17 feMIMO such discussion and current Rel-19 discussion, there is no difference. By our understanding, it is the reason that why current requirements “There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS” and why we proposed to follow no requirements when UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS in this asymmetric DL sTRP/UL mTRP scenarios.

Proposal 4: For FR2, no joint TCI state is supported. Only separate TCI UL state can be supported. Only the UL TCI states can only be associated to SRS.

Observation 3: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement in Rel-17/Rel-18.

Proposal 5: RAN4 to decide: If UL TCI states can only be associated to SRS

    - Alt1: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

    - Alt2: The delay is MAC-CE processing time if define the known condition of target TCI state.

Proposal 6: We prefer Alt in Proposal 5. Otherwise, it is a little strange of “There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS” in Rel-17 and new definition of Rel-19, although there is no difference for the same thing.

Issue 4-3: RRM core impacts of active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS.

When we can achieve the agreement for Issue 4-2, the only open part in Issue 4-3 is how to handle joint TCI state switching FR1.

If it is joint TCI in FR1, no matter one joint TCI or two joint TCIs, when joint TCI#1 is changed to joint TCI#2, the source RS is only applicable for DL reception which the source RS can be the DL RS which is only from DL/UL-TRP. And the PL-RS is only available from DL/UL-TRP, UE is indicated by PL-offset and perform UL transmission on UL-TRP.

If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy.

Proposal 7: RRM core impacts of active uplink TCI state switching delay for unified TCI

    - FR1: If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy. No RRM impacts.

    - FR2: depends on the conclusion of whether to specify the UL TCI state switching delay base on SRS.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for for “CJTC reporting” and “asymmetric DL sTRP/UL mTRP scenarios” in Rel-19 MIMO phase 5:

Observation 1: If SNR = -3dB, one shot measurement is not valid.

Observation 2: If SNR = 3dB, one shot measurement is enough for frequency offset reporting.

Proposal 1: RAN4 to check whether use SNR 3dB to verify one shot measurement is accurate for frequency offset reporting in most cases.

Proposal 2: Use simulation results by CDF curves of ∆f and ∆t for the estimated frequency offset reporting and delay offset reporting.

Proposal 3: In Rel-19 asymmetric DL sTRP/UL mTRP with two TAs, if UE is configured with PL offset in joint/UL TCI states, for both TAGs, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. The reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState.

Where NTA and NTA offset are commanded by the network independently for both TAGs. only one n-TimingAdvanceOffset is configured, otherwise it is NTA offset defined in table 7.1.2-2.

Proposal 4: For FR2, no joint TCI state is supported. Only separate TCI UL state can be supported. Only the UL TCI states can only be associated to SRS.

Observation 3: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement in Rel-17/Rel-18.

Proposal 5: RAN4 to decide: If UL TCI states can only be associated to SRS

    - Alt1: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

    - Alt2: The delay is MAC-CE processing time if define the known condition of target TCI state.

Proposal 6: We prefer Alt in Proposal 5. Otherwise, it is a little strange of “There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS” in Rel-17 and new definition of Rel-19, although there is no difference for the same thing.

Proposal 7: RRM core impacts of active uplink TCI state switching delay for unified TCI

    - FR1: If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy. No RRM impacts.

    - FR2: Depends on the conclusion of whether to specify the UL TCI state switching delay base on SRS.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2420108, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek

[3] R4-2420209, Simulation assumptions for CJT calibration, MediaTek

[4] R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1