3GPP TSG-RAN WG4 Meeting #114	R4-2501471
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.1.1.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1].

Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario2 Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation - Agreement in main session: - Extended CBW based approach - Where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW. Issue 1-1-2: Approaches of calculating the new inner RB region - Options - Option 1: The NW signals the extended number of RBs and RB shift towards the UE configured CBW or RRC signalling to indicate the extended RB number at each side of the UE CBW for the UE to calculate the new inner, outer, and edge regions - The extended number of RBs may not necessarily be the same as those in the BS CBWs - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 2: Based on the fixed extension ratio, it is up to the NW to judge the gap width on both sides of the UE and grant the permission to UE using the reduced MPR for the new inner region - The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 3: For a UE supporting [capability] and configured with a UE channel bandwidth within a carrier with a bandwidth greater than the transmission bandwidth configuration of this UE channel bandwidth, NRB = N´RB is the said carrier bandwidth as indicated by carrierBandwidth in [SIB1] and the condition for an inner RB allocation is modified accordingly - The intersection of the Inner region relative to the NW CBWs and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE Examples of new inner region just for illustration purpose: Example 1: single CC scenario; Example 2: multi-CC scenario. - WF - FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: The extended RBs on one side should be equal to NRB/2, where NRB is the maximum transmission bandwidth configuration of the original UE CBW - Option 2: The extended RB number for each side can be different and the value could vary from zero to the one specified in Table 5.3.2-1 in TS 38.101-1, but it is no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz. - Option 3: Width of Extended UE channel bandwidth can be up to BS channel bandwidth but no more or it can be narrower than BS CBW - Other options are not precluded - WF - Extension on each side of UE CBW could be different - FFS on extended RB numbers/channel bandwidth, which are up to X on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios - FFS whether extended RB number/channel bandwidth should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW - Fixed extended RB number/channel bandwidth to X is not precluded. - Extension is not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond X on each side does not further reduce the MPR. - FFS on X value Sub-topic 1-2: Applicable requirements Issue 1-2-1: Where to use extended IBE in the larger BS CBW - Proposals - Proposal 1: IBE should be used between edges of UE CBW and extended UE CBW. (vivo, ZTE, Huawei, Qualcomm, China Telecom, Skyworks) - Alt 1: extension of the IBE is NOT used in the guard band of UE CBW. - Alt 2: extension of the IBE is also used in the guard band of UE CBW. - Alt 3: extension of the IBE applies in the guard band of the UE CBW but not in the guard band of the extended UE CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS the application of IBE in guard band of UE CBW and/or extended UE CBW Issue 1-2-2: How to derive the extended IBE - Proposals - Proposal 1: (vivo) - For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW. (vivo, ZTE, Huawei) - The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW. (vivo, ZTE, Huawei) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified to: IBE_general = max⁡(PRB - 30 dB, max⁡{-25, 20*log10EVM -3 +5LCRB, -57dBm + 10log10(SCS/15kHz)- PRB}) - Proposal 2: (ZTE) - Use the IBE limit of -25 dB between edges of UE CBW and extended UE CBW for full RB allocation scenario - Proposal 3: (Huawei) - For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1). The equation for IBE derivation is: - max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB - Proposal 4: (China Telecom) - For full RB allocation, if the extended UE CBW exists, the IBE should be applied. - Proposal 5: (Qualcomm) - The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the signal within the UE BW. - WF - FFS the method to derive the extended IBE, which should cover both cases for partial RB allocation and full RB allocation in the UE CBW Issue 1-2-3: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW. (Nokia, Qualcomm, vivo, ZTE, LGE, OPPO, Huawei, China Telecom) - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW Alt 1. Alt 2. - WF - ACLR and SEM should be applicable from the edge of extended UE CBW. - FFS the following alternatives - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW Issue 1-2-4: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. (vivo, LGE) - Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz. (Nokia) - Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW (Qualcomm, OPPO) - WF - FFS with consideration of Issue 1-2-3 Sub-topic 1-3: Signaling and others Issue 1-3-2: Others - Proposals - Proposal 1: (Skyworks) - Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function with: - A-MPR based on the LCRB and RBstart and frequency based on the UE CBW (NRB and RBstart parameters) - MPR based on the extended UE BW and position (BS_NRB and RB0shift parameters). - WF - In cases where additional emission should be met, the UE performs the max(MPRenh, MPR, A-MPR) function with: - MPRenh here means the approach converting outer allocation to inner allocation i.e. the method that is discussed in this WI - Companies are encouraged to propose further refinement

This contribution further discusses power domain enhancement for single carrier.

- Discussion

In previous meetings, it was agreed that the approach of converting outer RB allocation to inner RB allocation is based on extended CBW.

To calculate the new inner RB region for UE CBW, we need first discuss how to decide the inner RB allocation based on extended CBW.

The MPR are related with the RB allocation ranges for Outer and Inner RB allocations and the RB allocations are the function of NRB, RBStart,Low, RBStart,High and LCRB. For example, the Inner RB allocation need meet the following conditions:

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in 38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

That is, the size of inner RB allocation is proportional with the UE CBW. Therefore, the inner RB allocation of extended CBW can follow the same conditions with the inner RB allocation of the UE CBW.

Actually, the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is a virtual carrier to reduce the MPR for the UE, the UE will be configured in the original UE CBW.

To facilitate define the inner RB allocation, the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case can be notated as a virtual CBW, and the related parameters of the RB allocation can notate as NRB,V, RBStart,Low,V, RBStart,High,V and LCRB,V, the conditions of Inner RB allocation for the virtual CBW are as following:

RBStart,Low,V  ≤  RBStart,V  ≤  RBStart,High,V, and

LCRB,V  ≤  ceil(NRB,V/2)

NRB,V is the maximum number of RBs for a virtual CBW, and

RBStart,Low,V = max(1, floor(LCRB,V/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High,V = NRB,V – RBStart,Low,V – LCRB,V

The Inner region for the virtual CBW is the soft blue colour and the original Inner region for the configured UE CBW is the blue colour, as shown in figure 2-1:

Figure 2-1 The MPR enhancement region

We can see the yellow part is the MPR enhancement region in Figure 2-1. If the UE has the MPR enhancement capability and the network enables the capability and signals the related information of the virtual CBW, and only when the configured RBs for the UE located in the yellow part, the UE need trigger new PHR report, if not, the UE keep the original PHR.  From this point, RAN 4 only need define the MPR enhancement region for new capability UE.

Therefore, the MPR enhancement region should be the intersection of the Inner region relative to the virtual CBW and the Outer region and edge region relative to the configured UE CBW where the inner MPR can be expected from UE, as the yellow part shown in figure 2-1.

Proposal 1: The extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case can be notated as a virtual CBW, and the related parameters of the RB allocation can be notated as NRB,V, RBStart,Low,V, RBStart,High,V and LCRB,V, the conditions of Inner RB allocation for the virtual CBW are as following:

RBStart,Low,V  ≤  RBStart,V  ≤  RBStart,High,V, and

LCRB,V  ≤  ceil(NRB,V/2)

NRB,V is the maximum number of RBs for a virtual CBW, and

RBStart,Low,V = max(1, floor(LCRB,V/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High,V = NRB,V – RBStart,Low,V – LCRB,V

Proposal 2: The MPR enhancement region (the extended Inner RB region) should be the intersection of the Inner region relative to the virtual CBW and the Outer region and edge region relative to the configured UE CBW where the inner MPR can be expected from UE, where the virtual CBW is fully covering the configured UE CBW and is larger than configured UE CBW.

In last meeting, it was agreed that

- Where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE.

- Both scenario 1 and 2 should be covered

- The network should ensure the co-existence be met when signaling the extended CBW.

In 38.331, the configured UE CBW are signaled by informing the carrierBandwidth and offetToCarrier in SIB1 and servingCellConfig, as following excerpt from 38.331:

We can see the carrierBandwidth is the width of the carrier in the number of PRB and the offetToCarrier is the offset using the number of PRB, that is carrierBandwidth informs the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in 38.101-1.

It is sufficient to define the number of PRB in the virtual CBW, since the virtual CBW is defined from the network perspective, and the network has ensured the co-existence be met when signaling the virtual CBW.

The guardband of the channel bandwidth is some reserved frequency resource to reduce the interference of adjacent channel, hence, the virtual CBW doesn’t need to define the guardband.

From signaling perspective, The NW only need signal the number of RBs in the virtual CBW and RB offset towards the lowest usage subcarrier on the configured UE CBW, as shown in Figure 2-2:

Figure 2-2 The RB offset towards the lowest usage subcarrier on the configured UE  CBW

0 RBoffset means the lowest subcarrier on the virtual CBW and the lowest usage subcarrier on the configured UE CBW are the same, Y RBoffset means the gap between the lowest subcarrier on the virtual CBW and the lowest usage subcarrier on the configured UE CBW is Y RBs.

In previous meeting, we provided the related simulation results for 20MHz channel bandwidth CP-OFDM QPSK modulation in [2] research the relationship of the MPR reduction and the extended UE CBW. And we got that:

- When extended UE CBW is 2 times of the UE CBW, all outer RB allocation converts to inner RB allocation and the MPR can be improved up to 2.5dB.

- When extended UE CBW is 1.5 times of the UE CBW, the MPR can still be improved up to 2dB although not all outer RB allocation are converted to inner RB allocation.

- When extended UE CBW is 1.25 times of the UE CBW, only small part of regions for outer RB allocation can get improvement.

That is, the MPR won’t reduce further when the gap between the edge of virtual CBW and the edge of configured UE CBW goes beyond 0.5*UE CBW on each side.

Therefore, the size of NRB,V and RBoffset need consider together. The size of RBoffset decides the lowest side gap between the lowest edge of virtual CBW and the lowest edge of configured UE CBW, and the size of NRB,V-RBoffset-NRB decides the highest side gap between the highest edge of virtual CBW and the highest edge of configured UE CBW, as shown in Figure 2-3. Considering it is unnecessary to introduce overlarge virtual CBW, RBoffset and NRB,V-RBoffset-NRB can go up to 0.5*NRB, NRB,V can go up to 1.5*NRB+ RBoffset. When RBoffset=0 RB, NRB,V can go up 1.5*NRB, When RBoffset=0.5*NRB, NRB,V can go up 2*NRB.

Figure 2-3 The gap between virtual CBW and configured UE CBW

Proposal 3: The virtual CBW (the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case) can be defined using the number of PRB (NRB,V) that has the same SCS as the configured UE CBW and without guardband.

Proposal 4: The network signals the number of PRB (NRB,V) on the virtual CBW and RBoffset that the lowest subcarrier on the virtual CBW towards the lowest usage subcarrier on the configured UE CBW.

Proposal 5: The limitation of RBoffset and NRB,V for the virtual CBW need meet following functions:

0≤RBoffset≤0.5*NRB

RBoffset+NRB≤NRB,V≤1.5*NRB+ RBoffset

Since the virtual CBW can be defined using the number of PRB (NRB,V), the extended IBE requirements can directly extend the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions:

max-25-10⋅log10(NRB,V/LCRB,V),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V,
-57dBm+10log10SCS/15kHz-PRB

And also apply to the guardband of the configured UE CBW, as shown in Figure 2-4.

Figured 2-4 The extended IBE requirement apply to guardband

Proposal 6: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions:

max-25-10⋅log10(NRB,V/LCRB,V),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V,
-57dBm+10log10SCS/15kHz-PRB

Proposal 7: The extended IBE requirements apply to the guardband of the configured UE CBW.

To convert outer RB allocation to inner RB allocation, it was agreed to shift the starting frequency of ACLR and SEM by extended UE CBW and no relaxation of ACLR/SEM/SE values. If we change the measurement bandwidth of ACLR from based on the original UE CBW to extended UE CBW, the ΔfOOB of SEM also need to change from based on the original UE CBW to extended UE CBW, it will cause further relaxation of the out of band emission. Therefore, the measurement bandwidth of ACLR should use the original UE CBW as the basis, as Figure 2-5:

Figure 2-5 the emission boundary area for extended UE CBW

Proposal 8: ACLR and SEM should be applicable from the edge of extended UE CBW and the OOBE requirements should be based on the configured UE CBW.

Proposal 9: The SE limits apply for the frequency ranges that are more than configured UE CBW+5MHz from the edge of the virtual CBW.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band, and proposed:

Proposal 1: The extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case can be notated as a virtual CBW, and the related parameters of the RB allocation can be notated as NRB,V, RBStart,Low,V, RBStart,High,V and LCRB,V, the conditions of Inner RB allocation for the virtual CBW are as following:

RBStart,Low,V  ≤  RBStart,V  ≤  RBStart,High,V, and

LCRB,V  ≤  ceil(NRB,V/2)

NRB,V is the maximum number of RBs for a virtual CBW, and

RBStart,Low,V = max(1, floor(LCRB,V/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High,V = NRB,V – RBStart,Low,V – LCRB,V

Proposal 2: The MPR enhancement region (the extended Inner RB region) should be the intersection of the Inner region relative to the virtual CBW and the Outer region and edge region relative to the configured UE CBW where the inner MPR can be expected from UE, where the virtual CBW is fully covering the configured UE CBW and is larger than configured UE CBW.

Proposal 3: The virtual CBW (the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case) can be defined using the number of PRB (NRB,V) that has the same SCS as the configured UE CBW and without guardband.

Proposal 4: The network signals the number of PRB (NRB,V) on the virtual CBW and RBoffset that the lowest subcarrier on the virtual CBW towards the lowest usage subcarrier on the configured UE CBW.

Proposal 5: The limitation of RBoffset and NRB,V for the virtual CBW need meet following functions:

0≤RBoffset≤0.5*NRB

RBoffset+NRB≤NRB,V≤1.5*NRB+ RBoffset

Proposal 6: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions:

max-25-10⋅log10(NRB,V/LCRB,V),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V,
-57dBm+10log10SCS/15kHz-PRB

Proposal 7: The extended IBE requirements apply to the guardband of the configured UE CBW.

Proposal 8: ACLR and SEM should be applicable from the edge of extended UE CBW and the OOBE requirements should be based on the configured UE CBW.

Proposal 9: The SE limits apply for the frequency ranges that are more than configured UE CBW+5MHz from the edge of the virtual CBW.

- Reference

[1] R4-2420527 WF on power domain enhancement, Huawei, RAN4 #113

[2] R4-2412351 R19 MPR reduction for single carrier, OPPO, RAN4 #112