3GPP TSG-RAN WG4 Meeting #114	R4-2501472
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.6.2

Source: 	OPPO

Title: 	Discussion on reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, the general issues on reducing the number of UE RC chains were discussed in WF [1]:

Agreements: Issue 2-1: Discussion on baseband assumptions - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 Issue 2-4: Signaling related - Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. Issue 3-1: Discussion on evaluation priority of example bands - It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase. Open issues Issue 2-7: Others - RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s Issue 3-5: Discussion on DL fragmented carriers in TDD bands Background: To consider transition time if there’s one LO for the TDD band and study the feasibility - Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation: - Option 1: One LO will be enough for both UL and DL. - Option 2: Separate LOs will be needed for UL and DL.

In this contribution, we further discuss the general issues on reducing the UE Rx chain for fragmented carriers.

- Discussion

In last meeting, the baseband assumption was discussed for fragmented carriers using one Rx RF chain, and it was agreed that single FFT or separate FFT can be left to UE implementation, but the condition of single FFT compared to separate FFT can be captured in the TR.

The subcarriers on different carriers need to guarantee orthogonality if using single FFT to treat multiple carriers for two fragmented carriers using one Rx RF chain. That is, the gap between two carriers cannot be arbitrary. And, in last meeting, it was agreed that consider only same SCS for both CCs for one Rx RF chain architecture. Therefore, BWgap+ GBChannel(1)+ GBChannel(2) should be an integer multiple of REs.

Proposal 1: BWgap+ GBChannel(1)+ GBChannel(2) should be an integer multiple of SCS for two fragmented carriers using one Rx RF chain and single FFT.

The performance will become worse due to there is no any suppression in the gap between two carriers when fragmented carriers sharing one Rx chain. From this point, it is better only to consider configure two fragmented carriers as purely DL scells.

Proposal 2: Considering configure two fragmented carriers as purely DL scells.

About the LO issue for two fragmented carriers in TDD bands, I think it is similar with the case that two intra-band contiguous DL CA with one UL in TDD band if there’s one LO, since two intra-band contiguous DL CA assumed using one Rx RF chain. In RAN 4, there is no any transition time and no LO number limitation for intra-band contiguous DL CA with one UL in TDD band. Therefore, for two fragmented carriers, it is unnecessary to define the LO number.

Proposal 3:  How many LOs are needed for fragmented carrier operation in TDD with one of the carriers having UL allocation should depend on the UE implementation.

- Conclusion

This paper discussed the general issues for fragmented carriers and proposed:

Proposal 1: BWgap+ GBChannel(1)+ GBChannel(2) should be an integer multiple of SCS for fragmented carriers using one Rx RF chain and single FFT.

Proposal 2: Considering configure two fragmented carriers as purely DL scells.

Proposal 3:  How many LOs are needed for fragmented carrier operation in TDD with one of the carriers having UL allocation should depend on the UE implementation.

Reference

[1] R4-2420519, WF on fragmented DL, MediaTek Inc., RAN4#113 2024-12