3GPP TSG-RAN WG4 Meeting #114	R4-2501473
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.6.3

Source: 	OPPO

Title: 	Discussion on impacts on UE RF requirements

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, the impacts on UE RF requirements and DL performance were discussed in WF [1]:

Agreements: Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode - Companies are encouraged to bring analysis with their own assumptions following the table format - A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1 Issue 3-4: Discussion on in-gap requirements RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier Open issues Issue 3-2: Discussion on maximum power level of in-gap interference - For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS

In this contribution, we further discuss the impact on UE DL RF requirements reducing the UE Rx chain for fragmented carriers.

- Discussion

To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers. Correspondingly, the LPF after the mixer will be adjusted from two LPFs to one wider bandwidth to cover two CC including the gap between two CCs, it will result in no any rejection in the gap between two CCs, as shown in Figure 2-1. And the Tx leakage will become obvious due to the suppression of the LPF with wider bandwidth become worse compared to two LPFs with narrower bandwidths.

Figure 2-1 The interferer in the gap between two CCs

Therefore, in previous discussion, the impact of Tx leakage, interferer in the gap, IMDs associated with the interferer in the gap and the receiver image located on one of two fragmented carriers were considered for two fragmented carriers using one Rx RF chain.

- The impact of the receiver image of one of two fragmented carriers located on another fragmented carriers in REFSENs test

In LTE, 25dB receiver image rejection was defined for DL intra-band C CA due to two component carriers are received the finite receiver image rejection will manifest itself such way that power from PCell will leak to SCell and vice versa, as shown in Figure 2-2:

Figure 2-2 The receiver image for intra-band C CA

From the definition in TS 36.101:

- Receiver image rejection ratio is the ratio of the wanted received power on a sub-carrier being measured to the unwanted image power received on the same sub-carrier when both sub-carriers are received with equal power at the UE antenna connector

We can see the ratio is per sub-carrier, here we use band n25 with 15kHz SCS as the example, the REFSENS and REFSENS/RE is shown in bellow table 2-1:

Table 2-1 REFSENS and REFSENS/RB for n25

| Operating Band |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n25 | REFSENS | -96.5 | -93.3 | -91.5 | -90.3 |
|  | REFSENS/RE | -121.3 | -121.3 | -121.3 | -121.3 |

Now we evaluate the impact of PCC image to the REFSENS of SCC for fragmented carriers, from above table, we can get the leakage power from PCC to SCC is -146.5 dBm/RB. The REFSENS/RB of SCC will still be -110.5dBm, the impact of receiver image to REFSENs can be ignored if 25dB rejection is assumed.

Observation 1: 25dB receiver image rejection for REFSENs test is enough, no REFSENS reduction for each fragmented carriers is needed.

Therefore, the impact of in-gap interferer and its image and IMDs associated with the interferer to the Rx requirements for PCC and SCC should be considered.

- The impact of Tx leakage in REFSENS test.

The Tx leakage will become obvious due to the suppression of the LPF with wider bandwidth become worse compared to two LPFs with narrower bandwidths. Maybe larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

Proposal 1: Larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

- The impact of in-gap blocker in ACS and IBB tests

The anther impact of the image for fragmented carriers is the blocker image that falling into the wanted received power for the ACS and IBB requirements test due to there is no in-gap filter.

As the analysis in [2], there are two cases for ACS and IBB testing:

Case 1: when testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap blocker may also fall into co-channel of CC0 as illustrated in Figure 3.

Case 2: On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of blocker may fall into the co-channel range of CC1 as illustrated in Figure 4.

Here we excerpt the figure from [2] to further analyse the image impact.

Since the conformance test does not measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, here we only analyse case 1 based on example band n25.

Here we summarized the parameters for ACS and IBBs test cases for single CC with 20MHz channel bandwidth in current Spec 38.101-1 as shown in Table 2-2:

Table 2-2 The existing parameters for ACS and IBBs test cases for n25 with 20MHz channel bandwidth

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| BWinterferer | 5MHz | 5MHz | 5MHz | 5MHz |
| Pwanted- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |

We assumed CC0 is 20MHz, when testing in-gap ACS on CC0, the image of interferer will fall into CC0 as Figure 3, based on Table 2-2 assuming 25dB receiver image rejection, we can get the related image power of the interferer and the worsen power in transmission bandwidth configuration as shown in table 2-3:

Table 2-3 The parameters with the image impact for ACS and IBBs test cases for n25 with 20MHz channel bandwidth

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pimage | -75.8 | -50 | -81 | -69 |
| Pwanted+Pimage (dBm) | -73 | -47.2 | -78 | -68.8 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| (Pwanted+Pimage)- Pinterferer (dB) | -22.2 | -22.2 | -22 | -24.8 |

Compared Table 2-3 with table 2-2, we can see the deterioration of ACS and IBB performance and it cannot meet current requirements

Table 2-4 the deterioration of ACS and IBB performance

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |
| (Pwanted+Pimage)- Pinterferer (dB) | -22.2 | -22.2 | -22 | -24.8 |
| Deterioration (dB) | 3.3 | 3.3 | 3.3 | 12.5 |

Observation 2: Assuming 25dB receiver image rejection, ACS and IBB1 performance decreased by 3.3 dB and IBB2 performance decreased by 12.5 dB.

If we keep parameters for ACS and IBBs test cases unchanged, the receiver image rejection needs up to 46dB to meet current requirements for ACS and IBBs as Table 2-5 calculated, it’s hard to achieve for current component performnce:

Table 2-5 The Receiver image rejection without deterioration of ACS and IBB performance

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pimage | -82.2 | -82.5 | -90 | -90 |
| Pwanted+Pimage (dBm) | -75.3 | -50.5 | -81.3 | -81.3 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| (Pwanted+Pimage)- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |
| Receiver image rejection | 31.4 | 57.5 | 34 | 46 |

Proposal 2: The relaxation of the in-gap ACS/IBB is needed, how to relax the requirement can further study.

- Conclusion

This paper discussed the RF requirements for fragmented carriers and proposed:

Observation 1: 25dB receiver image rejection for REFSENs is enough, no REFSENS reduction is needed.

Proposal 1: Larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

Observation 2: Assuming 25dB receiver image rejection, ACS and IBB1 performance decreased by 3.3 dB and IBB2 performance decreased by 12.5 dB.

Proposal 2: The relaxation of the in-gap ACS/IBB is needed, how to relax the requirements can further study.

Reference

[1] R4-2420519, WF on fragmented DL, MediaTek Inc., RAN4#113 2024-12

[2] R4-2416164, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc., RAN4#112bis 2024-10