3GPP TSG-RAN WG4 Meeting #114	R4-2501476
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.10.2.3.2

Source: 	OPPO

Title: 	Discussion on Rx requirements for IoT-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Rx RF requirements with IoT NTN operating in IoT-NTN bands were discussed and this is no any agreement in WF [1]:

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based) - Proposals - N256: - 0 dB (vivo, oppo, Ericsson) - [1] dB (Qualcomm) - N255: - 0 dB (oppo) - Using n3 RSD as reference (Samsung, last meeting) - N254: - [0] dB (vivo, Samsung, Ericsson) - WF - N254: [0] dB - Other bands FFS; Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based) - Proposals - Option 1: 0dB for n254/n255/n256 (vivo, oppo) - Option 2: others - WF - [0] dB for n254/n255/n256 Issue 3-1-x: General for RSD - Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

This contribution further discussed the UE Rx RF requirements for IoT NTN HPUE.

- Discussion

For IoT NTN, UE category M1 can operate in both of HD-FDD mode and FD-FDD mode, and Category NB1 and NB2 UE operate in HD-FDD duplex mode.

Therefore, only UE category M1 operating in FD-FDD mode need consider the RSD due to the higher output power.

For the handheld IoT-NTN UE, it can use similar RF components and designs as NR-NTN HPUE devices, it can use the similar RSD with NR NTN HPUE.

In previous meeting, it was agreed [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx in band n256. And PC2 IoT NTN can reuse a similar architecture to PC2 NR-NTN, therefore, 0dB RSD can reuse for PC2 IoT-NTN for 1Tx and 2Tx in band 256.

Proposal 1: 0dB RSD can be specified for PC2 handheld IoT NTN UE (M1) operating in FD-FDD mode in 256 and 255.

- Conclusion

In this paper we give further discussion on the RX requirement for IoT NTN HPUE. The proposals are as following:

Proposal 1: 0dB RSD can be specified for PC2 handheld IoT NTN UE (M1) operating in FD-FDD mode in 256 and 255.

- Reference

[1] R4-2420404, WF on HPUE for IoT NTN, vivo, RAN4#113 2024-11.