3GPP TSG-RAN WG4 Meeting #113	R4-2501482

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Adaptation of NR NTN mobility requirements for less than 5 MHz operation

Agenda item:	7.10.3.4

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Throughout the first meetings in this WI good progress was achieved by reusing most of the requirements from the less than 5 MHz WI for terrestrial networks with the necessary adaptation for NTN. There are a few open issues left, one of them, which is the topic of this document is related to requirements for SSB time index detection.

Discussion

Another open issue is related to the Index detection requirements during neighbor cell measurements. For TN deployments in less than 5 MHz, the Index detection was increased in 4 (intra-frequency measurements) and 3 (inter-frequency measurements).  Based on this, two set of options were proposed:

Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

There is a consensus in the companies to maintain the baseline that was set based on the number of samples used for less than 5 MHz operation in Rel-18. Even though the scenario and channels observed in TN that based the simulation models are different from those observed for NTN, there was a consensus in the group to utilize the same side conditions for this WI. Therefore, it makes sense to re-utilize the same amount of samples established before.

However, the problem is that if this is the sole approach adopted, it will lead to a significantly high number of samples on the time to identify the SSB of the measured cells, which will lead up to increasingly high mobility times.

In scenarios where completion time is paramount, it's more effective to thrive for faster completion times. This is the case of NTN, where the satellite might be available for very limited span of times, and in certain scenarios (such as earth moving cells) a significant amount of mobility events are to be conducted, and shortened interruption times are preferred.

Even more so in NTN where the delays are further increased already due to the "split" of measurement opportunities across different satellites. This delay will be further exacerbated by the "DL Coverage enhancement" feature in Rel-19.

RAN1 has agreed on SSB periods of 160ms (with cell DTX). Increasing the measurement delay to 320ms would add at least 640ms to UE measurement operations, and this delay can be significantly worse when measurements across multiple satellites are required.

While the DL coverage enhancement is not part of this RAN4-led work item, its combined implementation with Less than 5 MHz over NTN in Rel-19 is a possibility, and it will not be precluded when the specification of Release 19 has ended. If RAN4 can anticipate a harmonious deployment of both features without additional cost, a more efficient system can be achieved for all companies.

The main objections raised by companies to adopt a second – faster – completion time is related to the following:

- Potential lack of reference values where the side conditions can be used to establish the number of samples needed, which would require additional simulation effort.t

- It may require more complex implementation of some UEs which are only capable of doing the “slower” set of measurements.

We will address both of these concerns in this document. The first one, related to the lack of measurements, might be addressed by the same document that was used as baseline for creating the original requirements for Less than 5 MHz in TN. In the R4-2321344, which provides simulation summary from several companies results for one-shot detection of PBCH are provided for both -6dB and -4dB. The document also provides the reference value for one-shot detection in dB for different channels: TDLC300 (1 dB), HST (-3.2 dB), AWGN (-4.3 dB).

One important point to remark is that the TDLC300 channel model is used to establish the baseline of the requirements, even though, the NTN channel model TDLC5, which has much smaller delay spread is closer to the HST and AWGN models. But even if we stick to the TDLC model, based on the previous references, we can establish that, we have reference values for a set of measurements at -6 dB and at -4 dB. It has also been established by specification that when the Es/IoT is above -2 dB the UE is capable of decoding PBCH in 3 samples:

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Therefore, we have the following agreed and well established set of number of needed sambles per Es/IoT condition: 7 samples for Es/IoT -6 dB, 6 samples for -4 dB and 3 samples for -2 dB.  From that, we can establish that a set of faster samples might be provided at -2dB. For compromise we would consider to have 4 instead of 3 samples considered for the set of faster measurements.

In what regards the second objection raised by companies that some UEs might not be designed to comply with the set of faster measurements, we would like to offer a compromise that the faster measurements are to be accomplished only by UEs that declare the capability to do so.

Based on that, we propose the following:

Favor option 2, where two sets of requirements for Tidentify_xxx_with_index  are proposed.

Introduce a new capability for supporting the new set of requirements

The set of faster requirements require 4 samples to complete Tidentify_xxx_with_index  when the Ês/IoT is equal to -2 dB.

Conclusion

In this paper we provided a detailed discussion on the problem of the duration of

Proposal 1: Favor option 2, where two sets of requirements for Tidentify_xxx_with_index  are proposed.

Proposal 2: Introduce a new capability for supporting the new set of requirements

Proposal 3: The set of faster requirements require 4 samples to complete Tidentify_xxx_with_index  when the Ês/IoT is equal to -2 dB.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2413974, "WF for RRM of NTN with channel BW less than 5MHz", Moderator (Xiaomi), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R4-2413189, “Support of less than 5MHz for NR NTN”, Qualcomm, RAN4 #112, Maastricht, Netherlands, 19th – 23rd August, 2024