3GPP TSG-RAN WG4 Meeting #113	R4-2501488

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	The impact of the NTN IOT TDD Periodic Pattern on RRM requirements

Agenda item:	7.31.5

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts of such frame and provided their view on [2].

In the plenary meeting RAN #106, the WID was updated to provide the objectives for RAN4 based on identified challenges [3]. In this document we discuss the potential RRM impacts.

Discussion

In RAN4 #113, when the group of companies debated the new frame structure being proposed as part of the IOT_NTN_TDD Work Item, several potential impacts for RAN4 requirements were identified related to RF, RRM and Demod aspects.  In this document we will focus our analysis on the RRM impacts. Currently the identified issues, according to [2] are:

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

In this document we will provide our views in some of the requirements above.

On the DRX Configurations and measurement related requirements

Some of the issues identified above, as seen from discussion papers provided by companies are related to potential  impact on DRX measurements caused by “unmatched” patterns of DRX Configuration and the N=9 frames cycle, which creates periodicities that are non-multiple of each other. For example, for the case of cell evaluation for cell reselection, where DRX Cycles may have periodicities that are multiple of 320 ms, and therefore, non-multiple of 90 ms. This would indicate that the active part of the DRX Cycle slips slightly over time every cycle of N frames, and cannot be matched with the active time on this cell.  For example, there are potential changes on the HFN calculation, which might impact the frame counting:

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

The active part of the DRX Cycle will slide over time and in certain cases would correspond to the “non-active” part of the cell.

However, our understanding is that RAN1 is currently working in how to deal with such problems and is likely to come up with solutions that align DRX cycles with monitoring of the cell.

For requirements related to the configuration of the DRX Cycle, wait for RAN1 conclusion on how to align the periodicity of the cycle with the cell transmit times.

In the following subsections, the requirements are discussed assuming alignment between DRX Cycle and periodicity of the active period of the cell.

RLM Measurements

The RLM measurement requirements for NB1 UEs connected over NTN is found in TS 38.133, clause 7.23A and are specified for two different scenarios, with DRX and non DRX Cycle configured.

The pre-conditions for RLM monitoring establish that requirements are applicable provided “at least 1 DL subframe per radio frame of serving NB-IoT cell is available at the UE during Qout_NB-IoT and Qin_NB-IoT evaluation periods”. This pre-condition does not seem to pose additional challenges for the IoT TDD operation, as currently there will be 8 DL subframes every 90 ms, and 90 ms is smaller than the evaluation periods mentioned above.

There will be at least one DL subframe available per evaluation period, but not per radio frame, as some of the radio frames will not be transmitted.

- For the non-DRX case, the default values for the evaluation periods are provided by Table 7.23A.2.1-1 and reproduced below for convenience.

Table 7.23A.2.1-1 Qout and Qin Evaluation Period in non-DRX for Category NB1 UE Configured NPDCCH Rmax; TEvaluate_Qout_NB-IoT; TEvaluate_Qin_NB-IoT. Rmax ≤ 64; 400ms; 200ms. Rmax> 64; 4000ms; 2000ms.

It is first important to establish whether the total number of repetitions might be as large as 64 (or more) even when the only blocks of 8 consecutive UL or DL subframes are expected.  This subject is currently being investigated in RAN1 with the following way forward:

RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

For the time being, values which consider R>64 are currently out of scope of this analysis, which will focus on R ≤ 8.

At every 90 ms, the UE will have an opportunity to measure DL subframes, which means that there will be at least 3 opportunities of Qin evaluation within the evaluation period, and 5 opportunities of Qout evaluation within the evaluation period. Therefore, in our assessment, is not necessary to update this requirement.

On the other hand, it might be necessary to update this requirement depending on how RAN1 decides to handle the number of repetitions that exceed the available DL frame size.

Do not update the RLM requirements when DRX Cycle is not configured, unless RAN1 changes cause impact to this requirement.

Likewise, for the case when the DRX Cycle is configured, the evaluation periods are configured as multiples of the DRX Cycle (5, 10 or 20, depending on the duration of the DRX Cycle). Assuming a minimum DRX Cycle of 256 ms, there will be several opportunities for UE measurements within the evaluation period.

For RLM requirements when DRX Cycle is configured, wait for RAN1 conclusions on whether DRX settings will be modified for IoT TDD.

RRC Connection redirection to Non-Anchor Carrier.

The requirements for RRC Connection Redirection to Non-Anchor Carrier establish the maximum delay in time for the UE to complete the transition (RRC Redirection) to the Non-Anchor Carrier upon receiving the reconfiguration indication through the IE “CarrierConfigDedicated-NB”.  This delay depends on several components, as described below:

6.9A.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB [2] and can be 10 ms or 40 ms. TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

As we highlight above, two of the parameters need to be revisited during this WI, as they are impacted by the periodic pattern introduced.

The Tperiod_DL_bitmap defines the periodicity of the DL subframe configuration on the non-anchor carrier. It could be previously configured for 10 or 40 ms. However, in the current WI, due to periodic patterns with cycle periods of 9 radio frames, the “off time”  of a cell might be close to 90 ms. Therefore, the value of the parameter requires updating.

Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Also the TDL-UL switch  which measures the time between the end of the last subframe in the repetition period of the NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the NPUSCH transmitted on the non-anchor carrier requires updating. Currently the delay value is set to 8 ms. But in practice this value might be much longer than 8 ms, as there might be a gap between UL and DL active frames in the TDD periodic pattern.

Update TDL-UL switch to a value compatible with a period of 90 ms.

Cell Measurements

In Clause 8.14A, it is established that the measurements of the serving cell are applicable provided that some side conditions are fulfilled:

The UE shall meet all applicable requirements specified in clause 8.14A under the following conditions: - at least 1 DL subframe per radio frame of serving NB-IoT cell is available at the UE during measurement period. - Valid information for the serving satellite has been provided

The first condition might be a non-reachable condition for the new IoT TDD frame. The reason is that only one or two frames are partially transmitted at every 90 ms. Therefore, the condition might never be fulfilled if the radio frames not transmitted by the network are considered as unavailable. The only scenario where this condition can be achieved depends on whether RAN1 adapts the radio frame counting such that only active radio frames are counted or considered valid ones.

Revise the cell measurements condition where at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period.

The second condition is also a critical one, and it was introduced in Rel-17, such that the UE can only be required to perform accurate cell measurements if provided with satellite ephemeris, such that Doppler delay can be compensated. For the new frame structure, though, it is important to remember that several synchronization signals will be “not-transmitted” within a period of N=9 radio frames. So in order to assist UE measurements:

Serving cell measurement requirements apply provided the UE is provided with the configuration about the “active DL” periods.

Conclusion

In this paper we provided our views on the impacts caused to RRM requirements by changes on the TDD periodic pattern in NTN. The following proposals and observations were made:

Observation 1: The active part of the DRX Cycle will slide over time and in certain cases would correspond to the “non-active” part of the cell.

Proposal 1: For requirements related to the configuration of the DRX Cycle, wait for RAN1 conclusion on how to align the periodicity of the cycle with the cell transmit times.

Observation 2: There will be at least one DL subframe available per evaluation period, but not per radio frame, as some of the radio frames will not be transmitted.

For the non-DRX case, the default values for the evaluation periods are provided by Table 7.23A.2.1-1 and reproduced below for convenience.

Proposal 2: Do not update the RLM requirements when DRX Cycle is not configured, unless RAN1 changes cause impact to this requirement.

Proposal 3: For RLM requirements when DRX Cycle is configured, wait for RAN1 conclusions on whether DRX settings will be modified for IoT TDD.

Proposal 4: Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Proposal 5: Update TDL-UL switch to a value compatible with a period of 90 ms.

Proposal 6: Revise the cell measurements condition where at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period.

Proposal 7: Serving cell measurement requirements apply provided the UE is provided with the configuration about the “active DL” periods.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2419795,  Way Forward for [113][317] IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  RAN4 #113, Orlando, US, 18th – 22nd November, 2024

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, RAN#106, December 2025

- R1-2409038, “Feature lead summary #1 on IoT-NTN TDD mode” , Qualcomm, RAN1#118bis

- R1-2409341, “Report of RAN1#118bis meeting”, RAN1 #119, preview on RAN1 Chair’s Notes found in: https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_118b/Inbox/Chair_notes/Chair%20notes%20RAN1%23118bis%20eom0.zip