3GPP TSG-RAN WG4 Meeting #114							         			R4-2501489

Athens, GR, 17th – 21st February 2025

Title: 	BW or RB extension approach for improving MPR when NW BW is higher than UE CBW

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.2.1	Power domain enhancements for single carrier	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In R4#113, we provided further details on our proposal to signal extended emission BW for UEs based on RB extension [1]. Additionally, there were questions as to whether the proposal should only be relative to the UE CBW as captured in way forward [2] together with high-level agreements. In this contribution, we further explain the concept of extended bandwidth based on RB extension, and its relation to the permitted emission bandwidth.

# Discussion

There were only several agreements related to the extended UE bandwidth in way forwards [2] worth copying here. The rest being more the collection of options on the table.

Scenarios discussed in previous RAN4 meetings:

- Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

- Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

- Scenario 2: Narrower UE channel BW within wider BS bandwidth

#### Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation

Agreement in main session:

- Extended CBW based approach

  - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE

  - Both scenario 1 and 2 should be covered

  - The network should ensure the co-existence be met when signaling the extended CBW.

## 2.1 Using CBW or NRB Extension

In the last meeting there was an argument that bandwidth extension should be expressed in term of channel bandwidth (ratio) and not in term or number of RBs. In fact, the two are equivalent, as the RB bandwidth can be deducted from the channel BW, minus the left and right guard bands. The advantage of expressing the UE BW extension in terms of NRB is that it offers great granularity, and the number of RBs is what allows the UE to calculate inner and outer allocations. However, when a given RB extension is provided, it is essential that the associated guard band is known and sufficient to guarantee the emissions based on the UE channel bandwidths that have been tested.

Table 1 provides the NRB and guard bands for the different channel bandwidths and determines what is the safe largest guards band for any NRB in the following manner: For a given number of RBs, the maximum of the guard band (GBmax) of the next lower CBW, and next higher channel BW, is taken and the BS needs to ensure that such guard band is available for the UE to meet emissions requirements.

Table 1: NRB and guard bands vs CBW and calculated max guard band between consecutive CBW

| SCS [kHz] | CBW [MHz] | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | NRB | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | GB [kHz] | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N.A | N.A | N.A | N.A | N.A |
|  | GBmax [kHz] | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 592.5 | 572.5 | 712.5 | 712.5 | 692.5 | N.A | N.A | N.A | N.A | N.A |
| 30 | NRB | N.A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
|  | GB [kHz] | N.A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
|  | GBmax [kHz] | N.A | 665 | 665 | 805 | 805 | 945 | 945 | 925 | 1065 | 1065 | 1045 | 965 | 965 | 925 | 885 | 845 |

For example, the UE CBW is 20MHz (106RB at 15kHz) and the RB extension is 40RB, this means that the total number of RB is 146RB. From the table below, this corresponds to:

- Next lower channel BW is 25MHz and the next upper is 30MHz (green highlights).

- As long as the guard band available for the emissions is ≥592.5kHz, the emissions can be met, since this is the guard band applicable to the next higher CBW.

- Note that there are cases where the guard band decreases with increased CBW, however, this is not an issue with our proposal, since we take the maximum guard band out of the next lower and next higher CBW.

In the case where the extended bandwidth corresponds to a contiguous CA case larger than 100MHz, the CA bandwidth and channel spacing ensures that the highest guard band of the two CCs is available, and thus, since we are focusing on a single CC UL, the guard band needed for emissions is granted.

In all cases, it is the BS responsibility to ensure the RB extension enables the highest guard band to the emission bandwidth between the next lower and next higher CBW, compared to the sum of the UE RB and the two sides RB extensions.

One aspect related to the emissions is that a UE with extended RB bandwidth has a lower emission bandwidth as compared with a UE that would use a channel bandwidth equal to extended bandwidth.

This is illustrated in Figure 1 below (note that the drawing is not to scale):

- In green at the bottom is the UE RBs corresponding to the UE CBW (for example 106RB for a 20MHz channel)

- In yellow the left and right RB extensions (let’s say 30RB on the left and 24RB on the right)

  - In grey, the ACLR1 and ACLR2 regions, based on the extended bandwidth.

  - The dashed red line represents the SEM mask associated with the extended bandwidth.

- The sum of the green+yellow RBs is 160, which correspond to a 30MHz CBW.

- In red, the worst case wanted RB and its image for the (20MHz CBW) UE operating with the extended bandwidth (LO is centered on the UE CBW)

  - The related IMD3 and IMD5 are shown in brown.

- In blue, a virtual UE using the entire extended bandwidth (equivalent to 30MHz CBW)

  - The related IMD3 and IMD5 are shown in dark blue.

Figure 1: Comparison between the emission bandwidth of a UE with extended bandwidth (red) versus a virtual UE using the entire extended bandwidth (blue).

It can be observed that:

- The IMD3 and IMD5 of the red UE falls within ACLR1 of the extended bandwidth.

- The IMD3 of the virtual blue UE using the entire extended bandwidth fall in ACLR1 while IMD5 falls in the ACLR2 region.

- This is why the red allocation that would be an outer based on the UE CBW can be considered an inner with the extended bandwidth

- If the BS ensures that the left and right extended RB values gurantees that the guard band to the emission bandwidth is higher or equal to the maximum guard band value of 25MHz and 30MHz CBW, then the red UE may redefine its outer allocation to inner, based on the extended RB and meet emissions requirements.

  - Note that this guard band may be related to the bandwidth in which the BS operates. However, in some case, it could be the guard band to a band to be protected.

Observation: the emission bandwidth of a UE with extended bandwidth is lower than the equivalent UE using the entire extended bandwidth providing additional margin on emissions and enabling turning some/all outer allocations into inner allocations.

Based on this guaranteed guard band approach, the UE that has been tested for emissions, up to a maximum CBW can meet the emission requirements for any extended bandwidth between two supported CBWs when redefining its outer/edge allocations to inner allocations.

Proposal: bandwidth extensions are expressed in a number of RBs, and the related emission bandwidth is known, as the BS guarantees that the available guard band is larger than the larger guard band between:

- The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB).

- The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB).

- In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region.

- For extensions beyond 100MHz CBW, the guard band for emissions is guaranteed by the contiguous CA BW and channel spacing equations.

## 2.2 Signalling of extended BW with RB extension

Based on the above proposals, we further evaluate RB extension options for different UE positions within a higher BS bandwidth.

Several cases are illustrated in Figure 2. For the allocation triangles, the BS-related triangle is stopped at NRB/2 as there no inner allocations above that. For the example, the number of RBs is kept minimal and does not correspond to any existing CBW. Additionally, the corresponding UE guard band is set to 1RB (GB1, GB2). As well, the UEs position is random within the BS RBs.

Figure 2: Inner/outer allocations for two UEs within a larger BS BW before RB extension (top) and after (bottom)

Illustration:

- The BS NRB (NRB_NW in orange) is 32RB and the related GBNW (light orange) is higher than the maximum guard band of the next lower/next higher CBW, as related to the UEs extended bandwidths.

  - The related inner/outer/edge allocations are denoted “i”/”o”/”e” in the allocation triangle.

- UE1 has 8RB (NRB_UE1 in blue) at an offset of 4RB within the BS RBs.

- UE2 has 8RB (NRB_UE2 in brown) at an offset of 4RB within the BS RBs.

  - The UEs CBW-related inner/outer/edge allocations are in light green/yellow/red in the top triangle.

  - The UEs extended BW related inner/outer allocations are in dark green/orange in the bottom triangle.

- The required RB extension to be signalled to the UE by the BS is illustrated in the middle as follows:

  - RBextL_UE1 and RBextH_UE1 in blue for UE1.

  - RBextL_UE2 and RBextH_UE2 in brown for UE2.

Observation:

- UE1 can “turn” all its allocations to inner.

- UE2 only has several outer allocations left.

- Left and right RB extensions beyond NRB_UE/2, are not useful as all outer and edge allocations are already “turned” to inner.

- UEs in different regions of the higher BS bandwidth will have different left and right RB extensions.

- UEs placed at the BS bandwidth edge will only have an RB extension on one (opposite edge) side. This is the case for UEs that use one CC of a BS supporting intra-band contiguous CA.

- It is essential that RB or BW extension are not set to NRB/2 or CBW/2 only, as this is an exception that the UE would be positioned in the middle of the BS BW, and if this is the case, then would be UEs on each side which would not be able to benefit from it.

Based on the above, we can establish a number of rules on signalling RB extension.

Proposal on RB extension:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH

- Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW.

- Inner and outer allocation can be redefined by the UE, using the existing inner/outer allocation equations where:

  - NRB is replaced by NRB+RBextL+RBextH

  - RBstart is replaced by RBstart+RBextL

- With 273RB for 100MHz CBW, the maximum RB extension value is 137, thus can be signalled with 8 bits, corresponding to 16bit total for the lower and higher side RB extensions. For 15kHz SCS cases, 50MHz has NRB/2=135, thus the same number of bits.

- Note that for the 100MHz case in 30kHz SCS and 50MHz in 15kHz SCS, the RB extension would correspond to BS bandwidth cases with DL CA or multiple channels (for example band n41 100+90MHz case in the US, or 100+60MHz case in China)

- several unused codes can be used as “short-cut” signalling: for example, all 1s would mean that all allocations can be turned to inner.

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered:

  - One where the UE is able to reassign outer/edge to inner, based on 1RB granularity of signalled RB extension.

  - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs).

## 2.2 Covering IBE in extended region.

In our discussions in the last meeting there was consensus that IBE must be met within the extended region. In our view, this is not an issue since IBE must be met by inner allocations and, as illustrated in Figure 1, the IMD3/5 of the UE under extended BW operation has a smaller frequency range than a UE using the entire extended bandwidth. Nevertheless, while limiting the RB extension to NRB_UE/2, the extended region may not cover the entire BS RBs, but at the same time will have less emissions than a UE that would use the entire extended bandwidth.

Proposal: If the region where IBE must be met should be redefined, this should account for the fact that the RB extensions are limited to NRB_UE/2.

# Conclusion

In this contribution, we further explained and defined our proposals from [1] based on last meeting discussions.

Proposal: bandwidth extensions are expressed in a number of RBs, and the related emission bandwidth is known, as the BS guarantees that the available guard band is larger than the larger guard band between:

- The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB).

- The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB).

- In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region.

- For extensions beyond 100MHz CBW, the guard band for emissions is guaranteed by the contiguous CA BW and channel spacing equations.

Proposal on RB extension:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH

- Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW.

- Inner and outer allocation can be redefined by the UE, using the existing inner/outer allocation equations where:

  - NRB is replaced by NRB+RBextL+RBextH

  - RBstart is replaced by RBstart+RBextL

- With 273RB for 100MHz CBW, the maximum RB extension value is 137, thus can be signalled with 8 bits, corresponding to 16bit total for the lower and higher side RB extensions. For 15kHz SCS cases, 50MHz has NRB/2=135, thus the same number of bits.

- Note that for the 100MHz case in 30kHz SCS and 50MHz in 15kHz SCS, the RB extension would correspond to BS bandwidth cases with DL CA or multiple channels (for example band n41 100+90MHz case in the US, or 100+60MHz case in China)

- several unused codes can be used as “short-cut” signalling: for example, all 1s would mean that all allocations can be turned to inner.

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered:

  - One where the UE is able to reassign outer/edge to inner, based on 1RB granularity of signalled RB extension.

  - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs).

Proposal: If the region where IBE must be met should be redefined, this should account for the fact that the RB extensions are limited to NRB_UE/2.

# References

[1] R4-2418015 Redefining allocation types for improved MPR with BS BW larger than UE CBW, Skyworks Solutions Inc., R4#113

[2] R4-2417792 WF on Power domain enhancement, Huawei, HiSilicon, R4#113