3GPP TSG-RAN WG4 Meeting #114								R4-2501493
Athens, GR, 17th – 21st February, 2025

Title:	R19 MPR for PC1.5 NCCA

Source:	OPPO

Agenda item:	7.1.1.1.2

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA and NCCA MPR based on WF [1] and also requirement clarifications in [2][3].

This paper is based on [4].

- MRP simulation results

The configurations for MPR simulation are:

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- Different CBW and GAP combinations

- scs is 15khz

- LO number include 2LO, and 1LO+TxD

- Modulations QPSK in both CCs

- Equal PSD

- Target -30dBm/MHz SEM requirements

The simulation results can be found in:

- Annex A: DFT-S-OFDM MPR simulation results

- Annex B: CP-OFDM MPR simulation results

Based on simulation results in [2] it was observed that:

- the MPR results are similar for QPSK and 64QAM, therefore here only QPSK is further analysed.

- for the case that DC leakage falls into the SEM region, stricter LO leakage has been adopted.

- the results are similar for DFT-S-OFDM and CP-OFDM.

Below figure 1 use the max value from DFT and CP OFDM for analysis.

Figure 1 NC CA MPR results

The proposed MPR values for PC1.5 NC CA are as below:

- If dualPA-Architecture is not indicated (1LO+TxD)

MPR =   22;		0 < B < 0.54

20;		0.54 =< B < 6.48

16;		6.48 =< B < 16.56

14;		16.56 =< B < 21.6

12;		21.6 =< B

- If dualPA-Architecture is indicated (2LO)

MPR =     12;		0 < B < 2.16

10;		2.16=< B

Proposal 1:         Consider MPR results for NCCA as above with equal PSD to meet -30dBm/MHz SEM requirements.

For the unequal PSD cases, more simulations are needed.

- Conclusion

This paper analyzed the MPR for PC1.5 intra-band NC CA, got below proposals.

Proposal 1:         Consider MPR results for NCCA as below with equal PSD to meet -30dBm/MHz SEM requirements.

- If dualPA-Architecture is not indicated (1LO+TxD)

MPR =   22;		0 < B < 0.54

20;		0.54 =< B < 6.48

16;		6.48 =< B < 16.56

14;		16.56 =< B < 21.6

12;		21.6 =< B

- If dualPA-Architecture is indicated (2LO)

MPR =     12;		0 < B < 2.16

10;		2.16=< B

- Reference

[1] R4-2414277 WF on HPUE for CA in TN, Samsung, #112 2024-08

[2] R4-2412349 R19 MPR for PC1.5 contiguous CA, OPPO, #112 2024-08

[3] R4-2412350 R19 MPR for PC1.5 NC CA, OPPO, RAN4#112 2020-08

[4] R4-2417914 R19 MPR for PC1.5 NCCA, OPPO, RAN4#113 2020-11

Annex A: dft-s-ofdm with 1LO/2LO MPR simulation results

|  |  |  |  |  | 2LO (CC1 CBW + GAP + CC2 CBW) |  |  |  | 1LO + TxD (CC1 CBW + GAP + CC2 CBW) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 20+20+40 | 20+25+40 | 20+40+40 | 20+45+40 | 20+20+40 | 20+25+40 | 20+40+40 | 20+45+40 |
| 106 | 1 | 216 | 1 | 57.96 | 3.3 | 3.3 | 3.3 | 3.3 | 6.5 | 5.8 | 6.5 | 5.8 |
| 106 | 1 | 150 | 66 | 46.08 | 3.3 | 3.3 | 3.3 | 3.3 | 6.5 | 5.8 | 6.5 | 5.8 |
| 100 | 1 | 100 | 116 | 36 | 1.9 | 1.2 | 1.2 | 1.2 | 6.5 | 5.8 | 6.5 | 5.8 |
| 72 | 1 | 50 | 116 | 21.96 | 2.3 | 1.9 | 1.9 | 1.9 | 6.5 | 5.8 | 6.5 | 5.8 |
| 61 | 1 | 61 | 157 | 21.96 | 2.3 | 1.9 | 1.9 | 1.9 | 6.6 | 5.8 | 6.6 | 5.8 |
| 60 | 1 | 60 | 156 | 21.6 | 2.3 | 1.9 | 1.9 | 1.9 | 6.5 | 5.8 | 6.5 | 5.8 |
| 46 | 1 | 46 | 170 | 16.56 | 2.3 | 1.9 | 1.9 | 1.9 | 9.4 | 9.4 | 9.4 | 9.4 |
| 32 | 1 | 60 | 156 | 16.56 | 2.8 | 2.3 | 2.3 | 2.3 | 9.4 | 9.4 | 9.4 | 9.4 |
| 32 | 1 | 32 | 184 | 11.52 | 2.8 | 2.3 | 2.3 | 2.3 | 10.6 | 10.6 | 10.6 | 10.6 |
| 28 | 1 | 28 | 188 | 10.08 | 2.8 | 2.3 | 2.3 | 2.3 | 10.6 | 10.6 | 10.6 | 10.6 |
| 24 | 1 | 32 | 184 | 10.08 | 2.8 | 2.3 | 2.3 | 2.3 | 12.0 | 12.0 | 12.0 | 12.0 |
| 18 | 1 | 18 | 198 | 6.48 | 3.3 | 2.3 | 2.3 | 2.3 | 12.0 | 12.0 | 12.0 | 12.0 |
| 15 | 1 | 15 | 201 | 5.4 | 3.3 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 14 | 1 | 16 | 200 | 5.4 | 3.3 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 14 | 1 | 14 | 202 | 5.04 | 3.3 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 18 | 198 | 5.4 | 3.3 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 16 | 200 | 5.04 | 3.3 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 12 | 204 | 4.32 | 3.8 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 9 | 1 | 9 | 207 | 3.24 | 3.8 | 2.8 | 2.8 | 2.8 | 13.6 | 13.6 | 13.6 | 13.6 |
| 8 | 1 | 8 | 208 | 2.88 | 4.4 | 3.3 | 3.3 | 3.3 | 13.6 | 13.6 | 13.6 | 13.6 |
| 6 | 1 | 12 | 204 | 3.24 | 3.8 | 3.3 | 3.3 | 3.3 | 13.6 | 13.6 | 13.6 | 13.6 |
| 6 | 1 | 6 | 210 | 2.16 | 3.8 | 3.3 | 3.3 | 3.3 | 15.6 | 15.6 | 15.6 | 15.6 |
| 4 | 1 | 8 | 208 | 2.16 | 3.8 | 3.3 | 3.3 | 3.3 | 15.7 | 15.6 | 15.6 | 15.6 |
| 4 | 1 | 4 | 212 | 1.44 | 4.4 | 3.8 | 3.8 | 3.8 | 15.6 | 15.6 | 15.6 | 15.6 |
| 3 | 1 | 3 | 213 | 1.08 | 4.4 | 4.4 | 4.4 | 4.4 | 15.7 | 15.6 | 15.6 | 15.6 |
| 2 | 1 | 4 | 212 | 1.08 | 4.5 | 3.9 | 3.9 | 3.9 | 15.7 | 15.6 | 15.6 | 15.6 |
| 2 | 1 | 2 | 214 | 0.72 | 4.5 | 3.8 | 3.8 | 3.8 | 15.7 | 15.6 | 15.6 | 15.6 |
| 1 | 1 | 2 | 214 | 0.54 | 5.2 | 3.8 | 3.8 | 3.8 | 15.8 | 15.6 | 15.6 | 15.6 |
| 1 | 1 | 1 | 215 | 0.36 | 5.2 | 4.4 | 4.4 | 4.4 | 18.4 | 18.2 | 18.2 | 18.2 |

Annex B: cp-ofdm with 1LO/2LO MPR simulation results

|  |  |  |  |  | 2LO (CC1 CBW + GAP + CC2 CBW) |  |  |  | 1LO + TxD (CC1 CBW + GAP + CC2 CBW) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 20+20+40 | 20+25+40 | 20+40+40 | 20+45+40 | 20+20+40 | 20+25+40 | 20+40+40 | 20+45+40 |
| 106 | 1 | 216 | 1 | 57.96 | 5.8 | 5.8 | 5.8 | 5.8 | 6.6 | 5.8 | 6.6 | 5.8 |
| 106 | 1 | 150 | 66 | 46.08 | 5.1 | 4.6 | 4.6 | 4.6 | 6.6 | 5.8 | 6.6 | 5.8 |
| 100 | 1 | 100 | 116 | 36 | 3.5 | 3.5 | 3.5 | 3.5 | 6.6 | 6.6 | 6.6 | 6.6 |
| 72 | 1 | 50 | 116 | 21.96 | 3.5 | 3.5 | 3.5 | 3.5 | 6.6 | 6.6 | 6.6 | 6.6 |
| 61 | 1 | 61 | 157 | 21.96 | 4.0 | 4.1 | 4.1 | 4.1 | 6.6 | 6.6 | 6.6 | 6.6 |
| 60 | 1 | 60 | 156 | 21.6 | 4.0 | 4.0 | 4.0 | 4.0 | 7.4 | 7.4 | 7.4 | 7.4 |
| 46 | 1 | 46 | 170 | 16.56 | 4.5 | 4.0 | 4.0 | 4.0 | 7.4 | 7.4 | 7.4 | 7.4 |
| 32 | 1 | 60 | 156 | 16.56 | 4.5 | 4.6 | 4.6 | 4.6 | 7.4 | 7.4 | 7.4 | 7.4 |
| 32 | 1 | 32 | 184 | 11.52 | 4.5 | 4.6 | 4.6 | 4.6 | 9.4 | 9.4 | 9.4 | 9.4 |
| 28 | 1 | 28 | 188 | 10.08 | 4.5 | 4.6 | 4.6 | 4.6 | 10.6 | 10.6 | 10.6 | 10.6 |
| 24 | 1 | 32 | 184 | 10.08 | 5.1 | 4.6 | 4.6 | 4.6 | 10.6 | 10.6 | 10.6 | 10.6 |
| 18 | 1 | 18 | 198 | 6.48 | 5.1 | 4.0 | 4.0 | 4.0 | 12.0 | 11.9 | 11.9 | 11.9 |
| 15 | 1 | 15 | 201 | 5.4 | 5.8 | 4.0 | 4.0 | 4.0 | 12.0 | 12.0 | 12.0 | 12.0 |
| 14 | 1 | 16 | 200 | 5.4 | 5.8 | 4.0 | 4.0 | 4.0 | 13.6 | 13.6 | 13.6 | 13.6 |
| 14 | 1 | 14 | 202 | 5.04 | 5.8 | 4.1 | 4.1 | 4.1 | 13.6 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 18 | 198 | 5.4 | 5.8 | 4.0 | 4.0 | 4.0 | 13.6 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 16 | 200 | 5.04 | 5.9 | 4.1 | 4.1 | 4.1 | 13.7 | 13.6 | 13.6 | 13.6 |
| 12 | 1 | 12 | 204 | 4.32 | 5.8 | 4.6 | 4.6 | 4.6 | 13.7 | 13.6 | 13.6 | 13.6 |
| 9 | 1 | 9 | 207 | 3.24 | 5.8 | 4.6 | 4.6 | 4.6 | 13.6 | 13.6 | 13.6 | 13.6 |
| 8 | 1 | 8 | 208 | 2.88 | 5.8 | 4.6 | 4.6 | 4.6 | 15.6 | 15.6 | 15.6 | 15.6 |
| 6 | 1 | 12 | 204 | 3.24 | 5.8 | 4.6 | 4.6 | 4.6 | 13.6 | 13.6 | 13.6 | 13.6 |
| 6 | 1 | 6 | 210 | 2.16 | 5.8 | 5.2 | 5.2 | 5.2 | 15.7 | 15.6 | 15.6 | 15.6 |
| 4 | 1 | 8 | 208 | 2.16 | 6.6 | 5.2 | 5.2 | 5.2 | 15.7 | 15.6 | 15.6 | 15.6 |
| 4 | 1 | 4 | 212 | 1.44 | 6.6 | 5.8 | 5.8 | 5.8 | 15.6 | 15.6 | 15.6 | 15.6 |
| 3 | 1 | 3 | 213 | 1.08 | 6.5 | 5.8 | 5.8 | 5.8 | 15.6 | 15.6 | 15.6 | 15.6 |
| 2 | 1 | 4 | 212 | 1.08 | 6.7 | 5.8 | 5.8 | 5.8 | 15.7 | 15.6 | 15.6 | 15.6 |
| 2 | 1 | 2 | 214 | 0.72 | 6.7 | 5.2 | 5.2 | 5.2 | 15.8 | 15.7 | 15.7 | 15.7 |
| 1 | 1 | 2 | 214 | 0.54 | 6.7 | 5.2 | 5.2 | 5.2 | 15.7 | 15.6 | 15.6 | 15.6 |
| 1 | 1 | 1 | 215 | 0.36 | 6.8 | 5.9 | 5.9 | 5.9 | 15.9 | 15.7 | 15.7 | 15.7 |