3GPP TSG-RAN WG4 Meeting #114	R4-2501494
Athens, GR, 17th – 21st February, 2025

Title:	R19 on power class for 3T

Source:	OPPO

Agenda item:	7.22.1

Document for:	Approval

- Introduction

This paper discusses the power class related issues for 3Tx per WF [1].

- Discussion

  - PC1.5

It was agreed that PC3 and PC2 can be supported for 3Tx UE. And for PC1.5 it was FFS. In our view, PC1.5 can be supported in several configurations like single antenna port, and UL MIMO with 3Layer. Therefore, for one band with 3Tx it can support PC1.5.

It was questioned in last meeting whether UE has to achieve 29dBm in all kinds of configurations if it reports PC1.5 capability. Our view is that UE doesn’t need to achieve the reported power class in all the MIMO layer capabilities, otherwise, in Rel-15 there will be no HPUE since UE cannot achieve the reported power class due to no ULFPTx in Rel-15.

Observation 1:   Rel-15 UE cannot achieve the reported power class in one layer two port configurations due to no ULFPTx modes in Rel-15 which demonstrates that the reported power class is the max power capability and doesn’t mean UE has to achieve that in all MIMO configurations.

Observation 2:   3Tx UE can support PC1.5 via 3Layer or single antenna port and without ULFTPx modes in Rel-19 based on observation 1.

Proposal 1:         Introduce PC1.5 for 3Tx UE.

  - UE Type

This has already been discussed in RAN plenary when this WI was introduced. And no UE type restriction for 3Tx in this WI.

Proposal 2:         No UE type restriction for 3Tx UE in this WI.

- Reference

[1] R4-2417116 WF on UE RF requirements for MIMO enhancement, Samsung, #112bis 2024-10