# 3GPP TSG-RAN WG4 Meeting # 114                                    R4-2501503

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

During the last meeting, RAN4 discussed how to construct the SCM channel model and some simulation cases. Regarding the construction of the SCM channel model, there are essentially three approaches, CDL channel model based on the TR 38.827, extending current TDL channel model and TX-RX beam steering approach, and the simulation scenarios predominantly revolve whether MU-MIMO test case needs to be considered and the selection of current TDL channel model.

Cases for SU-MIMO

Random PMI as a baseline, FFS Fixed. Interested companies invited to choose based on most common PMI in PMI histogram.

In the last meeting, RAN4 agreed to use random PMI as a baseline for performance analysis, which is a common method used by RAN4 in baseband testing. Regarding fixed PMI, Fixed PMI, which needs to be statistically analyzed over multi simulation periods, can better reflect the statistical effects on the channel. It may potentially bring about certain gains under certain scenarios. A fixed PMI might always represent the optimal precoding choice for the current channel state. If channel conditions vary significantly and frequently, a fixed PMI may fail to fully exploit the spatial diversity and multiplexing gains by multiple gains offered by multiple antennas, thereby constraining the system’s capacity. However, the statistical methodology for the fixed PMI is indeed not specified, which depends on the implementation. For defining requirements, we believe that there is no need to consider the fixed PMI.

Proposal 1. For defining requirements, we believe that there is no need to consider the fixed PMI.

MU-MIMO scenario

In last meeting, RAN4 agreed to evaluate performance for SU-MIMO and MU-MIMO scenarios. For SU-MIMO scenario, RAN4 assumed a BS and a UE, and the channel propagation was based on TR 38.827 to generate the CDL channel parameter, including the power of every cluster, ray and others. Also the beamforming gain was included in this procedure. However, for the MU-MIMO scenario, two UEs should be modeled in link level simulation at least. In legacy TDL channel model, RAN4 always assumes that the target UE and co-schedule UEs experience the same fading environment. We can assume that the channel propagation conditions between multi users are independent for TDL channel model. Therefore, we can consider that the TDL channel can support the simulation verification of MU-MIMO scenario.

However, for the CDL channel model, which is the spatial channel model, the spatial characteristics should be fully utilized to support multi-UE channels. In last meeting, some companies mentioned that the current system simulation for multi users interference scenario is typically based on drop based system simulators in traditional channel models, which meas that the channel properties follow a statistically independent distribution for different moment channel state, and lack of spatial consistency. Spatial consistency in a channel refers to the smooth transition of channel characteristic during the movement or turning of the transmitter and receiver, avoiding any sudden changes or discontinues. There should be a high degree of correlation in channel properties between links that are in close proximity. On the other side, considering the beam assumption, beam management relies on the continuity and progressive evolution characteristics of channel state information and maintaining the spatial consistency of the link when UE is in motion is crucial.

Regarding the modeling of MU-MIMO channel propagation conditions under spatial channel model, firstly, the UE distribution should be considered. The position distribution of target UE and co-schedule UE determines whether the spatial channel exhibits correlation. If there are closely, we can assume that the spatial channel conditions show high spatial correlation, otherwise low correlation could be considered. And secondly, in link level simulation, the channel state at each transmission time interval is continuous. When the UE is in a state of motion, the spatial channel model parameters are updated based on the changes in the relative positions and movement between UEs.

Proposal 2. RAN4 needs to discuss how to model the spatial channel propagation conditions under MU-MIMO scenario. And the channel parameters could be captured into TR38.753.

In the previous meetings, RAN4 discussed methodologies for antenna array virtualization applied in spatial channel models, which are also the methods employed in practice for analogy beamforming with large scale antenna arrays. At the BS side, the gain generated by analogy beamforming can be utilized the signal power at the transmission end, which can enhance the cell coverage. In link level simulation, we always assume that the transmission signal power is one, and then the noise can be derived by signal power. By sweeping through various SNR value, the system’s throughput can be obtained. After RAN4 introduced analog beamforming technology, there was an increase in signal power in the link simulation. This resulted in the curve representing the relationship between SNR and throughput, calculated using traditional methods, becoming less accurate.

In the last meeting, RAN4 agreed to use normalization methodology to calculate the sum of signal power across all Rx RF chains. From our understanding, if the signal power is significantly enhanced due to beamforming, normalization can ensure that comparisons are made based on the improvement of SNR across different system configurations rather than absolute signal levels in link level simulations. Even after normalization, the directional of beamforming does not change. This means that the signal in a specific direction will be more concentrated and strong compared to when beamforming is not used. More importantly, the process of CDL channel modeling is relatively complex. Even in TR 38.827, the influence of stochastic elements has been somewhat diminished. In general, the research on spatial channel model is of paramount importance to RAN4 testing. Spatial channel model enable us to understand the propagation characteristics of signals in space, including phenomena such as multipath effects and fading, which are crucial for designing efficient wireless communication systems. In last meeting, RAN4 agreed to use normalization factor to normalize signal power. In order to derive actual SNR point in link simulation, normalization could

Observation 1. The CDL channel model brings more gains in the modeling process.

Proposal 2. RAN4 needs further check the feasibility of normalization among results.

As discussed in previous meeting, some companies mentioned that beam steering could also achieve spatial characteristic based on TDL channel model. This approach also captured in Annex of TS38.101-4. Compared to CDL approach, there is no doubt that the TDL approach is relatively simple and easy to implement. In contrast, the implementation challenges associated with analog beamforming in the CDL channel model cannot be overlooked. Given that the enhanced TDL channel model has proven to be a viable approach. At the current stage, RAN4 should not overlook any viable options, at the very least, it should theoretically demonstrate the limitations of these approaches.

Observation 2. Enhance TDL channel model based on the approach of beam steering, which is captured in TS38.101-4.

Proposal 3. RAN4 should not overlook any viable options, at the very least, it should theoretically demonstrate the limitations of these approaches.

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements, The conclusions are:

Observation 1. The CDL channel model brings more gains in the modeling process.

Observation 2. Enhance TDL channel model based on the approach of beam steering, which is captured in TS38.101-4.

Proposal 1. For defining requirements, we believe that there is no need to consider the fixed PMI.

Proposal 2. RAN4 needs to discuss how to model the spatial channel propagation conditions under MU-MIMO scenario. And the channel parameters could be captured into TR38.753.

Proposal 3. RAN4 should not overlook any viable options, at the very least, it should theoretically demonstrate the limitations of these approaches.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM. Nokia.