# 3GPP TSG-RAN WG4 Meeting # 114                                   R4-2501506

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.18.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial discussions on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation and intra-cell inter-user interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-18, RAN4 introduced 8Rx demodulation requirements for PDSCH and CSI performance. With 8Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios.

  - Sub-topic 2-1 General

Definition of MMSE-IRC receiver capability for 8Rx UE

- Agreement: - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

In Rel-18, RAN4 defined simplified receiver for 8Rx CPE/FWA/vehicle/industrial devices under SU-MIMO scenario without inter or intra interference. Also, to support this receiver, RAN4 defined a UE capability to describe this feature, 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors[3]. And now in Rel-19, RAN4 extended the interference cancellation scenario to MU-MIMO, which means RAN4 needs to consider the simplified receiver for MU-MIMO scenario. As illustrated in above, RAN4 needs to consider how to capture the instruction in the specification to support this feature under MU-MIMO scenario.

As describe in above candidate options, RAN4 considered different methods to capture the detailed illustrations. Firstly, we think that introduce a new capabilities is not necessary in this feature. In Rel-18, RAN4 introduced a capability to support simplified receiver. Actually, this capability doesn’t need to report to BS side, which only serves to make a difference between baseline and simplified receivers. If RAN4 continues to introduce a new capability, we think too many useless signalling is introduced in specification. Therefore, we prefer to consider redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1.

Proposal 1 . To consider redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1.

8Rx UE MMSE-IRC receiver coverage

Agreement: - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

Regarding baseline and simplified receivers, we still think that defining two set requirements is necessary, as they represent different implementation methods at the baseband side. At the same time, we provided simulation results for inter interference and intra interference cancellation in reference [4]. From our simulation results, we observed significant performance degradation between baseline and simplified receiver, particularity in Rank 4 about more than 3dB . And regarding the CQI interference scenario, RAN4 has not yet evaluated the results, making it difficult to decide which requirements can be applicable to these two types of receivers.

Observation 1. RAN4 has not yet evaluated all test cases, making it difficult to decide which requirements can be applicable to these two types of receivers.

Proposal 2. Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

  - Sub-topic 1-2 PDSCH requirements with inter cell interference

Number of interference cells

Agreement: - Background: - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555] - Proposals: - Option 1: Both 1 interference cell and 2 interference cells

In the previous meeting, RAN4 achieved an agreement of interference scenarios, to cover both HomNet and HetNet scenarios in this feature. The big difference between HomNet and HetNet is the number of INR values, two and one, which come from the legacy assumptions. From the practical deployment perspective, HomNet and HetNet are different conditions in actual network, which is same as legacy role.

Proposal 3. Cover both 1 interference cell and 2 interference cells.

  - Sub-topic 1-3 PDSCH requirements with intra cell inter user interference

MCS

Agreement: - Define requirements for: - Rank 2+2 - FFS: MCS17/19 - TBD: Rank 4+4: MCS13

  - From our simulations, both MCS 17 and MCS 19 are feasibility in evaluations. The SNR point for MCS 19 could be a little higher than MCS 17, it still within a measurement range. Thus, we think both MCS 17 and MCS 19 are reasonable for MU-MIMO test cases.

Proposal 4. To consider MCS 17 for Rank 2+2 test cases.

    - Sub-topic 1-5 Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

Agreement: - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

The motivation behind RAN4’s discussion of the test applicability rule that is to reduce the burden of UE if UE supports a high capability in testing. In Rel-17, RAN4 introduced inter interference scenario and intra-cell inter-iser interference scenario as mandatory feature for 2/4Rx UE. As illustrated in WID, 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario are also mandatory feature for UE. Hence, if UEs supporting both 8Rx and 2/4Rx, 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped. Otherwise, 2/4Rx tests could not be skipped.

Proposal 5. To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Test applicability for FDD/TDD

- Option 1: For 8Rx inter-cell interference scenario, for demodulation performance tests, introduce similar applicability rules defined in section 5.1.1.10 for UE supporting both duplex mode TDD and FDD as legacy 2Rx/4Rx, i.e., if UE is supporting both TDD and FDD with 8Rx, UE will pass test case under HomNet scenario with FDD mode, and pass test case under HetNet scenario with TDD mode. - Option 2: Do not reduce UE effort to only comply to either 1 or 2 interference cells.

In legacy test applicability, if UE is supporting both TDD and FDD with 2Rx or 4Rx, some test cases could be skipped during testing. In our understanding, similar applicable rules also could be defined for 8Rx demodulation performance tests.

Proposal 6. To consider similar applicable rules also could be defined for 8Rx demodulation performance tests.

Release independent

- Option 1: Discuss release independence requirements at a later stage in the WI. - Option 1a: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113. - Option 2: The new requirements for 8Rx should be release independent from Rel-18. - Option 2a: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

Considering 8Rx requirements was introduced in Rel-18, and inter interference cancellation is a mandatory feature. We prefer to consider the new requirements shall be release independent form Rel-18.

Proposal 7. The new requirements for 8Rx should be release independent from Rel-18.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for UE demodulation performance requirements for 8Rx with MMSE-IRC , The conclusions are:

Observation 1. RAN4 has not yet evaluated all test cases, making it difficult to decide which requirements can be applicable to these two types of receivers.

Proposal 1 . To consider redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1.

Proposal 2. Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

Proposal 3. Cover both 1 interference cell and 2 interference cells.

Proposal 4. To consider MCS 17 for Rank 2+2 test cases.

Proposal 5. To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 6. To consider similar applicable rules also could be defined for 8Rx demodulation performance tests.

Proposal 7. The new requirements for 8Rx should be release independent from Rel-18.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- R4-2419784, WF for [113][322] NR_demod_Ph5_Part2_UE. Nokia.

- 38.101-4 V18.5.0

- R4-2501507, Simulation results on UE demodulation performance requirements for 8Rx with MMSE-IRC. ZTE Corporation, Sanechips.