# 3GPP TSG-RAN WG4 Meeting # 114                                   R4-2501509

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on testability and interoperability issues for AI-CSI

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.20.3

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI compression and prediction.

# 2 Discussion

Option 3 next step - decoder(s) selection

Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

At the last meeting, RAN4 discussed how to choose the decoders and that agreed Track1 and Track 2 should select decoders based on company’s results from the RAN4 #113 meeting.

For Track 1, as illustrated in above, select 3 decoders based on low, medium, high means SGCS out of available decoders. According to the 113 meeting results summary, we select 3 decoders from CATT, Samsung and HW with low, medium, and high SGCS. Also, we still chose decoders from other companies to study the feasibility of Track 1. In our simulation, we followed the conclusions reached prior to RAN4, utilizing these decoders for the training of our encoder; meanwhile, the encoder structure still follows a CNN framework, which has been agreed in RAN4 previous meeting. In additional, our training data originates from system simulations and remains consistent with the previous simulation assumptions. Also, quantization is not considered in our training process. The final results are as shown in following Table 1.

Table 1. Simulation results for Track 1 of Option3

| Decoder |  | QC | Samsung | HW | MTK | Nokia |
| --- | --- | --- | --- | --- | --- | --- |
| CSI feedback | Training dataset | Test dataset | Test dataset | Test dataset | Test dataset | Test dataset |
|  |  | UMa | UMa | UMa | UMa | UMa |
| No quantization | UMa | 0.45 | 0.72 | 0.1 | 0.40 | 0.69 |

Based on our simulations results, there are significant differences in the encoders obtained through training with the same training dataset but different decoders, even when all companies use the same AI/ML architecture. Supposing that all companies’ datasets use the same preprocessing steps, perhaps we could draw a conclusion that variations in training data have a decisive impact on performance.

More importantly, when we training with the selected decoders, we discovered that the output data structures from some companies deviate from previous meeting conclusions. This implies that the dimensions of the training data did not align with the previous conclusions and highlights inconsistencies in the preprocssing of the training datasets.

Observation 1. The output data structures from some companies deviate from previous meeting conclusions, which also implies that the dimensions of the training datasets did not align with the previous meeting conclusions.

Proposal 1. Propose to completely align the decoder- related parameters, input/output formats, quantization, etc.

Regarding Track 2, that needs to crate a mixed dataset, companies train decoders based the mixed dataset. And then RAN4 needs to chose one or more decoders and companies develop encoders based on the selected decoders. The core of this approach lies in how to crate a mixed dataset. In previous email discussion, some companies proposed that mixed dataset is constructed from the 600k/n samples from each of the datasets, where n is the number of contributed companies. In general, we are agree this approach to crate a mixed training dataset. On the other side, the validation and testing datasets also need to follow the same approach, the only difference is the number of samples.

As described above, we used the mixed dataset that uploaded by other companies in 3GPP website. The AI/ML model also followed the CNN structure. We evaluated 2bit scalar quantization and without quantization in our training process. Our simulation results are shown in the following Table 2 and Table 3.

Table 2. Simulation results for Track 2 of Option3

| Model structure | Training dataset | Test dataset |
| --- | --- | --- |
|  | Mixed dataset |  |
| CNN-no quantization | 0.67 |  |

Table 3. Simulation results for Track 2 of Option3

| Model structure | Training dataset | Test dataset |
| --- | --- | --- |
|  | Mixed dataset |  |
| CNN- 2-bits Scalar Quantization | 0.61 |  |

Our simulation results show that the SCGS is approximately 0.67 without quantization and 0.61 with 2 bits scalar quantization. Compared to our previous result with our own training dataset, the performance has a little degradation. From our understanding, this phenomenon is reasonable, because we used a mixed dataset with samples from seven companies. The mixed dataset may have distinct data characteristics, with considerable variability existing among different companies. In order to adapt the differences in the training dataset, the trained model has a certain degree of generalization.

Observation 2. The results from the mixed dataset show degradation compared to our previous results for 2bits scalar quantization and without quantization.

Observation 3. The training data has an impact on performance.

Option 4a(Dataset based) for 2-sided model

- Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Option 4b (encoder based) for 2-sided model

- Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

At the previous meeting, RAN4 deliberated on potential candidate options for option 4, namely the means of achieving interoperability between DUT vendors and TE vendors. In TR 38.843, RAN4 defined partial knowledge in RAN4 specification for decoder to achieve interoperability. With regard to this partial knowledge of the decoder, it can bring more flexible approaches for TE vendors. To ensure interoperability, RAN4 needs to define more knowledge between DUT and TE side. After numerous discussions at meetings, RAN4 agreed two approaches to study the feasibility of Option 4, dataset based and encoder based. In generally, to achieve the interoperability, decoder is designed by TE vendor, DUT vendor needs to ensure that the encoder can be paired with decoder.

For the above steps, steps 1~3 need to reuse the results of Option3. From the perspective of simulation evaluation, this is entirely feasible. The next step is about how to chose the dataset, the dataset generated from the selected encoder an decoder pair or mixed dataset. For the first approach, the dataset generated from the selected reference model. The datasets correspond one to one with the reference models. RAN4 needs to make sure that the results provided by all companies based on the reference mode and datasets are consistent. Then, RAN4 can go to the next step. And for the track 2 with mixed dataset, without a doubt, the final result of SGCS is inferior to that of the track 1.

Regarding step 5, the selected dataset corresponding to the selected test decoders, encoder should at least have the agreed structure. In previous steps, we selected test decoders based on the dataset. Thus, TE vendor can use this to test the encoder of DUT vendor. Because different DUT vendors may have different types of encoder. RAN4 needs to consider different structures at least, also needs to evaluate the differences between encoder and decoder structures to ensure that this approach can work.

For the step 7 of feasibility criteria, to achieve interoperability, RAN4 needs to check that whether all test decoders and encoders can derive same performance for different structure. For the previous agreed steps, RAN4 assumes that the encoder and decoder have the same structure, the simulation evaluation also based on the same assumption. However, in practical implementation, it is very hard to ensure that all DUT vendors have the same structure of the encoder, TE vendors also may have different implementations for test decoders. The matching problem between encoders and decoders with different structures is particularly common, so it is necessary to study the matching of different structural structures. In other words, the feasibility of Option4a and Option 4b need to perform testing all the DUT encoders using all test decoders. The reason is that the test decoder is implemented by different TE vendors. Ensuring that all TE vendors based on a uniform structure poses a challenge. Moreover, RAN4 allows a certain degree of flexibility for the test decoder. From RAN4 conformance testing perspective, RAN4 expects that all testing results from different TE vendors should be consistent.

Observation 4. The selection criteria for Option 4 could be waiting for the results of Option 3.

Proposal 2. For option 4 with partial knowledge for test decoder, RAN4 needs to check that whether all test decoders and encoders can derive same performance for different structure.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI-CSI , The conclusions are:

Observation 1. The output data structures from some companies deviate from previous meeting conclusions, which also implies that the dimensions of the training datasets did not align with the previous meeting conclusions.

Observation 2. The results from the mixed dataset show degradation compared to our previous results for 2bits scalar quantization and without quantization.

Observation 3. The training data has an impact on performance.

Observation 4. The selection criteria for Option 4 could be waiting for the results of Option 3.

Proposal 1. Propose to completely align the decoder- related parameters, input/output formats, quantization, etc.

Proposal 2. For option 4 with partial knowledge for test decoder, RAN4 needs to check that whether all test decoders and encoders can derive same performance for different structure.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2417212, WF on requirements for AI/ML air interface.

- 3GPP TS38.101-4.

# 5 Appendix

- System simulation assumption

Table 5-1. System simulation assumptions for training data

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | ideal channel estimation |

- AI model structure

- Parameters

| Parameter/Configurations | Options/Values | Company comments |
| --- | --- | --- |
| Channel/data-set related |  |  |
| UE antenna element assumptions | - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other | [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1 |
|  | Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions |  |
| Sub-band channel averaging | - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD | CATT: Prefer not to do the averaging before SVD. |
|  | Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected |  |
| Any other pre-processing | - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands | [Intel] prefer no other pre-processing |
|  | Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1. |  |
| Data collection assumptions | - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval. |  |
|  | Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs. |  |
| Training dataset size | - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K |  |
|  | Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used. |  |
| Dataset quantization | - Float32 |  |
| Other data-set characteristics/statistics | - Companies to propose |  |
|  | - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII. |  |
| Model-related |  |  |
| Quantization of encoder output | - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training | CATT: Consider to align the activation function and quantization method. |
|  | - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization |  |
| Normalization before quantization in the encoder | - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder. |  |
|  | Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional. |  |
| Use and configuration of drop-out | - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used. |  |
|  | Configuration for alignment: - Not to use drop-out layers |  |
| Training loss function, use of regularization | - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization |  |
|  | Configuration for alignment: - Use (1-SGCS) as loss function as reference |  |
| Optimizer | - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW |  |
|  | Configuration for alignment: - Use Adam |  |
| Learning rate | - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch |  |
|  | Configuration for alignment: - Fixed learning rate of 10^-4 |  |
| Batch size | - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400 |  |
|  | Configuration for alignment: - 256 |  |
| (Max) Number of epochs | - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500 |  |
|  | Configuration for alignment: - 150 epochs |  |
| Stopping criteria | - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs |  |
|  | Configuration for alignment: - No improvement over 25 epochs. |  |
| Weight initialization | - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling |  |
|  | Configuration for alignment: - Default |  |
| Kernel Parameters: - Dilation - Padding Type The value of groups (VG) | - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default |  |
|  | Configuration for alignment: - Default |  |