3GPP TSG-RAN WG4#114	R4-2501579

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.1.1.1.1

Source: 	Qualcomm France

Title: 	MSD beyond PC3

Document for:	Approval

# Introduction

Considerations and proposals for HPUE MSD are provided in this contribution.

- Background

WF was agreed in RAN4#113 [1]:

- Work on the feasible unified Equations/LUT/Simplified guidelines for MSD for HPUE band combinations as normative requirements

  - Targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed.

  - The existing MSD requirements for band combinations for HUPE won’t be changed.

  - Discuss whether the request procedure for HPUE band combinations should be kept

- For Rel-19, follow tradition procedure, including the request of HPUE band combinations, until the work on the feasible unified Equations/LUT/Simplified guidelines is finished.

1UL HPUE:

- For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as:

PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

  - Companies are invited to evaluate two options,

- option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]:

  - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB,

  - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB,

  - option 2: RAN4 specifies a single set PC2 requirement [3]:

    - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB.

- To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming:

  - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded,

  - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation. .

- Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD:

  - option 1: FFS LUT rule and FFS on test point configuration

  - option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point

  - option 3: no HPUE MSD requirement for those cases.

For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps)

2UL HPUE:

- Companies are invited to

  - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD,

  - Study means to minimize the number of LUTs / set of equations that cover all cases.

  - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7].

  - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for.

FFS to account for other IMD contributors than just the power amplifier.

- Discussion

General aspects:

We begin by discussing general aspects related to this rather big evolutionary step. As discussed earlier, the MSD requirements specified before concluding all the agreements related to HPUE MSD rules should remain intact in the specifications. This is to respect all the work done during the past releases, as well as not to change any of the existing MSD requirements.

Proposal 1: All PC2 MSD and PC1.5 MSD tables specified before concluding all new HPUE MSD rules will remain in untouched TS 38.101-1 and TS38.101-3 after new HPUE MSD rules apply for new HPUE combinations.

WF is worded in a way that either Equations or Look-Up tables could be included in TS for HPUE MSD. Equations which by their nature are very complex are used to derive the values for the Look-Up Tables. Table presentation is way simpler in TS. Our original proposal to use Equations was proposed for the case when the 3GPP procedure of explicitly specifying MSD for PC2 and PC1.5 would have continued. Now that the group has agreed not to specify HPUE MSD explicitly for each band but using equations/LUT’s instead, LUT is much better approach for that purpose.

Proposal 2: Agree to use Look-Up Tables in TS38.101-1 and TS 38.101-3 to define HPUE MSD for the new HPUE combinations (For which HPUE MSD is not explicitly specified).

1UL cases:

We assumed 6dB PRX/DRX imbalance between aggressor power, and 0.5dB additional noise for 2Tx cases. For single Tx we assumed interferer power to increase 3dB when power increases 3dB. Specifically, for the PRX7DRX assumption we did look at both practical RFFE implementations as well as the resulting 2Tx MSD. These two should be reasonably well balanced, meaning the PC2 2Tx MSD should be reasonable compared to PC2 1Tx MSD. We note that all the parameters used are coarse simplifications, and in real devices they may be different. For instance, cross-band noise may increase more or less than 3dB depending on the case and UL harmonic can increase a bit more than 3dB, or a bit less depending on the case.

Based on that, we propose the following LUT for 1UL cases for consideration. Please note that it would be possible to further reduce the number of lines in the table by applying some kind or merging; we didn’t propose any before seeing everyone’s proposals.

Proposal 3: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for discussions.

Figure 1 LUT for 1UL aggressor cases

Cases where PC3 MSD does not exist are potentially very tricky, as there is no non-zero reference value. We have had some frequency relation-based proposals earlier on how and for which combinations to capture the HPUE MSD, however they would be rather complex to be used in specifications using LUT format. Instead of that, we propose to capture such HPUE MSD cases still explicitly in existing HPUE MSD tables.

Proposal 4: PC2/PC1.5 MSD is specified for each applicable combination in existing PC2/PC1.5 MSD tables for the HPUE combinations for which PC3 MSD is not specified.

2UL cases:

There are multiple different TX configurations and respective power combinations for 2UL cases. The number of Tx per band can be 1 or 2, TX power per band can be 20/23/26/29dBm, and UE power class can be PC3/PC2/PC1.5. All this combined with IMD order, IMD indexes per band, and power increase per band could make things extremely complicated in worst case.

To tackle all of this, we did derive the proposed LUT based on simple approach, where the interferer power increase is calculated by multiplying the UE power class increase by the IMD order using scaling factor 1. For instance, PC2 IMD4 interferer increase over PC3 is 3dB*4*1=12dB. This approach is universal and can be used for 1+1, 2+1, 1+2, 2+2 Tx per band i.e. all cases discussed by now. It is important to agree that 2UL LUT applies for any combination of number of TX’es per band, to avoid extra hassle for 3Tx and 4Tx. We assumed same 6dB PRX/DRX interference imbalance as for 1UL case when deriving the reference PC3 interferers.

Like in 1UL case, it would be possible to further reduce the number of lines in the table by applying some kind or merging; we didn’t propose any before seeing everyone’s proposals.

An alternative for this could be using one step more rigorous approach in which the interferer power increase is calculated by multiplying the power increase per each band by the IMD indexes per each band. This method would give more accurate interferer increase for combinations where the power per band in is inequal, e.g. UE PC1.5 with PC3 FDD+PC1.5 TDD where the power config in PC1.5 MSD is 20+27.8dBm, but the big drawback is that then referencing the right LUT table would be extremely difficult and the evolutionary step of defining HPUE MSD easily would be lost.

The IMD interferer may behave differently depending on the scenario and source of the interference. The interference may not scale up proportionately to the multiplication of power and IMD order i-e scaling factor could be <1. Consequently, with scaling factor 1, HPUE IMD interferers especially for cases with high IMD order (IMD4, IMD5, IMD>5) become large. To address this, we propose to discuss if this is something which should be resolved somehow. Among possibilities could be to use scaling factor <1 for IMD4 and above, or to limit the HPUE ΔMSD for IMD4 and above to a certain value.

Proposal 5: LUT for 2UL cases (Intermodulations) applies for 1Tx+1Tx, 1Tx+2Tx, 2Tx+1Tx, and 2Tx+2Tx cases.

Proposal 6: Use the following LUT’s for 2UL cases (Intermodulations) for initial discussions. Final LUT format(s) should be further simplified.

Proposal 7: RAN4 should discuss if some kind of limitations to HPUE MSD’s for ≥[IMD4] should be considered.

- Conclusion

Considerations on PC2 and PC1.5 MSD were provided with the following proposals.

Proposal 1: All PC2 MSD and PC1.5 MSD tables specified before concluding all new HPUE MSD rules will remain in untouched TS 38.101-1 and TS38.101-3 after new HPUE MSD rules apply for new HPUE combinations.

Proposal 2: Agree to use Look-Up Tables in TS38.101-1 and TS 38.101-3 to define HPUE MSD for the new HPUE combinations (For which HPUE MSD is not explicitly specified).

Proposal 3: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for discussions.

Proposal 4: PC2/PC1.5 MSD is specified for each applicable combination in existing PC2/PC1.5 MSD tables for the HPUE combinations for which PC3 MSD is not specified.

Proposal 5: LUT for 2UL cases (Intermodulations) applies for 1Tx+1Tx, 1Tx+2Tx, 2Tx+1Tx, and 2Tx+2Tx cases.

Proposal 6: Use the following LUT’s for 2UL cases (Intermodulations) for initial discussions. Final LUT format(s) should be further simplified.

Proposal 7: RAN4 should discuss if some kind of limitations to HPUE MSD’s for ≥[IMD4] should be considered.

- Reference

[1] R4-2420380, “WF on MSD rule”, Skyworks, Murata, Nokia, Qualcomm