3GPP TSG-RAN WG4#114	R4-2501582

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.1.1.3.3

Source: 	Qualcomm France

Title: 	SRS antenna switching and ΔTRxSRS

Document for:	Discussion

# Introduction

SRS antenna switching and ΔTRxSRS considered in this contribution.

- Discussion

WF was agreed in RAN4#113 [1]

## Sub-topic 1-1:	General considerations for SRS antenna switching and ΔTRxSRS

Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details.

-	Option 2: Consider all available patterns for 3T6R AS-SRS.

We are in favor of specifying all available patterns for 3T6R AS-SRS, so we support Option 2.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

## Sub-topic 1-2:	ΔTRxSRS values

Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6

Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach.

Table: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

These values serve as a reasonable compromise, so we are ok with these.

Proposal 2: Use the values in the table in WF for 3T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability. As usual, 3dB is added for each entry for the case of SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

- Conclusion

SRS antenna switching and ΔTRxSRS considered in this contribution.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

Proposal 2: Use the values in the table in WF for 3T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability. As usual, 3dB is added for each entry for the case of SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

- Reference

[1] R4-2420376, “WF on 6Rx requirements”, AT&T