3GPP TSG-RAN WG4 Meeting # 114		R4-2501605

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.1.1.1.3

Source:	vivo

Title:	Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx

Document for:	Approval

- Introduction

In RAN4 discussion for UL inter-band UL CA or DC with 2Tx or 3Tx, In RAN4#114, the topic summary [1] and agreed WF [2] has included several issues on inter-band HPUE case for 2Tx/3Tx, which is as following in the WF:

  - Configurations

Agreement:

- To create and maintain a configuration table listing all configurations can be considered in Rel-19 in next meeting, for information purpose

- FFS specific MSD calculation procedure details for the band combination including PC2 FDD as a constituent band (details in section 4.1)

  - No need to enter such combination into the WID

  - Re-organize 2Tx/3Tx Notes in clause 5 and clause 6

Agreement:

- Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings

  - Release independence of 3Tx

Agreement:

- For FWA, Rel-17

- For handheld UE, FFS in next meeting

This contribution further discussed these issues. It is noted that the duty cycle solution and MSD issues have been moved to another dedicated agenda, thus not covered here anymore.

- Discussion

Configurations

As in [1], there were previous agreements that general requirements can be specified including considerations from previous WF in RAN4#111:

In the recommended WF in the summary [1], it says:

  - The high-level principle is to strive to define general requirements in a configuration-agnostic way. On top of that, RAN4 can identify some exceptions owning to implementation concern and/or new MSD framework, such as [PC2/PC1.5 inter-band UL combinations consisting of PC2 FDD band]

We think that this general principle is actually quite constructive. Although detailed requirements are necessary for every combination, and it is still needed for combinations to have different respective requirements. This principle of keeping the requirements as general as possible is important to handle the various cases that listed, apart from the BC specific requirements.

Proposal 1: Keep the requirements other than Band combination specific as general as possible, to leave more flexibility on the spectral work.

Re-organize 2Tx/3Tx Notes in clause 5 and clause 6

In [1], two proposals from [3][4] were summarized regarding the notes in clause 5 and clause 6.

Among them, [3] is a more comprehensive discussion including both clause 5 and clause 6, while [4] has made very concrete and complete solution for clause 6.

For clause 5, [3] has already made quite complete analysis on the status and problems of current spec. For 1UL and ULCA, the following:

Quite detailed proposals are made and would not be re-iterated. It should be noted that there were discussions on the MSD issue, and how to do the MSD simplification would have an impact on how much simplification can be made.

In our understanding, the current MSD simplification, though details have not been fully settled yet, is already closing to the original target that only PC3 requirements were almost enough, as the WF for MSD [5] shows that a lot of parameters such as 2Tx already being considered in the LUT. Thus we would have a preference on “Remove all footnotes related to ULCA power classes” , and also similar principles as much as possible for 1UL, which is a more simple and clean solution.

Proposal 2a: Propose to remove the footnotes related to power classes as much as possible in clause 5 as a general principle.

For clause 6, different ways of simplification were proposed in [3][4]. In [3] two different alternatives were raised, and in [4] one new table is raised.

In [3]:

Alt 1:

Alt 2: Do not include the indication of which band can support UL MIMO nor power class, replace all these HPUE notes by general description above the table.

In [4]:

Currently we think the Alt1 in [3] may be a good compromise as starting point. However, more discussion is apparently needed.

Proposal 2b: Further simplify the Notes would be preferred for clause 6. (As Alt1 in R4-2418305).

Release independent for 3Tx

This issue was also raised in the previous meeting.

For FWA, since it is already decided in Rel-18 that would be started from Rel-17, it is natural to keep newly defined configurations from Rel-17, and currently we already have such contents and dedicated restriction of FWA in 38.306.

However, for handheld devices, we think it is different condition, and not appropriate to apply the same time frame of FWA. It is proposed to either not have release independence, or release independent from Rel-18, for 3Tx handheld devices.

Proposal 3: For 3Tx handheld devices, the requirements are either not have release independence, or release independent from Rel-18.

- Conclusion

In this paper, the following views provided:

Configurations

Proposal 1: Keep the requirements other than Band combination specific as general as possible, to leave more flexibility on the spectral work.

Re-organize 2Tx/3Tx Notes in clause 5 and clause 6

Proposal 2a: Propose to remove the footnotes related to power classes as much as possible in clause 5 as a general principle.

Proposal 2b: Further simplify the Notes would be preferred for clause 6. (As Alt1 in R4-2418305).

Release independent for 3Tx

Proposal 3: For 3Tx handheld devices, the requirements are either not have release independence, or release independent from Rel-18.

- References

- R4-2418139, Topic summary for [113][117] NR_ENDC_RF_Ph4_part2, Moderator (Samsung), RAN4#113.

- R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113

- R4-2419714, Improvements to valid uplink HPUE configurations , Skyworks, RAN4#113

- R4-2418305, Reorganize the 3Tx Notes, Samsung, RAN4#113

- R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113