TSG-RAN Working Group 4 (Radio) meeting #114	R4-2501648

Athens, EU, 17th to 21th February 2025

Source:	Ericsson

Title:	TP to TR 38.908: Conformance test aspects for EEIRP

Agenda item:	7.13.4.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#113 in Orlando) extensive discussions resulted in a working document distributed by the rapporteur with information to be used as baseline for coming text proposals to the technical report TR 38.908.

In this contribution we provide technical background information relevant for the new requirement defined to protect FSS UL within band n104 and simulation results to conclude on test beam constellation.

The technical background information is formatted as a text proposal for appropriate clauses in TR 38.908. The text proposal is attached at the end of this contribution, and it is presented for approval.

# Discussion

In the following we are going to present the latest efforts in the direction of deciding the number of test beams to be considered for evaluation of Expected EIRP.

During the last period we have conducted two different studies based on two different methodologies to derive required number of test beams to secure acceptable measurement uncertainty for EEIRP. The methodologies can be summarised as:

- Evaluation of EEIRP spread per elevation bins: A methodology based on analysing Confidence Interval (CI) for EEIRP per elevation bin for different steering ranges and antenna configurations.

- Evaluation of EEIRP spread with respect to a reference case: A methodology based on defining a reference case, described by a very large number of test beams. The analysis of smaller test beam sets is presented with regard to the  deviation from the reference case.

Both methodologies are based on simulations and the results are showing how EEIRP depends on different array antenna configurations and test beam sets.

In addition, a text proposal for TR 38.908 has been created to capture technical background information related to the introduction of protection requirement for FSS UL within band n104 into TS 38.104 and TS 38.141-2.

The changes added by the text proposal can be summarised as:

- In clause 4, background information has been updated with respect to improved readability. Some additional information was added to link the requirement concept with normal BS operation as indicated in WRC-outcome.

- In subclause 6.3, technical information related to the description of test beam definitions is captured. This section cannot be complete before RAN4  comes to a conclusion on test beam constellation to be considering for testing this requirement.

- In subclause 6.4, technical information related to how the EIRP pattern sampling resolution is determined. Since the current text suggested to have both physical size and beamwidth as declarations producing different results, the text was cleaned up to just provide an approach based on physical size of the transmitter antenna array. If needed, appropriate expression for relating physical and beamwidth can be added later.

- In subclause 6.5.1, technical information related to how to measure an EEIRP profile is added.

- In subclause 6.6, technical information related to the measurement uncertainty evaluation is added. Definition of error sources unique for EEIRP is updated.

Proposal 1: Include information from text proposal attached at the end of this contribution to TR 38.908.

  - Evaluation of EEIRP spread within elevation bins

In this section we analyse the EEIRP variation within different vertical segments (bins). The vertical segments considered here are aligned with the requirement mask derived by WRC-23.

The EEIRP variation is analysed by studying the Confidence Interval (CI) for each elevation bin including different number of test beams and BS array antenna geometries for three different steering ranges.

For the evaluation of the CIs one can use Monte Carlo based simulation methodology. However, this is a demanding process in terms of computation resources. Instead, we could use the sampling process described at previous RAN4 meetings where the estimation of the CI is based on Student’s t-distribution and a finite population correction. This is considered to be a much faster approach for calculating CIs.

Factors that will affect the EEIRP CI levels are listed below:

- Number of test beams, K

- Number of supported beams, M (reference)

- Size of antenna aperture

- Size of sub-array

- The size of the steering range

Apparently, for K=M there is sampling of the real, near-infinite, population without replacement, since all the finite beams are used. Hence, we can get a realistic CI (i.e. 97.5%) for a certain set of antenna aperture, sub-array size and steering range. By evaluating different antennas and steering ranges, through simulations, we obtain an estimation of the EEIRP variation within a specific vertical range (aka bin). This may seem trivial since it is unfeasible to measure all M beams for a given BS. However, it will give a refence of a realistic accuracy to be expected and can be possibly used for the proper selection of the set of test beams of size K.

The azimuth steering range can be safely assumed to be fixed between -60 to 60 degrees. As a result, there can be defined 5 instances of horizontal steering ranges as visualised in Figure 2.1-1.

# Figure 2.1-1: Horizontal steering ranges

For a selected number of K-beams (and the corresponding sampling) the EEIRP CIs are evaluated for a different number of antennas apertures, different number of sub-arrays and for different steering ranges. To capture a good variation of antennas with different apertures and beamwidths and a variety of sub-array formations we can consider the 23 antennas listed in Table 2.1-1.

Table 2.1-1: Array antenna configurations

| Id | #rows | #cols | Vertical element separation (l) | Sub-array size |
| --- | --- | --- | --- | --- |
| 1 | 16 | 8 | 0.5 | 1 |
| 2 | 32 | 16 | 0.5 | 1 |
| 3 | 24 | 16 | 0.7 | 3 |
| 4 | 12 | 16 | 0.5 | 1 |
| 5 | 12 | 16 | 0.5 | 2 |
| 6 | 12 | 16 | 0.6 | 1 |
| 7 | 12 | 16 | 0.6 | 2 |
| 8 | 16 | 8 | 0.6 | 1 |
| 9 | 16 | 8 | 0.7 | 1 |
| 10 | 16 | 8 | 0.7 | 2 |
| 11 | 32 | 16 | 0.5 | 2 |
| 12 | 32 | 16 | 0.5 | 4 |
| 13 | 32 | 16 | 0.6 | 1 |
| 14 | 32 | 16 | 0.6 | 2 |
| 15 | 32 | 16 | 0.6 | 4 |
| 16 | 32 | 16 | 0.7 | 1 |
| 17 | 32 | 16 | 0.7 | 4 |
| 18 | 24 | 16 | 0.6 | 1 |
| 19 | 24 | 16 | 0.6 | 3 |
| 20 | 24 | 16 | 0.6 | 6 |
| 21 | 24 | 16 | 0.7 | 1 |
| 22 | 24 | 16 | 0.7 | 2 |
| 23 | 24 | 16 | 0.7 | 4 |

In this way antenna vertical apertures from 6 to 22.4 wavelengths are covered (6, 7.2, 8, 9.6, 11.2, 14.4, 16, 16.8, 19.2, 22.4), vertical sub-array sizes from 0.5 to 3.6 wavelengths (0.5, 0.6, 0.7, 1, 1.2, 1.4, 1.8, 2, 2.1, 2.4, 2.8, 3.6) with various number of elements (1, 2, 3, 4, 6).

The considered steering ranges are listed in Table 2.1-2.

Table 2.1-2: Steering ranges

| Deployment scenario | Steering range (degrees) | Sub-array down-tilt (degrees) |
| --- | --- | --- |
| Macro rural | 90 to 97 | 3 |
| Macro urban | 90 to 105 | 6 |
| Dense urban | 90 to 112 | 6 |

The sampling instances of the vertical steering range can be defined as listed in Table 2.1-3.

Table 2.1-3: Sampling instances

| Steering range (degrees, below horizon) | Sampling instances | Note |
| --- | --- | --- |
| 90 to 97 | 8 4 | 1 deg. interval (1x1 1 deg. resolution) 2 deg. interval (2x1 1 deg. resolution) |
| 90 to 105 | 16 8 4 3 | 1 deg. interval (1x1 1 deg. resolution) 2 deg. interval (2x1 1 deg. resolution) 4 deg. interval (4x1 1 deg. resolution) 4 deg. interval (2x1 3 deg. resolution) |
| 90 to 112 | 23 6 4 3 | 1 deg. interval (1x1 1 deg. resolution) 3 deg. interval (2x1 2 deg. resolution) 5 deg. interval (3x1 2 deg. resolution) 7 deg. interval (4x1 2 deg. resolution) |

The vertical sampling instances per steering range are visualised in Figure 2.1-2.

# Figure 2.1-2: Vertical steering ranges

From all the above, we calculate the total numbers of K test beams to be evaluated for every antenna configuration from the above 23. Those that can support each vertical steering range are listed in Table 2.1-4.

Table 2.1-4: Beams with steering ranges

| Steering range (degrees) | Beams |
| --- | --- |
| 90 to 97 | 121x8 = 968 beams (reference) 11x8 = 88 beams 7x8 = 56 beams 7x4 = 28 beams 5x8 = 40beams 3x8 = 24 beams 5x4 = 20 beams 3x4 = 12 beams |
| 90 to 105 | 121x16 = 1936 beams (reference) 11x16 = 176 beams 7x16 = 112 beams 5x16 = 80 beams 7x8 = 56 beams 3x16 = 48 beams 5x8 = 40 beams 7x4 = 28 beams 3x8 = 24 beams 5x4 = 20 beams 3x4 = 12 beams 3x3 = 9 beams |
| 90 to 112 | 121x23 = 2783 beams (reference) 11x23 = 253 beams 7x23 = 161 beams 5x23 = 115 beams 3x23 = 69 beams 11x6 = 66 beams 7x6 = 42 beams 5x6 = 30 beams 5x4 = 20 beams 3x6 = 18 beams 3x4 = 12 beams 3x3 = 9 beams |

By performing the above analysis, we can observe the deviation with respect to the calculation of the EEIRP, when considering different antennas on different vertical coverages for a different number of K beams.

Example:

To calculate the CI for antenna configuration 1 for steering range 90 to 105 deg. and K=24 test beams (here M = 1936) the space is split in 8 vertical and 3 horizontal intervals as shown below and the methodology of random sampling (concept discussed at last RAN4 meeting) is applied.

Vertical: [0 1] [2 3] [4 5] [6 7] [8 9] [10 11] [12 13] [14 15]

Horizontal: [-60 -45 -30] [-15 0 15] [30 45 60]

According to the random selection of columns/rows the grid will look like as the pattern depicted in Figure 2.1-3.

# Figure 2.1-3: Beam distribution

And this means that the K=24 beams to be tested are:

[91,-45], [90,0], [90,60], [93,-60], [92,-15], [92,30], [95,-30], [94,15], [94,60], [97,-30], [96,15], [96,60], [99,-60], [98,-15], [98,30], [101,-45], [100,0], [100, 45], [103,-60], [102,-15], [102,30], [105,-45], [104,0] and [104,45]

For each of those individual beams, the expected EIRP is calculated per elevation bin (Xibfor beam ‘i’ and elevation bin ‘b’). Then, for each bin, the sample mean Xb and the sample variance sb2=1KXib-Xb 2K-1 is calculated. Due to sampling without replacement, the variance needs to be corrected with the population correction factor (1 – K/M) and thus the confidence interval for bin ‘b’ will be given by:

Confidence Interval: Xb ±ta,K-1 1-KMsb2K= Xb ±ta,K-1 1-241936sb224

The term ta,v  can be found from the student’s t-distribution tail table found in textbooks [1]. Assuming we are interested in the α = 95% confidence interval, it is found from the table that the term gets the value of 2.069. All the calculations to be done in linear scale and converted to dB at the end.

The simulation results for all three steering ranges are visualised in Figure 2.1-4.

| Steering range 90 to 97 deg. | Steering range 90 to 106 deg. | Steering range 90 to 112 deg. |
| --- | --- | --- |
| Confidence Interval for reference fine grid (968 beams): 0.48 dB | Confidence Interval for reference fine grid (1936 beams): 0.45 dB | Confidence Interval for reference fine grid (2783 beams): 0.43 dB |

# Figure 2.1-4: CI simulation results

Based on the results the following observations are made:

Observation 1: The following can be observed by analysing the CI simulation results:

- As expected, the maximum interval for a specific CI is significant, but less than 4.5 dB.

- The interval generally reduces when considering more beams.

- The elevation bin defined for the range 60 to 90 degrees (upwards) and 0 to 5 degrees (towards horizon) is generating largest interval.

Observation 2: It is expected that the EEIRP will vary for different types of BS array antenna implementations using different array geometries and number of considered beams. This behaviour is following the reasoning of the requirement derivation at WRC-23 and is a consequence of average EIRP.

Observation 3: Based on the analysis of CI for EEIRP per elevation bin, according to methodology 1, we cannot conclude on appropriate number of test beams. The beams are expected to deviate quite a lot within an elevation range, thus it is not reasonable to  use CI as a sole criterium to decide what is the appropriate number of test beams for a specific product.

The CI analysis on the deviation among different antennas would make more sense compared to the deviation among beams. This is presented in the next clause. The beams are expected to deviate quite a lot within an elevation range, thus it is not reasonable to be used as a mean to find an appropriate number of test beams.

  - Evaluation of EEIRP spread with respect to reference

In this section we analyse the impact of different test beam constellations to evaluate required test beam number for different steering ranges and BS array antenna geometries.

In the evaluation we consider a BS steering range -60 to 60 degrees along azimuth axis and 0 to 16 degrees along vertical axis. The beam sets are compared to a reference beam set defined as 17x121 beam constellation (2057 beams) within the steering range.

In our evaluation we have included 9 different beam constellations (4x3, 4x4, 6x3, 5x4, 4x5, 6x4, 5x5, 17 beams in a star and 20 random beams), visualised in Figure 2.2-1, Figure 2.2-2 and Figure 2.2-3.

# Figure 2.2-1: Beam constellation BS1 (left), BS2 (middle) and BS3 (right)

# Figure 2.2-2: Beam constellation BS4 (left), BS5 (middle) and BS6 (right)

# Figure 2.2-3: Beam constellation BS7 (left), BS8 (middle) and BS9 (right)

The beam sets have been constructed so that uniform (BS1, BS2, BS5) and non-uniform (BS3, BS4, BS6, BS7, BS8 and BS9) distributions with different number of total beams are included.

To include variations due to array antenna configuration several configurations including different array size and sub-array size was considered, as listed in Table 2.2-1.

Table 2.2-1: Array antenna configuration

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 |
| Element | Vertical HPBW (°) | 90 | 90 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | - | - | - | - |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 |
|  | Element power (dBm/100 MHz) | 0 | 0 | 0 | 0 |
| Array configuration | Number of rows | 16 | 32 | 8 | 8 |
|  | Number of columns | 8 | 16 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 4x1 |
|  | Sub-array pre-tilt (°) | - | - | 6 | 6 |
|  | Vertical element separation (λ) | 0.5 | 0.5 | 2.1 | 2.8 |
|  | Horizontal element separation (λ) | 0.5 | 0.5 | 0.5 | 0.5 |

#### Calculation methodology:

The EEIRP is calculated for each of the (seven) different elevation bins for each considered array antenna configuration and beam-set (BS1 to BS9) including the reference beam set with 2057 beams (17 along elevation axis and 121 along azimuth axis). The EEIRP difference between each beam set and the reference grid is calculated for each of the elevation bins.

For every combination of array configuration and beam set, three metrics are then considered based on this difference.

- The average deviation “AVG deviation” is the linear average for each elevation bin (average of 7 values, one for each elevation bin) and is intended to represent the deviation of the calculated EEIRP compared to the reference beam set.

- The maximum over-estimation “Max over-estimation” is the maximum positive value over the 7 bins and is intended to represent the largest error that is observed ABOVE the levels of the reference EEIRP.

- The maximum under-estimation “Max under-estimation” is the minimum negative value over the 7 bins and is intended to represent the largest error that is observed BELOW the levels of the reference EEIRP.

The aggregated results per beam-set as presented in Table 2.2-2 are the WORST values observed over the different array configurations, i.e. the highest values of “AVG deviation”, “Max over-estimation” and “Max under-estimation” between all considered antenna arrays.

The fourth metric “RMS_TBS_dBm” is the RMS representation of the error per beam set, over all antenna configurations and elevation bins where the error is considered as the difference of the beam-set specific EEIRP to that of the reference beam set.

i.e., for Beam Set (BS) 1 to 9,  RMSbs=  10log10mMnNEEIRPnmbs-EEIRPnmref2NM 
where EEIRPnm is the calculated EEIRP for antenna configuration m and elevation bin n.

The difference between the two variations of RMS (“on dB diff”, “on linear diff”) is that the EEIRP difference, EEIRPnmbs-EEIRPnmref, is taken in linear scale in one case and on dB scale (and converting afterwards to linear) on the other.

The “Mean CI on regular grid” is the average of the worst Confidence Interval calculations, which are based on the beams in the corresponding regular-grid beam-sets, over the different elevation bins (i.e worst Confidence Interval over the different antenna configurations.)

In Table 2.2-2, the simulation result is concluded for each considered Beam Set (BS1 to BS9). The Beam Set is defined as Mrow x Ncolumn.

Table 2.2-2: Simulation result

|  | BS1 (4x3) | BS2 (4x4) | BS3 (6x3) | BS4 (5x4) | BS5 (4x5) | BS6 (6x4) | BS7 (5x5) | BS8 (Star) | BS9 (Random) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVG deviation | 1,51 | 1,48 | 0,36 | 0,34 | 1,53 | 0,36 | 0,39 | 0,65 | 2,51 |  |  |
| Max over-estimation | 1,21 | 1,19 | 0,24 | 0,84 | 1,19 | 0,61 | 1,23 | 1,68 | 6,87 |  |  |
| Max under-estimation | 4,45 | 4,46 | 1,11 | 1,04 | 4,47 | 1,11 | 1,05 | 0,61 | 0,99 |  |  |
| RMS_TBS_dBm | 0,56 | 0,55 | 0,15 | 0,14 | 0,57 | 0,14 | 0,13 | 0,15 | 0,97 | on dB diff |  |
| RMS_TBS_dBm | -6,8 | -6,8 | -12,1 | -15,3 | -6,8 | -12,0 | -15,2 | -13,0 | -12,7 | on linear diff |  |
| Mean CI on regular grid | 2,67 | 2,3 | 2,4 | 2,27 | 2,03 | 2,07 | 2,04 |  |  |  |  |

The result is colour coded to visualize good and bad beam sets. The beam set within a box are the ones selected for comparison. In addition, some more was added to analyse impact due to larger constellations.

It is interesting to compare the result for BS4 (5x4) and BS5 (4x5). It can be noticed that the number of beams along the vertical axis is essential to reduce deviations to acceptable levels.

Observation 4: The simulation result shows that Beam Set BS3, BS4, BS6, BS7 and BS8 produce acceptable deviations with respect to the reference case. Number of beams along the vertical axis is essential to reduce CI deviation. It is interesting to compare the result for BS4 (5x4) and BS5 (4x5). It can be concluded that the number of beams along the vertical axis is essential to reduce deviations to acceptable levels.

Observation 5: The simulation results indicate that following beams sets should be considered for conformance testing: BS3 (6x3), BS4 (5x4), BS6 (6x4), BS7 (5x5) and BS8 (17 beams in a star). The sizes of these beam sets ranges from 18 to 25 beams distributed within the steering range.

To guarantee robust conformance testing of different AAS BS implementations with support for narrow and wide vertical steering range BS7 seems to be the best candidate for EEIRP test procedure to be described in TS 38.141-2.

Proposal 2: Based on results from methodology 2 adopt a beam set with 5x5 beams uniformly distributed within the steering rage.

A detailed overview of all results including EEIRP deviation for all bins per array antenna configuration can be found in separate Annex in section 5 of this document.

It is worth to mention that simulation results provided to RAN4#113 in Orlando in [1] indicates similar trends as simulation results presented in this contribution.

# Conclusion

In this contribution we present simulation results and technical background information to TR 38.908. In our simulations we study the impact on calculated EEIRP levels for different beam constellations. It is essential to understand the relation between required minimum number of test beams and accuracy of EEIRP estimation. The study holds two different approaches producing results indicating relations between number of beams and how accurate EEIRP is estimated.

Definition of the test beam constellation has significant impact on the expected measurement result. Among all 9 beam sets considered in our study is can be concluded that BS3, BS4, BS6, BS7 and BS8 is suitable for conformance testing purposes.

To collect technical background information a text proposal to TR 38.908 has been created. The text proposal is attached at the end of this contribution.

Based on the simulations results we have observed following:

Observation 1: The following can be observed by analysing the CI simulation results:

- As expected, the maximum interval for a specific CI is significant but less than 4.5 dB.

- The interval generally reduces considering more beams.

- The elevation bin defined for the range 60 to 90 degrees (upwards) and 0 to 5 degrees (towards horizon) is generating largest interval.

Observation 2: It is expected that the EEIRP will vary for different types of BS array antenna implementations using different array geometries and number of considered beams. This behaviour is following the reasoning of the requirement derivation at WRC-23 and is a consequence of average EIRP.

Observation 3: Based on the analysis of CI for EEIRP per elevation bin, according to methodology 1, we cannot conclude on appropriate number of test beams. The beams are expected to deviate quite a lot within an elevation range, thus it is not reasonable to  use CI as a sole criterium to decide what is the appropriate number of test beams for a specific product.

Observation 4: The simulation result shows that Beam Set BS3, BS4, BS6, BS7 and BS8 produce acceptable deviations with respect to the reference case. Number of beams along the vertical axis is essential to reduce CI deviation. Observation 5: The simulation results indicates that following beams sets should be considered for conformance testing: BS3 (6x3), BS4 (5x4), BS6 (6x4), BS7 (5x5) and BS8 (17 beams in star). The sizes of these beam sets ranges from 18 to 25 beams distributed within the steering range.

Based on the observation we propose following proposals to progress the work:

Proposal 1: Include information from text proposal attached at the end of this contribution to TR 38.908.

Proposal 2: Based on results from methodology 2 adopt a beam set with 5x5 beams uniformly distributed within the steering rage.

# References

[1]	R4-2419586, “Discussion on conformance test aspects for EEIRP and TP to TR 38.908”, Ericsson

[2] 	R4-2511649, “TP for TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104”, RAN4 #114, Ericsson

[3]	https://en.wikipedia.org/wiki/Student%27s_t-distribution

# Annex: EEIRP deviation per bin

Details related to EERIP deviations for different array antenna configuration is listed in Table 5-1, Table 5-2, Table 5-3 and Table 5-4.

Table 5-1: Config 1

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -0,963 | -0,993 | -0,261 | 0,05 | -0,999 | -0,291 | 0,044 | -0,504 | -0,2 |
| 2 | 5<q<10 | -0,21 | -0,233 | -0,275 | -0,393 | -0,238 | -0,298 | -0,398 | -0,042 | 0,02 |
| 3 | 10<q<15 | -0,389 | -0,403 | -0,142 | -0,021 | -0,406 | -0,155 | -0,024 | -0,001 | -0,013 |
| 4 | 15<q<20 | 0,012 | 0,014 | 0,12 | 0,183 | 0,018 | 0,123 | 0,186 | -0,076 | -0,039 |
| 5 | 20<q<30 | -0,048 | -0,027 | -0,077 | -0,071 | -0,016 | -0,057 | -0,059 | 0,027 | 0,009 |
| 6 | 30<q<60 | 0,208 | 0,069 | 0,219 | 0,082 | 0,049 | 0,08 | 0,06 | 0,039 | -0,057 |
| 7 | 60<q<90 | -0,338 | 0,246 | -0,369 | 0,273 | 0,089 | 0,271 | 0,087 | 0,064 | -0,311 |
| Average deviation |  | 0,32 | 0,30 | 0,21 | 0,16 | 0,27 | 0,18 | 0,12 | 0,11 | 0,09 |
| Max over-estimation |  | 0,21 | 0,25 | 0,22 | 0,27 | 0,09 | 0,27 | 0,19 | 0,06 | 0,02 |
| Max under-estimation |  | 0,96 | 0,99 | 0,37 | 0,39 | 1,00 | 0,30 | 0,40 | 0,50 | 0,31 |
| RMS el_bin |  | 0,33 | 0,31 | 0,21 | 0,16 | 0,29 | 0,18 | 0,13 | 0,11 | 0,10 |

Table 5-2: Config 2

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -3,351 | -3,36 | -0,6 | -0,018 | -3,372 | -0,608 | -0,03 | -0,421 | -0,302 |
| 2 | 5<q<10 | -0,505 | -0,517 | -0,33 | -0,47 | -0,527 | -0,341 | -0,481 | -0,079 | 0,062 |
| 3 | 10<q<15 | 0,285 | 0,269 | -0,141 | -0,05 | 0,263 | -0,156 | -0,057 | -0,017 | -0,092 |
| 4 | 15<q<20 | -0,537 | -0,55 | 0,245 | 0,344 | -0,557 | 0,231 | 0,34 | -0,024 | -0,021 |
| 5 | 20<q<30 | 0,177 | 0,175 | -0,162 | -0,241 | 0,161 | -0,163 | -0,257 | 0,036 | 0,025 |
| 6 | 30<q<60 | 0,093 | -0,012 | 0,204 | 0,121 | -0,04 | 0,098 | 0,086 | 0,028 | -0,058 |
| 7 | 60<q<90 | 0,011 | -0,056 | -0,356 | 0,493 | 0,224 | 0,393 | 0,086 | 0,008 | -0,09 |
| Average deviation |  | 0,87 | 0,87 | 0,29 | 0,25 | 0,89 | 0,29 | 0,19 | 0,09 | 0,09 |
| Max over-estimation |  | 0,29 | 0,27 | 0,24 | 0,49 | 0,26 | 0,39 | 0,34 | 0,04 | 0,06 |
| Max under-estimation |  | 3,35 | 3,36 | 0,60 | 0,47 | 3,37 | 0,61 | 0,48 | 0,42 | 0,30 |
| RMS el_bin |  | 1,08 | 1,08 | 0,30 | 0,26 | 1,10 | 0,29 | 0,20 | 0,09 | 0,09 |

Table 5-3: Config 3

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -3,234 | -3,242 | -0,539 | -0,001 | -3,255 | -0,548 | -0,014 | -0,39 | -0,269 |
| 2 | 5<q<10 | -0,802 | -0,814 | -0,936 | -1,043 | -0,824 | -0,948 | -1,054 | -0,048 | -0,727 |
| 3 | 10<q<15 | -0,422 | -0,445 | -0,099 | 0,088 | -0,447 | -0,121 | 0,085 | -0,606 | -0,214 |
| 4 | 15<q<20 | 0,042 | 0,015 | -0,031 | -0,095 | 0,018 | -0,058 | -0,092 | 0,327 | 0,444 |
| 5 | 20<q<30 | -0,418 | -0,424 | -0,042 | 0,114 | -0,446 | -0,05 | 0,095 | -0,443 | 1,064 |
| 6 | 30<q<60 | -0,039 | -0,262 | 0,101 | 0,032 | -0,44 | -0,081 | -0,113 | 0,02 | 2,849 |
| 7 | 60<q<90 | -0,557 | 0,224 | -0,297 | 0,843 | -0,517 | 0,612 | -0,592 | 0,366 | 6,869 |
| Average deviation |  | 0,93 | 0,92 | 0,30 | 0,34 | 0,98 | 0,36 | 0,31 | 0,32 | 2,51 |
| Max over-estimation |  | 0,04 | 0,22 | 0,10 | 0,84 | 0,02 | 0,61 | 0,10 | 0,37 | 6,87 |
| Max under-estimation |  | 3,23 | 3,24 | 0,94 | 1,04 | 3,26 | 0,95 | 1,05 | 0,61 | 0,73 |
| RMS el_bin |  | 1,10 | 1,10 | 0,32 | 0,36 | 1,15 | 0,37 | 0,32 | 0,32 | 3,42 |

Table 5-4: Config 4

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -4,447 | -4,455 | -1,105 | -0,585 | -4,467 | -1,113 | -0,597 | -0,355 | -0,99 |
| 2 | 5<q<10 | 0,415 | 0,405 | 0,047 | 0,011 | 0,396 | 0,037 | 0,003 | -0,474 | 0,182 |
| 3 | 10<q<15 | 1,211 | 1,194 | 0,088 | 0,355 | 1,188 | 0,07 | 0,349 | 0,66 | 0,165 |
| 4 | 15<q<20 | 0,914 | 0,894 | -0,058 | -0,263 | 0,895 | -0,077 | -0,265 | -0,509 | 0,605 |
| 5 | 20<q<30 | -0,833 | -0,837 | -0,299 | -0,138 | -0,854 | -0,305 | -0,151 | -0,532 | 0,722 |
| 6 | 30<q<60 | -0,472 | -0,443 | -0,088 | -0,052 | -0,391 | -0,152 | -0,022 | -0,173 | 3,088 |
| 7 | 60<q<90 | -0,567 | 0,348 | -0,73 | 0,805 | 0,822 | 0,109 | 1,234 | 1,68 | 6,215 |
| Average deviation |  | 1,51 | 1,48 | 0,36 | 0,32 | 1,53 | 0,28 | 0,39 | 0,65 | 2,30 |
| Max over-estimation |  | 1,21 | 1,19 | 0,09 | 0,81 | 1,19 | 0,11 | 1,23 | 1,68 | 6,22 |
| Max under-estimation |  | 4,45 | 4,46 | 1,11 | 0,59 | 4,47 | 1,11 | 0,60 | 0,53 | 0,99 |
| RMS el_bin |  | 1,83 | 1,81 | 0,38 | 0,33 | 1,85 | 0,30 | 0,41 | 0,68 | 3,02 |

TEXT PROPOSAL:

# 1	Scope

The present document is a technical report to capture the background on new BS EIRP mask requirement for band n104 for protection of fixed satellite service (FSS) UL within 6425-7125MHz and how to conduct the conformance testing for the compliance of the new BS EIRP mask requirement.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU World Radiocommunication Conference 2023 (WRC-23) Final Acts, Resolution 220

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	 elevation angle above horizon (defined between 0° and 90°)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EIRP	Effective Isotropic Radiated Power

EEIRP	Expected EIRP

FSS	Fixed Satellite Service

NR	New Radio

RF	Radio Frequency

# Background

The frequency band of 6425 to 7125 MHz and parts thereof was identified for International Mobile Telecommunications (IMT) use by WRC 23 in different ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the Expected Equivalent Isotropic Radiated Power (EEIRP) spectral density of IMT base-stations for protecting Earth-to-Space Fixed Satellite Services (FSS) as in Resolution 220 (WRC-23). The EEIRP is subject to a new regulatory requirement specified as a mask for over-the-horizon emissions. The Annex to the Resolution 220 (WRC-23) [2] outlines a calculation of the EEIRP of an IMT base station for assessing the compliance of the IMT base station equipment with the limit on EEIRP.

The aim of this report is the following:

- To capture the technical background information relevant for the core requirement in BS RF core specification (i.e. TS 38.104 and other nodes like IAB, Repeater and NCR).

- To capture the technical background information relevant for the test procedures in the conformance test specification (i.e. TS 38.141-2 and other nodes like IAB, Repeater and NCR).

Having the requirement included in a 3GPP standard would guarantee a harmonized terminology and conformance test methodologies. Eventually, the concept for conformance testing can be adopted to similar situations involving co-channel spectrum sharing between IMT and FSS UL.

As part of the WRC-23, resolution 220, EEIRP for an IMT BS was introduced based on calculating average radiating power for seven elevation bins. Since the protection of FFS UL receiver is based on aggregated power from many base stations, the requirement breakdown to a single BS is based on an average EIRP pattern for a single BS, as used during normal operation. WRC-23 decided to adopt a concept where the average EIRP pattern is extracted based on measuring the EIRP patterns for many sequentially generated narrow beams. The choice of these narrow beams should be relevant for the coverage area considered for the BS in normal operation.

# 5	RF requirement

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions. This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the limits specified in table 5-1.

The expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed:

‒ over azimuth angles from −180º to +180º with reference to BS horizontal boresight and over the specified range of angles θHLi ≤ θ < θHHi , above the horizon as defined in Table 5-1, and

‒ over the supported beam directions.

Note: The beamforming directions over time means directions are measured sequentially in the test.

Table 5-1: EEIRP limits as function of elevation above horizon

| Bin number | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<30 | 16 |
| 6 | 30≤<60 | 15 |
| 7 | 60≤<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

The definitions of elevation angle θHLi and θHHi are illustrated in Figure 5-1.

Figure 5-1: Definitions of θHLi and θHHi angles.

# 6	Conformance testing requirement and procedures

## 6.1	General

The new requirement from ITU Resolution 220 [2] implies a challenging conformance testing, due to following aspects:

- Measurements can be performed on a limited set of test beams

- The set of test beams might be different for different supported mechanical tilts

The EEIRP estimation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

## 6.2	The calculation for EEIRP mask

    - 6.2.1  Angle definitions

The angles in the spherical coordinate system used in this section is as shown in figure 6.2.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 6.2.1-1: Angle definition for EEIRP

    - 6.2.2 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θHL and θHH as shown in Figure 6.2.1-1.

Figure 6.2.2-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRP EIRPθ,φ is measured in discrete steps and equation (1) needs to be written in discrete form, as described in the next section.

It should be noted that all the averaging is performed in linear units.

    - 6.2.2.1 Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 5.1.1.1-1 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over two discrete elevation angles with equal number of points in azimuth.

Figure 6.2.2.1-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.2.1.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.2.2.1-2: Illustration of the parameters involved

[For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθHLi , θHHi=1360°×sinθHHi-sinθHLiθLθH-180°180°EIRPθ,φ cosθ ∆φ ∆θ    ,                     2

By substituting the following:

dφ≈∆φ=360°M

dθ≈∆θ=θHHi-θHLiN

the overall equation can be written as:

EIRPθHLi , θHHi=θHHi-θHLi×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       3

where

θn	is the nth midpoint between the elevation intervals (midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°).

φm	         is the mth midpoint between the azimuth intervals

nL	         is the lowest elevation point within the θHLi , θHHi range

nH	         is the highest elevation point within the θHLi , θHHi range

M	         is the number of azimuth points within the (-180°, 180°) range

N	         is the number of elevation points within the θHLi , θHHi range]

Note: The equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

## 6.3	Test beams

The BS capability to provide coverage for different deployment situations is declared by the BS manufacturer. The declaration relying on declaring parameters for minimum angle along the q axis, maximum angle along the q axis, minimum angle along j axis and maximum angle along j axis, as visualised in Figure 6.3-1. The BS manufacture can declare multiple steering ranges to be used for different types of deployment scenarios.

# Figure 6.3-1: Declaration of steering range

### 6.3.1	Test beam set evaluation

The evaluation of the EEIRP estimation accuracy is highlighted as a function of number of test beams considered within the supported steering range.

#### 6.3.1.1	Company A

{Editor’s note: Simulation results from company A}

In the evaluation we consider a BS steering range -60 to 60 degrees along azimuth axis and 0 to 16 degrees along vertical axis. The beam sets are compared to a reference beam set defined as 17x121 beam constellation (2057 beams) within the steering range.

In our evaluation we have included 9 different beam constellations (4x3, 4x4, 6x3, 5x4, 4x5, 6x4, 5x5, 17 beams in ellipsoid and 20 random beams), visualised in Figure 6.3.1.1-1, Figure 6.3.1.1-2 and Figure 6.3.1.1-3.

# Figure 6.3.1.1-1: Beam constellation BS1 (left), BS2 (middle) and BS3 (right)

# Figure 6.3.1.1-2: Beam constellation BS4 (left), BS5 (middle) and BS6 (right)

# Figure 6.3.1.1-3: Beam constellation BS7 (left), BS8 (middle) and BS9 (right)

The beam sets have been constructed so that uniform (BS1, BS2, BS5) and non-uniform (BS3, BS4, BS6, BS7, BS8 and BS9) distributions with different number of total beams are included.

To include variations due to array antenna configuration several configurations including different array size and sub-array size were considered, as listed in Table 6.3.1.1-1.

Table 6.3.1.1-1: Array antenna configuration

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 |
| Element | Vertical HPBW (°) | 90 | 90 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | - | - | - | - |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 |
|  | Element power (dBm/100 MHz) | 0 | 0 | 0 | 0 |
| Array configuration | Number of rows | 16 | 32 | 8 | 8 |
|  | Number of columns | 8 | 16 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 4x1 |
|  | Sub-array pre-tilt (°) | - | - | 6 | 6 |
|  | Vertical element separation (λ) | 0.5 | 0.5 | 2.1 | 2.8 |
|  | Horizontal element separation (λ) | 0.5 | 0.5 | 0.5 | 0.5 |

The result is listed in Table 6.3.1.1-1, Table 6.3.1.1-2, Table 6.3.1.1-3 and Table 6.3.1.1-4.

Table 6.3.1.1-1: Config 1

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -0,963 | -0,993 | -0,261 | 0,05 | -0,999 | -0,291 | 0,044 | -0,504 | -0,2 |
| 2 | 5<q<10 | -0,21 | -0,233 | -0,275 | -0,393 | -0,238 | -0,298 | -0,398 | -0,042 | 0,02 |
| 3 | 10<q<15 | -0,389 | -0,403 | -0,142 | -0,021 | -0,406 | -0,155 | -0,024 | -0,001 | -0,013 |
| 4 | 15<q<20 | 0,012 | 0,014 | 0,12 | 0,183 | 0,018 | 0,123 | 0,186 | -0,076 | -0,039 |
| 5 | 20<q<30 | -0,048 | -0,027 | -0,077 | -0,071 | -0,016 | -0,057 | -0,059 | 0,027 | 0,009 |
| 6 | 30<q<60 | 0,208 | 0,069 | 0,219 | 0,082 | 0,049 | 0,08 | 0,06 | 0,039 | -0,057 |
| 7 | 60<q<90 | -0,338 | 0,246 | -0,369 | 0,273 | 0,089 | 0,271 | 0,087 | 0,064 | -0,311 |
| Average deviation |  | 0,32 | 0,30 | 0,21 | 0,16 | 0,27 | 0,18 | 0,12 | 0,11 | 0,09 |
| Max over-estimation |  | 0,21 | 0,25 | 0,22 | 0,27 | 0,09 | 0,27 | 0,19 | 0,06 | 0,02 |
| Max under-estimation |  | 0,96 | 0,99 | 0,37 | 0,39 | 1,00 | 0,30 | 0,40 | 0,50 | 0,31 |
| RMS el_bin |  | 0,33 | 0,31 | 0,21 | 0,16 | 0,29 | 0,18 | 0,13 | 0,11 | 0,10 |

Table 6.3.1.1-2: Config 2

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -3,351 | -3,36 | -0,6 | -0,018 | -3,372 | -0,608 | -0,03 | -0,421 | -0,302 |
| 2 | 5<q<10 | -0,505 | -0,517 | -0,33 | -0,47 | -0,527 | -0,341 | -0,481 | -0,079 | 0,062 |
| 3 | 10<q<15 | 0,285 | 0,269 | -0,141 | -0,05 | 0,263 | -0,156 | -0,057 | -0,017 | -0,092 |
| 4 | 15<q<20 | -0,537 | -0,55 | 0,245 | 0,344 | -0,557 | 0,231 | 0,34 | -0,024 | -0,021 |
| 5 | 20<q<30 | 0,177 | 0,175 | -0,162 | -0,241 | 0,161 | -0,163 | -0,257 | 0,036 | 0,025 |
| 6 | 30<q<60 | 0,093 | -0,012 | 0,204 | 0,121 | -0,04 | 0,098 | 0,086 | 0,028 | -0,058 |
| 7 | 60<q<90 | 0,011 | -0,056 | -0,356 | 0,493 | 0,224 | 0,393 | 0,086 | 0,008 | -0,09 |
| Average deviation |  | 0,87 | 0,87 | 0,29 | 0,25 | 0,89 | 0,29 | 0,19 | 0,09 | 0,09 |
| Max over-estimation |  | 0,29 | 0,27 | 0,24 | 0,49 | 0,26 | 0,39 | 0,34 | 0,04 | 0,06 |
| Max under-estimation |  | 3,35 | 3,36 | 0,60 | 0,47 | 3,37 | 0,61 | 0,48 | 0,42 | 0,30 |
| RMS el_bin |  | 1,08 | 1,08 | 0,30 | 0,26 | 1,10 | 0,29 | 0,20 | 0,09 | 0,09 |

Table 6.3.1.1-3: Config 3

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -3,234 | -3,242 | -0,539 | -0,001 | -3,255 | -0,548 | -0,014 | -0,39 | -0,269 |
| 2 | 5<q<10 | -0,802 | -0,814 | -0,936 | -1,043 | -0,824 | -0,948 | -1,054 | -0,048 | -0,727 |
| 3 | 10<q<15 | -0,422 | -0,445 | -0,099 | 0,088 | -0,447 | -0,121 | 0,085 | -0,606 | -0,214 |
| 4 | 15<q<20 | 0,042 | 0,015 | -0,031 | -0,095 | 0,018 | -0,058 | -0,092 | 0,327 | 0,444 |
| 5 | 20<q<30 | -0,418 | -0,424 | -0,042 | 0,114 | -0,446 | -0,05 | 0,095 | -0,443 | 1,064 |
| 6 | 30<q<60 | -0,039 | -0,262 | 0,101 | 0,032 | -0,44 | -0,081 | -0,113 | 0,02 | 2,849 |
| 7 | 60<q<90 | -0,557 | 0,224 | -0,297 | 0,843 | -0,517 | 0,612 | -0,592 | 0,366 | 6,869 |
| Average deviation |  | 0,93 | 0,92 | 0,30 | 0,34 | 0,98 | 0,36 | 0,31 | 0,32 | 2,51 |
| Max over-estimation |  | 0,04 | 0,22 | 0,10 | 0,84 | 0,02 | 0,61 | 0,10 | 0,37 | 6,87 |
| Max under-estimation |  | 3,23 | 3,24 | 0,94 | 1,04 | 3,26 | 0,95 | 1,05 | 0,61 | 0,73 |
| RMS el_bin |  | 1,10 | 1,10 | 0,32 | 0,36 | 1,15 | 0,37 | 0,32 | 0,32 | 3,42 |

Table 6.3.1.1-4: Config 4

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<q<5 | -4,447 | -4,455 | -1,105 | -0,585 | -4,467 | -1,113 | -0,597 | -0,355 | -0,99 |
| 2 | 5<q<10 | 0,415 | 0,405 | 0,047 | 0,011 | 0,396 | 0,037 | 0,003 | -0,474 | 0,182 |
| 3 | 10<q<15 | 1,211 | 1,194 | 0,088 | 0,355 | 1,188 | 0,07 | 0,349 | 0,66 | 0,165 |
| 4 | 15<q<20 | 0,914 | 0,894 | -0,058 | -0,263 | 0,895 | -0,077 | -0,265 | -0,509 | 0,605 |
| 5 | 20<q<30 | -0,833 | -0,837 | -0,299 | -0,138 | -0,854 | -0,305 | -0,151 | -0,532 | 0,722 |
| 6 | 30<q<60 | -0,472 | -0,443 | -0,088 | -0,052 | -0,391 | -0,152 | -0,022 | -0,173 | 3,088 |
| 7 | 60<q<90 | -0,567 | 0,348 | -0,73 | 0,805 | 0,822 | 0,109 | 1,234 | 1,68 | 6,215 |
| Average deviation |  | 1,51 | 1,48 | 0,36 | 0,32 | 1,53 | 0,28 | 0,39 | 0,65 | 2,30 |
| Max over-estimation |  | 1,21 | 1,19 | 0,09 | 0,81 | 1,19 | 0,11 | 1,23 | 1,68 | 6,22 |
| Max under-estimation |  | 4,45 | 4,46 | 1,11 | 0,59 | 4,47 | 1,11 | 0,60 | 0,53 | 0,99 |
| RMS el_bin |  | 1,83 | 1,81 | 0,38 | 0,33 | 1,85 | 0,30 | 0,41 | 0,68 | 3,02 |

#### 6.3.1.2	Company B

{Editor’s note: Simulation results from company B}

#### 6.3.1.3	Company C

{Editor’s note: Simulation results from company C}

### 6.3.2	Description of test beam set

Considering uniformly distributed test beam directions within the declared steering and the steering range is divided into K angular sub-regions with a test beam in the center of each sub-region. Then the test beam constellation consisting of K=AB test beam directions can be described as:

θa,b=aAθmax-θmin-θmax-θmin2+θminφa,b=bBφmax-φmin-φmax-φmin2+φmin

, where a=1..A, b=1..B, A=TBD rows and B=TBD columns. Parameters jmin, jmax, qmin and qmax is provided from manufacturer declaration of steering range.

## 6.4	Spatial sampling grid

Considering the requirement for OTA spatial emission above the horizon there are two aspects to consider in relation on how to define the EIRP pattern spatial resolution required to guarantee accurate measurement of EEIRP:

- The sampling resolution required to capture the radiation above the horizon. For this sufficient number of spatial samplers are needed to sufficiently capture emission generated in the upper hemisphere including side lobes and grating lobes.

- The size of the elevation bins varies over the elevation range from 5 degrees at low elevations to 30 degrees at high elevation angles. This means that the number of vertical samples with is limited by the bin size.

The first aspect can be resolved by taking enough spatial samples required the spatial sampling Nyquist criteria.

Considering the minimum bin size of 5 degrees in the WRC-23 outcome, it would be safe to set the maximum vertical sampling resolution to 1 degree.

For the horizontal sampling resolution, the maximum sampling resolution can be determined by the physical size of the array antenna panel, or the narrowest beamwidth produced by the array antenna. Which both requires additional manufacturer declarations or access for physical inspection.

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD

, where l is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=MMsubdel2+Ndaz2

, where M is the number of sub-array rows, Msub is the number of elements in the sub-array, del is the vertical element separation, N is the number of sub-array columns and daz is the azimuth element separation.

In Table 6.4-1, the maximum sampling resolution step along the horizontal axis is listed for some example array sizes assuming element separation del=0.7l and daz=0.5l.

Table 6.4-1: Maximum allowed sampling resolution

| Array configuration (Msubx1, MxN) | Horizontal sampling resolution, Dj (deg.) |
| --- | --- |
| 3x1, 4x8 | 6.2 |
| 3x1, 8x16 | 3.1 |
| 3x1, 16x32 | 1.5 |

Δϕref=180∘πλDcyl

Dcyl=d2+w2

∆ϕref=λλo BeWϕ

## 6.5	Method of test

### 6.5.1	General

Traditionally, the measurement can be divided into two different stages: Test range calibration and measurement. For an Expected EIRP measurement a third stage is introduced.

      - Test range pathloss calibration and measurement receiver absolute power level calibration.

      - EIRP pattern measurements for a set of defined test beam directions.

      - Post-processing to calculate EEIRP for defined elevation bins.

The post-processing stage is required for some test methods, e.g., TRP test methods which are based on spatial EIRP samples. For EEIRP, the post processing stage consists of two sub-stages:

            - Calculation of average EIRP pattern

            - Calculation of average EIRP within specified elevation bin (referred to EEIRP per bin).

For each test beam, the EIRP pattern above the horizon (in the upper hemi-sphere) is measured for horizontal angles within the range -π≤φ≤π and elevation angles within the range 0≤θ≤π2 .

The discrete EIRP pattern EIRPkθn,φm measured per test beam using a specific sampling grid with a specific sampling grid resolution. For the reference test method described in this document a uniform sampling grid is considered. Considering practical measurement aspects, such as test complexity and test time the EIRP pattern per test beam is measured for a set of discrete spatial angles θn,φm, where m=1..M and n=1..N.

The EIRP pattern measurement stage and corresponding details on maximum allowed sampling resolution needs to be documented in an Annex in TS 38.141-2.

In technical background information from WRC-23 the EEIRP profile is calculated from an average EIRP pattern followed an integration over elevation bins and horizontal angles.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=1Kk=1KEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam.

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,φmcosθn

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the  (θLi, θHi) bounding range, with  N= nHi-nLi ; M is the number of sampling points in the azimuth range.

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.

In TS 38.141-2 clause 4.14 there is a definition of reference coordinate system used for the present requirements. For this requirement, RAN4 may need to define the elevation angle according to conventions used in ITU-R.

A new Annex would be required in the conformance test specification for the technical background information related to the post-processing calculations.

In the calibration stage the antenna test range transmission loss is characterized with the intention to minimize error sources related to test setup. A common approach used is the substitution method, where a reference antenna with known antenna gain is used to determine the transmission loss in the test range. The test range calibration is described in detail in TR 37.941, subclause 8.3.

In addition, the measurement uncertainty can be improved by calibrating the absolute power measurement uncertainty of the measurement receiver (spectrum analyzer with fairly high absolute power measurement uncertainty) using a power meter (with very low measurement uncertainty). A detailed description of suitable test ranges and corresponding calibration procedures can be found in TR 37.941, subclause 8.3.1.

### 6.5.2	Initial conditions

The initial conditions section contains the test set up initial conditions including:

- the test environment,

- the channels to be tested

- the beams to be tested

- etc

Test environment

EEIRP is to be measured under normal conditions

NOTE: Currently, only a few parameters are measured under extreme conditions which is very challenging as the OTA chamber cannot be environmentally controlled. Controlling the temperature of the DUT locally is implemented for limited tests, but this is not practical to EEIRP measurements.

RF channels

EEIRP measurement is a very long measurement, the number of channels tested will increase the test time.

The frequency variation across the EEIRP band is approximately 10%, over this range, the gain of the main beam will slightly change. However, EEIRP estimated value is dominated by the sidelobe behaviour of the antenna pattern rather than the main beam. As such, looking at the EEIRP over the frequency range there is no clear worst case, the performance is no worse at Bottom channel or Top channel than it is at the Middle channel.

As no RF channel is worse than any other it could be argued any channel could be used. Middle channel is the obvious choice.

Test signal and test model

As EEIRP requirement is based on power spectral density (PSD), testing should be carried out with the highest PSD the BS is capable to support.

NOTE: Maximum PSD might be achieved when using the smallest channel bandwidth, despite limited output power.

The performance is dependent on the radiation beam shape, non-linearities or distortions potentially generated by multi-carrier signals will not significantly alter the beam shape of the wanted signal.

For this reason single carrier test signal is sufficient for the scope of conformance testing, together with test model NR-FR1-TM1.1.

### 6.5.3	Test procedure

The test procedure is based on a CATR/IAC where the BS is placed on a positioner and rotated so the required measurement direction is in line with the measurement antenna. The procedure has similarities to existing EIRP and TRP measurement procedures.

The test procedure is as follows:

1. Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3

2. Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

3. Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

4. The measurement device characteristics shall be:

-	Detection mode: True RMS.

5. Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

6. Set the positioner at location for measurement point θn,φm for m=1..M and n=1.. N.

7. Enable test beam  k for k=1..K as described in subclause 6.3.2 for all declared steering ranges (D.10).

8. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

9. Repeat step 6 and 7 for all K test beams.

10. Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

11. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in subclause 6.5.

NOTE: step 9 and step 10 could be exchangeable.

## 6.6	Measurement uncertainty

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets from TR 37.941 can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

One major difference with EEIRP, compared to TRP, is the necessity to measure complete EIRP pattern with focus on the sidelobe region (i.e. for a real-life site this is the region pointing towards the sky). This means measurement receiver must be considered in such way that the dynamic range can handle the power created by the peak main beam as the maximum level and detecting the low levels in the side lobe region with accurate power. The power level accuracy is typically different for high power with large SNR compared to coming closer to the noise floor of the measurement receiver. For a specific test range the dynamic range can be calibrated with a power meter and would probably lead to an uncertainty value of around [0.3 dB] TBC.

EEIRP sampling Error

The EEIRP sampling error is caused by the fact that coverage steering range is sampled with a limited set of test beams.

The measure EEIRP profile using different granularity of test beams distributed within the setting range will proceed error within EEIRP bin. All the errors over bins are averaged to produce the total EEIRP sampling error.

These two errors have been added to the MU budget as follows:

| Table 6.6-1: IAC MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.26 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | 0.30 | Gaussian | 1 | 1 |  |  |  |  | 0.30 |  |
| *new | [EEIRP sampling error] |  |  | TBD | Gaussian | 1 | 1 |  |  |  |  | TBD |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Table 6.6-1: CATR MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.260 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | 0.30 | Gaussian | 1 | 1 |  |  |  |  | 0.30 |  |
| *new | [EEIRP sampling error] |  |  | TBD | Gaussian | 1 | 1 |  |  |  |  | TBD |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.