3GPP TSG-RAN WG4 Meeting #114                                        R4-2501674

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              Discussion on general aspects and requirements impact

Agenda item:			7.21.2

Document for:	Discussion

1. Introduction

Some AI mobility general aspects were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1]. The updated RAN2 agreements can be find in Appendix.

2 Discussion

2.1 General and Scope

- Sub-use-case priorities

Observations from RAN2 simulation results

- Based on the observations summerized in our Tdoc submitted in last meeting, for RRM measurement prediction, at least 5 dimensional discussions can be involved in AI mobility (Evaluation scenarios, Sub-use-cases including cell-specific and/or cluster approaches, Sliding/Non-sliding filtering, OW and PW including MRRT/MRRS ratio), for each dimension, multiple elements are contained. According to the simulation results, the variation in each element in each dimension can indirectly/directly affects the prediction accuracy. However, considering full combinations of above all dimensions to define RRM requirements are unnecessary. In this sense, it is necessary to down select and focus on some/one of them to define requirements.

- The simulation results evaluation for different Sub-use-cases based on simulation assumptions has begun in RAN2. Although RAN2 did not reach consensus on which Sub-use-case can be taken as the starting point/highest priority, based on their simulation analyses and observations,

  - majority of companies suggest to focus on L3 filtered cell level RSRP prediction since it can achieve excellent performance intuitively at least with shorter prediction window. While, the issue of how to define filtering scheme for input L3-RSRP of the AI model for FR1 intra-frequency intra-cell temporal domain case B is inevitable and pending on further discussion.

  - however, some companies still prefer to take Sub-use-case 1 into consideration, their main reasons are: I. Better prediction accuracy it can achieve with abundant previous instances information obtained from L1 filtered beam level RSRP (e.g., under the same OW length condition for FR1 intra-frequency temporal domain prediction case 2, non-sliding); II. Prediction error accumulation the L3 previous predictive results incurs.

  - regarding Sub-use-case 3, it can achieve better performance than other Sub-use-cases under certain conditions/AI models (e.g., TF-based predictor for FR1 inter-frequency prediction case 3) mentioned by minority.

- Impacts on beam level measurements

- Based on the following clarifications in TR 38.744,

## 5.2 RRM measurement prediction ### 5.2.1 Evaluation methodology, metrics and assumptions Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

it is necessary and reasonable to define L3 cell level RSRP prediction accuracy requirements and define L3 measurement prediction accuracy requiremente at point C

- Regarding whether to define accuracy requirement at point A1. First, based on our simulation results presented in RAN2, the necessity of L1 filtering can be observed and verified. Second, from our understanding, if L1 beam level prediction (Sub-use-case 1) is considered, it is better to define accuracy requirements at A1, since the L3 filtered RSRP will be derived based on measured L1 beam results and the predicted beam L1 results following cell-level metrics derivation and L3 filtering procedure defined in TS 38.331 Section 5.5. However, the predicted beam qualities are different from measured beam qualities, in this sense, the beam measurement prediction accuracy should be defined to ensure the AI model can learn the best and can provide the acceptable and accurate predictions directly. And to define the accuracy at A1 actually aligns with our traditional non-AI L3 RSRP requirement definition logic.

- However, we have some concerns that if we to define accuracy requirement at A1, whether it is necessary to define an additional prediction accuracy requirement [on top of the requirement at C]. Since: I.  Signalling/memory/measurement overhead and computational complexity for model training/inference; II.  Accuracy evaluation. The beam prediction accuracy requirement defined at A1 may be rigorous for cell-level prediction, which complies with beam management rather than system performance evaluation, in other words, L3 mobility performance including neighbout cell; III. For case B, the applicability of sliding and non-sliding; IV. RSRP difference interpretion, are different by comparing directly leveraging L3 to predict L3 and indirectly leveraging L1 to predict L3.

- Furthermore, regarding whether to define accuracy requirement at E, we suggest to deprioritize the case, since the model may need to learn the logic of the beam consolidation mechanism, which is different from the ones discussed for A1/C.

- FR2 to FR2 intra-frequency spatial domain

First, we admit that scenario 6 is an important scenario, but we prefer to start the discussions in terms of scenario 2, 3, 4 agreed in previous meeting, which aligns to the mainstream discussions in RAN2. And, the intra-cell spatial RRM prediction can be categorized as the extension of Rel-18 AI/ML BM, which it is already studied in R18 AI/ML Air SI. Besides, if spatial RRM prediction is considered, from our understanding at least the following issues would be paid attention on: I. How to derive cell quality combined with predicted beam qualities and measured beam qualities; II. The feasibility of defining the requirements including cell detectability, measurement, side conditions and accuracy based on different beam related observation/prediction data, which are intricate.

In accordance to the above analyzes, and to consistent with RAN2 simulation observations, we have the following proposals:

Proposal 1: RAN4 to consider the Sub-use-case 2 as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction

Proposal 2:

- RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

Proposal 3:

- If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

  - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

Proposal 4: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

2.2 Absolute and/or Relative RSRP Prediction Accuracy

Scenario 3

From our understanding, since FR1 intra-frequency intra-cell, FR1 inter-frequency inter-cell and FR2 intra-frequency intra-cell are all considered, from legacy measurement performance/test requirement perspective, both absolute and relative accuracy should be discussed but with AI mobility specific definitions of absolute and relative accuracy, notably, the ideal RSRP. Specifically,

<Legacy absolute/relative accuracy specified in TS 38.133> - FR1 intra-frequency intra-cell: The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1 - FR1 inter-frequency inter-cell: - The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell. - (relative accuracy) The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1. - FR2 intra-frequency intra-cell: The requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2. <Agreements in RAN2 and RAN4 on L3 RSRP prediction accuracy > - RAN2: Average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged - RAN4: Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

Back to scenario 3, inter-frequency cell (s) to measure and cell (s) to predict are co-located in the same sector. To accommodate the existing accuracy definition, since the cells are on different frequency layers, it is plausible to define relative RSRP accuracy requirement besides absolute RSRP accuracy requirement. And theoretically, the motivation of the test requirements is to make sure that the RSRP difference will be located within [lower bound, upper bound] to verify whether the relative L3 predicted RSRP accuracy requirements are fulfilled. However, how to define absolute and/or relative L3 predicted RSRP accuracy is more complicated, particularly in terms of reference/baseline measurement:

- Since the intention corresponding to scenarios 3 is to reduce the measurement overhead, especially the measurement gap needed for the measurements on other frequency bands, from our understanding, one of the references would be measure all time instances based on the MG configuration, not just difference of RSRP measured and RSRP predicted from different cells on different frequencies.

- Cell-specific and cluster (spatial cluster and frequency cluster) approaches are considered, the later one means multiple cells are used as input of the model, and the output are the predictions of the measurements for the corresponding cells. In this sense, two cells based definition considered for relative L3-RSRP accuracy requirement maybe is not enough.

Proposal 5: For scenario 3, to accommodate the existing accuracy definition, it is plausible to define relative RSRP accuracy requirement besides the absolute RSRP accuracy requirement

Observation 1: The intention of scenarios 3 is to reduce the measurement overhead, one of the references would be measure all time instances based on the MG configuration

Proposal 6: If to consider absolute and relative RSRP prediction accuracy for scenario 3, the accuracy definitions should be further discussed by taking at least the following into account:

- Reference/baseline measurements related to 1st goal (measurement overhead reduction)

- Difference of RSRP measured and RSRP predicted from different cells on different frequencies

- Cell-specific and/or cluster prediction approaches

2.3 Impacts of measurement error on prediction accuracy

Prediction accruacy requirement-General

Recalling that for measurement accuracy requirement, AWGN is typically specified as a side condition, and SNR side condition, number of samples, and measurement accuracy are linked together as a packet. For mobility measurements, 5 measurement samples are assumed for L3-RSRP, and e.g.,±4.5 dB RSRP intra-frequency absolute accuracy when side condition Ês/Iot = -6 dB for FR1. While for L1-RSRP measurements, the measurement accuracy is defined based on the single sample (R4-1902565). From this, from our understanding, similar issue should be discussed here, since for AI mobility prediction, we have sliding and non-sliding filterings and also consider side condition under the fast fading channel environment.

- For silding, new L3 filtered results are generated every sampling period

  - UE keeps measuring all the time. Each Qn+x can be computed by using the updated qn+t. The observation window and prediction window slide forward with sampling period (s) in case the UE does not necessarily perform inference in every sample period.

Fig.1 FR1 sliding filtering

  - It can predict L3-RSRPs on a small time scale and capture short term variation in channel conditions in the temporal domain

  - Another benefit: For the same OW length, there are more observed measurements compared to non-sliding case.

- For non-sliding, new L3 filtered results are generated every measurement period

  - The measurement period: Minimum granularity 200ms for FR1, 400 ms for FR2. While, in RRM requirments, the measurement period can encompass: total number of samples M (for FR2), CSSF, Kp, MGRP, DRX cycle, etc.

  - Certain amount of noise/fast fading fluctuation can be smoothed by using long filtering to the time series of L3-RSRP measurements, and the reflection of the real time channel variation, or the random variation of L3-RSRP measurments will be rather limited

  - The interval between two consecutive samples becomes larger, which leads to greater channel variation/decreased channel correlation between the two samples

Fig.2 FR1 non-sliding filtering (in RAN2 case)

Based on above analyzes, that’s why the prediction accuracy obtained based on sliding and non-sliding filtering cases are different from principle perspective. From this, we think the SNR side conditions or for the same SNR range, the improvement on prediction accuracy of the two filtering cases are different.

Observation 2: For temoral domain prediction, the prediction accuracy obtained based on sliding and non-sliding filtering cases are different under the same simulation assumptions

In this sense, from our understanding, under the purpose of L3 AI mobility prediction, how to define the prediction accuracy requirements for the proposed sub-use-case (s) and evaluation scenario (s) would be better to take which filtering case (i.e., sliding and/or non-sliding filtering) we use and the corresponding SNR side condition into consideration especially for FR2 considering the dynamic propagation environment.

Proposal 7: RAN4 to study accuracy requirements of predicted cell level RSRP for the proposed sub-use-case (s) and evaluation scenario (s), suggest to consider the following together

- Sliding and/or non-sliding filtering cases

- Total number of samples

- SNR Side conditions

Measurement error on prediction accuracy

- On the other hand, recalling that for measurement accuracy again, the RSRP accuracy containing measurement error, which is made up of RF calibration error and baseband measurement error, is specified in Section 10 as fixed limits (±X), and actually, the calibration errors in FR2 are different from that in FR1. Thereionto, the measurement error will have a distribution that can be assumed as gaussian distribution~(μ,σ2), and should be ensured 95% statistical confidence.

Observation 3: The existing RSRP accuracy contains measurement error, which is made up of RF calibration error and baseband measurement error. The RF calibaration on FR1 and FR2 may be different

- However, based on our observations and understandings, so far, all the RAN2 simulations are based on ideal conditions/ideal L3-RSRP. Considering the channel environment, the measurements taken of the channel, especially the fast fading in temporal domain, always contain a certain amount of error, which should be considered as one of the inputs to the AI/ML model. But how to model the error distribution and how to combine the error with filtering operation and PW/OW set for FR1, we think, is AI mobility specific issues since AI BM only covers FR2 (spatial domain predicition, spatial domain+temporal domain predicition) at the current stage. In this sense, for FR1 there is no guarantee that whether the error consideration added to the input measurements and ground truth in AI BM can be accounted in AI mobility before any clear conclusions achieved in such WI.

- Besides, from our understanding, to study measurement error impact on prediction accuracy performance is RAN4 issue, just like what AI/ML for BM did, some key methodolgies/conclusions, especially related to FR2, can wait and be inherited from AI/ML BM, while if necessary simulation analyzes are needed, suggest to make them in WI due to the limited TU in SI.

Proposal 8: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

- If additional simulations are needed, suggest to do the required simulation in WI phase

Proposal 9: Suggest to wait for more progress/evaluation outcomes in AI/ML BM (especially for FR2 temporal domain prediction) to study the impact of measurement error on the prediction accuracy requirements for AI mobility

2.4 Inter-frequency prediction scenario

For our understanding, for inter-frequency prediction, RAN4 can to study the impact on MG reduction in measurement period/scheduling restriction based on the measurement overhead reduction goal. Besides, since measurements of the co-located cell (s) can be reduced, the CSSF requirements can be enhanced.

Proposal 10: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

2.5 Configuration of observation and prediction windows

The OW and PW length directly affect the number of samples that can be used as AI/ML model input to observe and the number of samples that should to be predicted, which bring severe impact to accuracy performance, accordingly. For example in temporal domain prediction, if the OW length is too short, the simulation accuracy may be decreased since there are no enough samples/input date can be trained, while if the OW length is too large, the improvement in accuracy is not significant and may even decrease, since the RSRP measured long time ago has already meaningless due to the fast fading characteristic. Besides, based on simulation analysis and agreements, RAN2 found that under some cases (e.g., FR2 intra-frequency temporal domain case A), once the OW length exceeds a certain threshold, further increases do not yield significant benefits, while RAN2 will not define the actual threshold. At the same time, for inter-frequency prediction, the correlation between the two frequencies is considered, with the increase of OW, the prediction accuracy declines due to the reduced inter-frequency correlation (~the measurement samples on the serving frequency cell more distant in time). Furthermore, for case A, increasing PW length will slightly deteriorates the accuracy performance but can obtain the measurement reduction gain.

Proposal 11: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for temporal domain prediction case A

Furthermore, the prediction can be used to reduce the inter-frequency measurement the UE needs to take, so I. The measurement latency for the certain MO (s) can be reduced, since in the legacy, MG is used to perform RF tuning, and the simplest way to complete measurements of all the measurement objects in in turn; II. The data interruption time can be reduced and scheduling restriction can be relaxed, since in the legacy, during MGs, the UE can not receive/transmit data of the serving cell.

Proposal 12: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

3 Conclusion

In this contribution, we provided our initial viewpoints to General discussion on AI mobility general aspects and requirements impact. The following observations and proposals are obtained:

Observation 1: The intention of scenarios 3 is to reduce the measurement overhead, one of the references would be measure all time instances based on the MG configuration

Observation 2: For temoral domain prediction, the prediction accuracy obtained based on sliding and non-sliding filtering cases are different under the same simulation assumptions

Observation 3: The existing RSRP accuracy contains measurement error, which is made up of RF calibration error and baseband measurement error. The RF calibaration on FR1 and FR2 may be different

Proposal 1: RAN4 to consider the Sub-use-case 2 as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction

Proposal 2:

- RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

Proposal 3:

- If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

  - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

Proposal 4: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

Proposal 5: For scenario 3, to accommodate the existing accuracy definition, it is plausible to define relative RSRP accuracy requirement besides the absolute RSRP accuracy requirement

Proposal 6: If to consider absolute and relative RSRP prediction accuracy for scenario 3, the accuracy definitions should be further discussed by taking at least the following into account:

- Reference/baseline measurements related to 1st goal (measurement overhead reduction)

- Difference of RSRP measured and RSRP predicted from different cells on different frequencies

- Cell-specific and/or cluster prediction approaches

Proposal 7: RAN4 to study accuracy requirements of predicted cell level RSRP for the proposed sub-use-case (s) and evaluation scenario (s), suggest to consider the following together

- Sliding and/or non-sliding filtering cases

- Total number of samples

- SNR Side conditions

Proposal 8: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

- If additional simulations are needed, suggest to do the required simulation in WI phase

Proposal 9: Suggest to wait for more progress/evaluation outcomes in AI/ML BM (especially for FR2 temporal domain prediction) to study the impact of measurement error on the prediction accuracy requirements for AI mobility

Proposal 10: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

Proposal 11: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for temporal domain prediction case A

Proposal 12: For the RRM impact on prediction window, at least the following may need to be discussed

- Scheduling restriction

- Measurement latency for MO (s), MO (s) reduction

4 References

[1] R4-2420095 “WF on RRM requirements for FS_NR_AIML_Mob”, Nokia

Appendix

- The latest progresses for RRM measurement prediction & generalization are duplicated as below

<Agreements/Observations to be captured in TR> 1 The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. 2 For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance. 3 For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. 4 For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. 5 In cluster approach the model takes measurements from more than one cell as inputs. <Agreements/observations> 1. Companies can compare results with non-AI approaches. Temporal domain, sample and hold, and frequency domain, pathloss offset. Companies can consider other simple models (e.g. ARIMA). 2. For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold. 3. In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset. 4. For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed. <Agreements on generalization> 1. Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, • Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. • Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. • Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. 5 Study generalization over UE speeds 6 The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations 7 The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations <Agreements on generalization> 1 Between GC#1 and GC#2, RAN2 focus on GC#2 2 simulation combination for FR1 generalization study: Training @Dataset: 30km/h; Training @Dataset: 60km/h; Training @Dataset: 90km/h; Inference @30km/h; Inference @60km/h; Inference @90km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. 2a simulation combination for FR2 generalization study: Training @Dataset: 60km/h; Training @Dataset: 90km/h; Training @Dataset: 120km/h; Inference @60km/h; Inference @90km/h; Inference @120km/h. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes. Baseline; Yes; Yes. GC#1; Yes; Yes; Yes. GC#2; Yes; Yes; Yes; Yes.. <Further agreements on generalization> 1. Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz. FFS if we introduce a third frequency. 2. Study model generalization across different cell configurations (e.g. ISD, gNB height, power, beam pattern, etc). FFS which parameters we prioritize.