3GPP TSG-RAN WG4 Meeting #114                                        R4-2501678

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              Discussion on L3 measurement enhancement by optimizing Rx BSF

Agenda item:			7.17.2.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had extensively discussions on L3 measurement enhancement by optimizing Rx BSF. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

As an optional function, it is not necessary and expected for UE to perform RRM measurement using FBS continuously. For example, in some cases, there is no need to receive signals from all panels simultaneously for saving power. In this sense, mechanism to quit BFS based L3 measurement is also needed. From our understanding, the approaches could be:

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication

- Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement, e.g., after reporting the valid measurement results

Observation 1: Suggest to discuss the mechanism to quit BFS based L3 measurement, the Alt.s can be

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication; Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement, e.g., after reporting the valid measurement results

Based on the discussion happened in last meeting, we prefer that UE fall back to normal measurement mode after UE has reported qualified measurement results. Since from our understanding, UE in wireless communication system needs to monitor the channel quality not only for serving cell, but also for some neighbor cell. That is to say, although UE is doing business with serving cell, it also needs to find some candidate neighbor cell and monitor them to shorten traffic interruption when serving cell quality becomes poor. Such thing may happen at cell edge. Fast L3 measurement can expedite neighbor cell related action, avoid UE suffering from long interruption due to neighbor cell search/measurement latency. And if UE has reported valid measurement results, that can mean the HO expectation is realized, and can fallback to single panel. Besides, in some cases, e.g., UE already moves to cell center, there is no need to receive signals by using multiple panel simultaneously for saving power.

Proposal 1: Apart from the threshold conditions, the UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results.

2.2 Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

From our understanding, the legacy Rx beam sweeping procedure assumes that UE does not move or rotate significantly/quickly from the start of the beam sweep until it is complete, otherwise, the worst case is that the UE reports false beams, which were the best ones when the measurement was performed, but become rather weaker when the measurement period is complete, and the result is reported. In this sense, one of the benefits on faster beam sweeping in our case is that short delay in cell detection and measurement will shorten the difference between valid RSRP measurement report and actual condition when the report is provided, which can improve the system performance.

However, there is a tradeoff between mobility performance and number of samples to be measured

- The L3 delay requirements are based on total number of required samples for measurement M. To be specific, the M is proportional to the number of samples and RX beam sweeping factor N (i.e., 24= 3 × 8 ).

- In the previous release, there were two candidate values for the N, that is N=4 and N=8. The N=8 was finally compromised by taking mobility performance verification based on SLS and UE antenna implementation (spherical coverage) into consideration. Actually, from our understanding, from system performance point of view only, the N=4 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies.

- If the UE is able to simultaneously sweep K beam directions at one time instance, the multi-Rx capable and available UE need Nbeam/K measurement samples for scanning once over all the Rx beam directions. The BSF due to Rx beam sweeping operations equals to Nbeam/K. For K, here we can assume, e.g., K=2, for 2 Rx chains

- Besides, if we focus on N only, we have some concerns on the number of samples, especially for N=6, since in theory, if N is reduced, more samples are needed to get an accurate measurement, we can use sample=4 at least, or keep the sample=3. While, there is no change when sample=4 since M still =24.

If N/measured sample is reduced, the performance degradation issue may unavoidably arise. While, since no SLS results were obtained based on the new sweeping behaviour, we cant observe how much performance loss the UE will experience with different number of Rx beam sweeping, and cant conclude how to reduce the N, i.e, the total number of required samples for measurement accordingly under the new scenario. But we prefer to use N=4, the corresponding total samples is 20= 5 × 4 as baseline, at least one of the candidate values to define the delay requirements of inter-/intra-frequency measurement for Rel-19 L3 multi-Rx measurement, since as we mentioned before, the performance based on N=4 can be accepted by the companies in the previous release.

Proposal 2: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 multi-Rx measurement. The following observations and proposals are obtained:

Observation 1: Suggest to discuss the mechanism to quit BFS based L3 measurement, the Alt.s can be

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication; Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement, e.g., after reporting the valid measurement results

Proposal 1: Apart from the threshold conditions, the UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results.

Proposal 2: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

4 References

[1] R4-2420093 “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple