3GPP TSG-RAN4 Meeting #114	R4-2501685

# Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Duty cycle solutions and MSD rules for HPUE UL CA and EN-DC

Agenda item:	7.1.1.1.1

Document for:	Discussion

Introduction

In this paper, we provide our views on duty cycle solutions and MSD rules for HPUE UL CA and EN-DC.

Discussion (duty cycle solutions)

In last RAN4 meeting, the following WF was approved [1].

4.2 Duty cycle solution for SAR compliance - Introduction of new duty-cycle IE is not considered - For ENDC (FDD+FDD), only reply on P-MPR, and there is no further spec changes needed - Following as starting point - For PC1.5 of intra-band contiguous and non-contiguous CA: for sub-clause 6.2A.1.1 and 6.2A.1.2 of 38.101-1, refer to R4-2419448 - For PC1.5 of inter-band CA: for sub-clause 6.2A.1.3 of 38.101-1, refer to R4-2419448 - For EN-DC (FDD+TDD) for sub-clause 6.2B.1.3 of 38.101-3 refer to R4-2418744 - For EN-DC(TDD+TDD) for sub-clause 6.2B.1.3 of 38.101-3 (similar as R4-2418744)

PC1.5 intra-band contiguous and non-contiguous CA

The duty cycle solution in R4-2419448 can be supported with some modification (yellow highlighted below) for PC1.5 intra-band contiguous and non-contiguous CA.

Proposal 1: For PC1.5 NR intra-band contiguous and non-congituous CA, consider the below uplink duty cycle solution.

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1/2 and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 23dBm or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 25% but less than or equal to 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 but less than or equal to maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than or equal to 2*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 26dBm or lower; - shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2.

PC1.5 UE for NR inter-band uplink CA with 2Tx and/or 3Tx

Table 2.2-1 shows list of HPUE NR inter-band CA.

Table 2.2-1: List of HPUE NR inter-band CA

|  | Number of Tx | NR | NR | Whether to support or not in Rel-19 |
| --- | --- | --- | --- | --- |
| PC1.5 CA | 2Tx/3Tx | PC2 TDD (1Tx) | PC2 TDD (1Tx/2Tx) | O |
|  | 3Tx | PC3 TDD (1Tx) | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC2 TDD (1Tx) | PC1.5 TDD (2Tx) | O |

The existing uplink duty cycle solution in 6.2H.3 for PC1.5 inter-band UL CA with UL-MIMO covers the following case.

  - PC3 FDD + PC1.5 TDD

It can also apply to PC3 TDD + PC1.5 TDD if this combination is introduced in Rel-19.

  - PC3 TDD + PC1.5 TDD

    - Need to reuse the existing requirement in 6.2H.3

However, the existing uplink duty cycle solution needs to be modified to cover the combinations, such as, PC2 TDD + PC2 TDD, and PC2 TDD + PC1.5 TDD if these are introduced in Rel-19.

  - PC2 TDD + PC2 TDD

    - Need to modify the existing requirement in 6.2H.3

  - PC2 TDD + PC1.5 TDD

    - Need to modify the existing requirement in 6.2H.3

Proposal 2: For PC1.5 NR inter-band CA, consider the below uplink duty cycle solution which is modified from 6.2H.3.1.

If a UE supports power class 2 for the band combination listed in Table 6.2A.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%. If a UE supports power class 1.5 for the band combination listed in Table 6.2A.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one band within the band combination is power class 1.5 – if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%; – else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported; – the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability; – else – the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1; – if power class of one or both of the bands within the band combination is power class 2 – if the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is reported; – the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability; – if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

PC1.5 and PC2 for EN-DC with 2Tx and/or 3Tx

Table 2.3-1 shows list of HPUE PC1.5 and PC2 EN-DC.

Table 2.3-1: List of HPUE EN-DC

|  | Number of Tx | E-UTRA | NR | Whether to support or not in Rel-19 |
| --- | --- | --- | --- | --- |
| PC2 EN-DC | 2Tx/3Tx | PC3 FDD (1Tx) | PC3 FDD (1Tx/2Tx) | O |
|  | 2Tx/3Tx | PC2 TDD (1Tx) | PC3 TDD (1Tx/2Tx) | O |
| PC1.5 EN-DC | 2Tx/3Tx | PC2 TDD (1Tx) | PC2 TDD (1Tx/2Tx) | O |
|  | 3Tx | PC3 FDD | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC3 TDD | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC2 TDD | PC1.5 TDD (2Tx) | O |

The existing uplink duty cycle solution covers the following cases with UE capability.

- PC3 E-UTRA FDD + PC3/PC2 NR TDD

  - maxUplinkDutyCycle-FDD-TDD-EN-DC1

  - maxUplinkDutyCycle-FDD-TDD-EN-DC2

- PC3 E-UTRA TDD + PC3/PC2 NR TDD

  - maxUplinkDutyCycle-interBandENDC-TDD-PC2

RAN4 agreed that duty cycle solution for SAR will discussed for the scenario without new IE introduction.

For PC2 EN-DC,

- PC3 E-UTRA FDD + PC3 NR FDD

  - It can be out of scope based on the scenario without new IE introduction. Because, the existing IEs cannot support it.

- PC2 E-UTRA TDD + PC3 NR TDD

  - Need to modify the existing requirement in 6.2B.1.3 of TS 38.101-3 because it is based on PC3 E-UTRA TDD. For PC2 E-UTRA TDD, it needs to be modified with;

  - maxUplinkDutyCycle-interBandENDC-TDD-PC2

For PC1.5 EN-DC,

There is no the existing uplink duty cycle solution, so it is necessary to be defined.

The following cases need to be included in uplink duty cycle solution.

- PC3 E-UTRA TDD + PC1.5 NR TDD

  - maxUplinkDutyCycle-interBandENDC-TDD-PC2

- PC2 E-UTRA TDD + PC1.5 NR TDD

  - maxUplinkDutyCycle-interBandENDC-TDD-PC2

- PC2 E-UTRA TDD + PC2 NR TDD

  - maxUplinkDutyCycle-interBandENDC-TDD-PC2

- PC3 E-UTRA FDD + PC1.5 NR TDD

  - maxUplinkDutyCycle-FDD-TDD-EN-DC1

  - maxUplinkDutyCycle-FDD-TDD-EN-DC2

For PC2/PC1.5 EN-DC(TDD+TDD),

For PC2 E-UTRA TDD + PC3 NR TDD, the percentage of maximum E-UTRA/NR uplink transmission is different from the existing PC3 E-UTRA TDD + PC3 NR TDD as seen in Table 2.1 and Table 2.2.

Table 2.1 shows the percentage of maximum E-UTRA/NR uplink transmission for PC2 E-UTRA TDD + PC3 NR TDD.

Table 2.1: Percentage of maximum E-UTRA/NR uplink transmission in PC2 E-UTRA TDD + PC3 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (23dBm) |  |  |
|  | Ymax | Zmax | Z <= D | Z>D |
| 0 | 70 | N/A | PC2 | PC3 |
| 1 | 50 | N/A |  |  |
| 2 | 30 | 40 |  |  |
| 3 | 40 | 20 |  |  |
| 4 | 30 | 40 |  |  |
| 5 | 20 | 60 |  |  |
| 6 | 60 | N/A |  |  |

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

  - D = maxUplinkDutyCycle-interBandENDC-TDD-PC2

maxUplinkDutyCycle-interBandENDC-TDD-PC2 = 20/40/50/60/70/80/90/100, default = 30

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance. A minimum percentage of 20% for maximum NR uplink transmission during evaluation period can be assumed as threshold when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

For example,

–	if threshold(D) is indicated

Z <= D,  PC2 applies

Z > D,    PC3 applies

–	else,

Z <= 20, PC2 applies

Z > 20,   PC3 applies

For PC3 E-UTRA TDD + PC3 NR TDD

Table 2.2 shows the percentage of maximum E-UTRA/NR uplink transmission for the existing PC3 E-UTRA TDD + PC3 NR TDD. Threshold of 30% was specified with the minimum percentage of maximum NR uplink transmission for PC3 E-UTRA TDD+PC3 NR TDD when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

Table 2.2: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA TDD + PC3 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (23dBm) |  |  |
|  | Ymax | Zmax | Z <= D | Z>D |
| 0 | 70 | 30 | PC2 | PC3 |
| 1 | 50 | 50 |  |  |
| 2 | 30 | 70 |  |  |
| 3 | 40 | 60 |  |  |
| 4 | 30 | 70 |  |  |
| 5 | 20 | 80 |  |  |
| 6 | 60 | 40 |  |  |

For example,

–	if threshold(D) is indicated

Z <= D,      PC2 applies

Z > D,         PC3 applies

–	else,

Z <= 30,      PC2 applies

Z > 30,        PC3 applies

For PC3 E-UTRA TDD + PC2 NR TDD

Table 2.3 shows the percentage of maximum E-UTRA/NR uplink transmission for the existing PC3 E-UTRA TDD + PC2 NR TDD. Threshold of 30% was specified with the minimum percentage of maximum NR uplink transmission for PC3 E-UTRA TDD+PC2 NR TDD when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. However, it needs to be modified with 15% as seen in Table 2.3.

Table 2.3: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA TDD + PC2 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (26dBm) |  |  |
|  | Ymax | Zmax | Z <= D | Z>D |
| 0 | 70 | 15 | PC2 | PC3 |
| 1 | 50 | 25 |  |  |
| 2 | 30 | 35 |  |  |
| 3 | 40 | 30 |  |  |
| 4 | 30 | 35 |  |  |
| 5 | 20 | 40 |  |  |
| 6 | 60 | 20 |  |  |

For example,

–	if threshold(D) is indicated

Z <= D,      PC2 applies

Z > D,         PC3 applies

–	else,

Z <= 15,      PC2 applies

Z > 15,        PC3 applies

For PC3 E-UTRA TDD + PC1.5 NR TDD,

Table 2.4 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.4: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA TDD + PC1.5 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (29dBm) |  |  |  |  |
|  | Ymax | Zmax |  | Z > D | D/2<=Z<=D | Z < D/2 |
| 0 | 70 | 7.5 |  | N/A | N/A | N/A |
| 1 | 50 | 12.5 |  | N/A | N/A | N/A |
| 2 | 30 | 17.5 |  | PC3 | PC2 | N/A |
| 3 | 40 | 15 |  | PC3 | PC2 | N/A |
| 4 | 30 | 17.5 |  | PC3 | PC2 | N/A |
| 5 | 20 | 20 |  | PC3 | PC2 | PC1.5 |
| 6 | 60 | 10 |  | N/A | N/A | N/A |

For example,

–	if threshold(D) is indicated

Z < D/2,               PC1.5 applies

D/2 <=Z <= D,     PC2 applies

Z > D,                   PC3 applies

–	else,

Z <= 7.5,                PC2 applies

Z > 7.5,                  PC3 applies

For PC2 E-UTRA TDD + PC1.5 NR TDD,

Table 2.5 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying SAR solution.

Table 2.5: Percentage of maximum E-UTRA/NR uplink transmission in PC2 E-UTRA TDD + PC1.5 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (29dBm) |  |  |  |  |
|  | Ymax | Zmax |  | Z > D | D/2<=Z<=D | Z < D/2 |
| 0 | 70 | N/A |  | N/A | N/A | N/A |
| 1 | 50 | N/A |  | N/A | N/A | N/A |
| 2 | 30 | 10 |  | PC3 | PC2 | N/A |
| 3 | 40 | 5 |  | PC3 | PC2 | N/A |
| 4 | 30 | 10 |  | PC3 | PC2 | N/A |
| 5 | 20 | 15 |  | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A |  | N/A | N/A | N/A |

For example,

–	if threshold(D) is indicated

Z < D/2,              PC1.5 applies

D/2 <=Z <= D,   PC2 applies

Z > D,                 PC3 applies

–	else,

Z <= 5,                PC2 applies

Z > 5,                  PC3 applies

For PC2 E-UTRA TDD + PC2 NR TDD,

Table 2.6 shows this example for the percentage of maximum E-UTRA/NR UL transmission.

Table 2.6: Percentage of maximum E-UTRA/NR uplink transmission in PC2 E-UTRA TDD + PC2 NR TDD

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) |  |  |  |  |
|  | Ymax | Zmax |  | Z > D | D/2<=Z<=D | Z < D/2 |
| 0 | 70 | N/A |  | N/A | N/A | N/A |
| 1 | 50 | N/A |  | N/A | N/A | N/A |
| 2 | 30 | 20 |  | PC3 | PC2 | N/A |
| 3 | 40 | 10 |  | PC3 | PC2 | N/A |
| 4 | 30 | 20 |  | PC3 | PC2 | N/A |
| 5 | 20 | 30 |  | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A |  | N/A | N/A | N/A |

For example,

–	if threshold(D) is indicated

Z < D/2,                PC1.5 applies

D/2 <=Z <= D,     PC2 applies

Z > D,                   PC3 applies

–	else,

Z <= 10,                PC2 applies

Z > 10,                  PC3 applies

Table 2.7 shows the summary of minimum percentage of maximum NR uplink transmission for above cases.

Table 2.7: Summary of minimum percentage of maximum NR uplink transmission for PC2/PC1.5 EN-DC

| EUTRA\NR | Min.% of NR uplink Tx |  |  |
| --- | --- | --- | --- |
|  | PC3 | PC2 | PC1.5 |
| PC3 | 30 | 15 | 7.5 |
| PC2 | 20 | 10 | 5 |

Based on the summary, the existing value of 30% and maxUplinkDutyCycle-interBandENDC-TDD-PC2 can be scaled with K for above cases as Table 2.8.

Table 2.8: K for PC2/PC1.5 EN-DC

| EUTRA\NR | K |  |  |
| --- | --- | --- | --- |
|  | PC3 | PC2 | PC1.5 |
| PC3 | 1 | 1/2 | 1/4 |
| PC2 | 2/3 | 1/3 | 1/6 |

Proposal 3: For PC2 & PC1.5 EN-DC with E-UTRA TDD and NR TDD, consider the below uplink duty cycle solution.

If a UE supports power class 2 a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination listed in Table 6.2B.1.3-1and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to K*30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the supported power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. If a UE supports power class 1.5 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination listed in Table 6.2B.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 but less than or equal to K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*K*30% but less than or equal to K*30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – Else – shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2B.4. Here, K is defined in Table 6.2B.1.3-2. Table 6.2B.1.3-2: K for inter-band EN-DC (TDD-TDD) EUTRA\NR; K. PC3; PC2; PC1.5. PC3; 1; 1/2; 1/4. PC2; 2/3; 1/3; 1/6.

For PC1.5 EN-DC(FDD+TDD),

For PC3 E-UTRA FDD + PC1.5 NR TDD, the percentage of maximum E-UTRA/NR uplink transmission is different from the existing PC3 E-UTRA FDD + PC3/PC2 NR TDD as seen in Table 2.9, Table 2.10 and Table 2.11.

Table 2.9 shows the percentage of maximum E-UTRA/NR uplink transmission for PC3 E-UTRA FDD + PC1.5 NR TDD.

Table 2.9: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA FDD + PC1.5 NR TDD

| Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | 2 thresholds(DC1 and DC2) (Yes/No – indicated?) DC1 = maxUplinkDutyCycle-FDD-TDD-EN-DC1 DC2 = maxUplinkDutyCycle-FDD-TDD-EN-DC2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA (23dBm) | NR (29dBm) | Yes (DC1) |  |  | Yes (DC2) |  |  | No |
| Y | Z | Z<= DC1/2 | DC1/2<Z<= DC1 | Z>DC1 | Z<= DC2/2 | DC2/2 <Z<= DC2 | Z>DC2 |  |
| Y<=40 | Z=0~25 |  |  |  | PC1.5 | PC2 | PC3 | PC1.5 |
| 40<Y<70 | Z=0~25 | PC1.5 | PC2 | PC3 |  |  |  |  |
| 70<=Y | Z=0~25 | PC3 |  |  |  |  |  |  |

For example,

–	if two thresholds(DC1 and DC2) is indicated

–	if Y<=40

Z <= DC2/2,              PC1.5 applies

DC2/2 < Z <= DC2,   PC2 applies

Z > DC2,                    PC3 applies

–	else if 40<Y<70

Z <= DC1/2,               PC1.5 applies

DC1/2 < Z <= DC1,    PC2 applies

Z > DC1,                     PC3 applies

–	else (Y>=70)

PC3 applies

–	else,

PC1.5 applies

Table 2.10 shows the percentage of maximum E-UTRA/NR uplink transmission for the existing PC3 E-UTRA FDD + PC3 NR TDD.

Table 2.10: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA FDD + PC3 NR TDD

| Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | 2 thresholds(DC1 and DC2) (Yes/No – indicated?) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA (23dBm) | NR (23dBm) | Yes |  |  |  | No |
| Y | Z | Z<= DC1 | Z>DC1 | Z<= DC2 | Z>DC2 |  |
| Y<=40 | Z=0~100 |  |  | PC2 | PC3 | PC2 |
| 40<Y<70 | Z=0~60 | PC2 | PC3 |  |  |  |
| 70<=Y | Z=0~30 | PC3 |  |  |  |  |

Table 2.11 shows the percentage of maximum E-UTRA/NR uplink transmission for the existing PC3 E-UTRA FDD + PC2 NR TDD.

Table 2.11: Percentage of maximum E-UTRA/NR uplink transmission in PC3 E-UTRA FDD + PC2 NR TDD

| Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | 2 thresholds(DC1 and DC2) (Yes/No – indicated?) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA (23dBm) | NR (26dBm) | Yes |  |  |  | No |
| Y | Z | Z<= DC1 | Z>DC1 | Z<= DC2 | Z>DC2 |  |
| Y<=40 | Z=0~50 |  |  | PC2 | PC3 | PC2 |
| 40<Y<70 | Z=0~30 | PC2 | PC3 |  |  |  |
| 70<=Y | Z=0~15 | PC3 |  |  |  |  |

For PC3 E-UTRA FDD + PC1.5 NR TDD, the uplink duty cycle solution is proposed as follows.

Proposal 4: For PC2 & PC1.5 EN-DC with E-UTRA FDD and NR TDD, consider the below uplink duty cycle solution.

If a UE supports power class 2 a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination listed in Table 6.2B.1.3-1 and the supported power class enables higher maximum output power than that of the default power class: If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2: – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the supported power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else – shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4; else – shall apply all requirements for the supported power class 2 and set the configured transmitted power as specified sub-clause 6.2B.4; If a UE supports power class 1.5 for an E-UTRA FDD and NR TDD EN-DC band combination listed in Table 6.2B.1.3-1: If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2: – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1 but less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 but less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else – shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4; else – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified sub-clause 6.2B.4;

Discussion (MSD rules)

1UL HPUE MSD simplification

Table 3.1 WF on 1UL HPUE MSD simplification [2]

Background: Figure 1 summarises the proposed 1UL HPUE (PC2 and PC1.5) delta MSD curves vs. the legacy Rel-18 MSD test points (“+” dots). Figure 1 left shows that three PC2 rules were presented at this meeting: - A single 3dB PC2 de-rating from the agreed PC3 MSD (Fig.1 left, light blue line [2]), - A single PC2 look-up table with plateau at ~ 6dB (Fig.1 left, orange curve [3]), - One PC2 LUT to specify PC2 1Tx MSD, another PC2 LUT to specify the PC2 2Tx MSD (Fig.1 left, dark blue and green curves [4,5]). These curves assume 9dB 1Tx PC3 interference imbalance. Figure 1: Agreed Rel-18 MSD test points plotted with “+”. Proposed PC2 1Tx/2Tx (left), PC2 1.5 (2Tx - right) [2,3,4,5]. It can be observed that at the end of Rel-18, the 1UL PC1.5 MSD may be smaller than the 1UL PC2 2Tx MSD [6]. This inconsistency may be accounted for different assumptions on the 1Tx PC3 interference Main/Diversity imbalance when the 1UL PC1.5 MSD were specified in Rel-17 vs the assumptions used in Rel-18 for 1UL PC2 2Tx MSD. Reminder on 1Tx PC3 interference imbalance: The 1Tx Main/Div interference is the level of interference which corresponds to the agreed MRC combined PC3 MSD level [6]. In most TP for TRs, 10dB antenna to antenna isolation results in 9dB Main/Div interference imbalance. An example of PC2 Tx MSD calculation based on the 1Tx PC3 interference level is reproduced from [6] in Figure 2. Figure 2: Example of PC2 1Tx / 2Tx MSD evaluation assuming 1Tx PC3 9dB interference imbalance for CA_n25-n77 [6]. Agreement: - For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, companies are encouraged to provide MSD simplification rules whereby the 1UL HPUE MSD is specified using a delta MSD relative to the default power class 3 MSD as: PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5, and PC2/PC1.5 may be specified with equations, look-up tables (LUTs), or a single 3dB delta MSD de-rating value. - Regarding the PC2 MSD simplification, companies are invited to evaluate three options, - option 1: RAN4 specifies using a single 3dB PC2 de-rating from the agreed PC3 MSD [2], - option 2: RAN4 keeps on specifying two PC2 requirements [4,5]: - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB, - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB, - option 3: RAN4 specifies a single PC2 LUT/equation [3]: - PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB. - To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations/fixed derating values assuming a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded. - For proponents of LUT simplification approach, FFS the preferred LUT granularity (e.g. 1dB steps)

When we look at the difference between the HPUE MSD equation and the default MSD equation in 1UL case (harmonic, harmonic mixing, and cross-band isolation), since the characteristics are the same, all passive element terms can be eliminated. As a result, what remains is only the interference components generated by the non-linear elements. Therefore, even if we do not know all the various parameters used to define the default MSD, it is not a problem to use the simplified HPUE MSD equation.

Proposal 5: Agree on using the simplified equation for HPUE MSD

- PCx MSD = PC3 MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

In the current spec for PC2 MSD, it is defined separately for cases that support 2Tx diversity and those that do not. The case that supports 2Tx diversity has an MSD that is approximately 6 dB higher than the case that does not support it. It is necessary to maintain this distinction and apply it to the simplified equation, defining different  MSDs for each case.

Proposal 6: Support option 2

- Option 2: RAN4 keeps on specifying two PC2 requirements:

  - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB,

  - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB,

The  MSD value for PC2 tends to saturate at 3 dB for case that do not support 2Tx diversity and at 9 dB for case that do support 2Tx diversity, so it is necessary to define them as 3 dB and 9 dB, respectively. The  MSD value for PC1.5 tends to saturate at 6 dB, so it should be defined as 6 dB. Additionally, it is not necessary to ensure that the MSD value of PC1.5 is greater than that of PC2, this is due to structural reasons related to whether the PC2 terminal supports 2Tx diversity. Furthermore, there has been no agreement in the current specification that the MSD value of PC1.5 must be greater than the PC2 MSD value that supports 2Tx diversity.

Proposal 7: Consider PCx values

- PC21Tx ≦ 3 dB

- PC22Tx ≦ 9 dB

- PC1.51Tx ≦ 6 dB

The value of  MSD can vary depending on the size of the default MSD value. For example, in the case of PC1.5, if the default MSD value is sufficiently large, the  MSD value is represented as 6 dB. However, if the default MSD value is small, it may have a value smaller than 6 dB. This is because when the default interference is closer to REFSENS, the degradation of REFSENS is affected not only by a specific noise but also by other additional noise influences simulataneously. As a result, the effects of specific noises (harmonic, harmonic mixing, and cross –band isolation) tend to be relatively less pronounced. Reflecting this characteristic, it is necessary to define the value of  MSD based on the default MSD value rather than using a fixed  MSD.

Proposal 8: It is necessary to specify different  MSD according to the default MSD value

2UL HPUE MSD simplification

Table 2 WF on 2UL HPUE MSD simplification [1]

Background: Up to ten different UL-CA (2UL) power class configurations have been identified for Rel-19 [7]. case #; 1 Baseline; 2; 3; 4; 5; 6; 7; 8; 9; 10. ULCA power class; Type; PC3 2Tx; PC2 2Tx; PC2 2Tx; PC2 3Tx; PC2 2Tx; PC2 3Tx; PC1.5 2Tx; PC1.5 3Tx; PC1.5 3Tx; PC1.5 3Tx. PCmax; 23; 26; 26; 26; 26; 26; 29; 29; 29; 29. per band power class; Bd1; PC3 1Tx; PC3 1Tx; PC3 1Tx; PC3 1Tx; PC2 1TX; PC2 1TX; PC2 1TX; PC2 1TX; PC3 1Tx; PC2 1TX. Bd2; PC3 1Tx; PC3 1Tx; PC2 1Tx; PC2 2Tx; PC2 1Tx; PC2 2Tx; PC2 1Tx; PC2 2Tx; PC1.5 2Tx; PC1.5 2Tx. Valid Release; R15; R17; R17; R18; R19; R19; R19; R19; R18; R19. Agreement: - Companies are invited to - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD, - Study means to minimize the number of LUTs / set of equations that cover all cases. - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7]. - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for.

For 2UL, it seems there is no problem using a simplified equation similar to that of 1UL for similar reasons.

Proposal 9: Consider similar HPUE MSD equation with 1UL case

- HPUE MSD(PC2, PC1.5) = Default MSD +  MSDpc2IMDX or 1.5IMDX

Due to the characteristics of intermodulation distortion caused by 2 UL, it is necessary to define different  MSD values for each intermodulation order. Below is a simplified equation that illustrates the process of calculating the  MSD value.

- IMD X frequency(X=IMD order) = |n*flow ± m*fhigh| (n+m = X)

- IMD X(X=IMD order) power (dBm) = n*Plow + m*Phigh – (X-1)*IPX(X=IMD order)

- MSDpc2IMDX or 1.5IMDX = default IMD power – HPUE IMD power

- Default IMD power = n*Plow + m*Phigh – (X-1)*IPXPC3(X=IMD order)

- HPUE IMD power = n*(Plow + Increased power(low))+ m*(Phigh + Increased power(high)) – (X-1)*IPXPC2orPC1.5(X=IMD order)

- MSD = (n+m = IMD order)*(Total increased power) - (IMD order -1)*(IPXPC2orPC1.5 - IPXPC3)

If assume the IPXPC3 = IPXPC2orPC1.5,

- MSD = (n+m = IMD order)*(Total increased power)

Observation 1: In the equation above, it aligns with the previous proposals from skyworks[2] and murata[3]. The ΔMSD value varies depending on the IMD order, but it can be seen that there is no difference based on the IMD index within the same IMD order. Therefore, the IMD index does not need to be taken into account.

Proposal 10: It is necessary to consider simple equation for deriving MSD value and further discussion on details is needed.

Proposal 11: It is not necessary to consider the IMD index for the  MSD value.

HPUE 2UL power class configuration may have 9 cases when considering up to Rel-19, as presented in the last meeting with WF, and this number could potentially increase in the future. Therefore, it is important to determine how to simplify the cases in order to increase the complexity of HPUE 2UL MSD simplification. It would be beneficial to cover not only the power class configurations that have been defined so far but also any future power class configurations that may be added. Considering these factors, simplifying the cases based on the PCmax standard rather than taking into account all power class configurations and their individual characteristics may be more beneficial from a simplification perspective.

Proposal 12: Consider simplifying power class configuration as shown in the table below

| case # |  | 1 Baseline | 2 | 3 |
| --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 | PC2 | PC1.5 |
|  | PCmax | 23 | 26 | 29 |

HPUE MSD simplification without default MSD

The discussion on HPUE MSD simplification has been conducted in cases where a default MSD exists. However, due to the characteristics of HPUE, as the aggressor power increases, interferes that did not affect REFSENS under the default power class may impact REFSENS in HPUE case. This case is illustrated in the figure 1. Therefore, it is necessary to consider how to apply HPUE MSD simplification in cases where a default MSD does not exist.

Figure 3.1 Example case on the HPUE MSD without the default MSD

Observation 2: The simplest way to address this issue is to assume a specific value in the absence of a default MSD. For example, if we assume this value to be 0, the HPUE MSD could be represented as 0 + delta MSD. However, the actual default MSD value can vary depending on the band combination, so a single default MSD may not be sufficient. If we were to define a default MSD considering all band combinations, it would lead to the definition of too many default MSDs, increasing complexity.

Proposal 13: It is necessary to study how to define the default MSD value in the absence of a default MSD for the HPUE MSD simplification.

Conclusion

In this contribution, we proposed our view on the duty cycle solutions and MSD rules for HPUE  inter-band CA and EN-DC. The proposals are as follows.

Duty cycle solutions

For PC1.5 UE supporting NR intra-band contiguous and non-contiguous CA,

Proposal 1: For PC1.5 NR intra-band contiguous and non-congituous CA, consider the below uplink duty cycle solution.

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1/2 and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 23dBm or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 25% but less than or equal to 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 but less than or equal to maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than or equal to 2*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 26dBm or lower; - shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2.

For PC1.5 UE supporting NR inter-band CA,

Proposal 2: For PC1.5 NR inter-band CA, consider the below uplink duty cycle solution which is modified from 6.2H.3.1.

If a UE supports power class 2 for the band combination listed in Table 6.2A.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%. If a UE supports power class 1.5 for the band combination listed in Table 6.2A.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one band within the band combination is power class 1.5 – if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%; – else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported; – the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability; – else – the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1; – if power class of one or both of the bands within the band combination is power class 2 – if the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is reported; – the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability; – if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

For PC2/PC1.5 UE supporting EN-DC(TDD-TDD),

Proposal 3: For PC2 & PC1.5 EN-DC with E-UTRA TDD and NR TDD, consider the below uplink duty cycle solution.

If a UE supports power class 2 a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination listed in Table 6.2B.1.3-1and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to K*30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the supported power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. If a UE supports power class 1.5 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination listed in Table 6.2B.1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 but less than or equal to K*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*K*30% but less than or equal to K*30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – Else – shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2B.4. Here, K is defined in Table 6.2B.1.3-2. Table 6.2B.1.3-2: K for inter-band EN-DC (TDD-TDD) EUTRA\NR; K. PC3; PC2; PC1.5. PC3; 1; 1/2; 1/4. PC2; 2/3; 1/3; 1/6.

For PC2/PC1.5 UE supporting EN-DC(FDD-TDD),

Proposal 4: For PC2 & PC1.5 EN-DC with E-UTRA FDD and NR TDD, consider the below uplink duty cycle solution.

If a UE supports power class 2 a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination listed in Table 6.2B.1.3-1 and the supported power class enables higher maximum output power than that of the default power class: If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2: – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the supported power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else – shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4; else – shall apply all requirements for the supported power class 2 and set the configured transmitted power as specified sub-clause 6.2B.4; If a UE supports power class 1.5 for an E-UTRA FDD and NR TDD EN-DC band combination listed in Table 6.2B.1.3-1: If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2: – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1 but less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 but less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) – shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4. – else – shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4; else – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified sub-clause 6.2B.4;

MSD rules

Observation 1: In the equation above, it aligns with the previous proposals from skyworks[2] and murata[3]. The ΔMSD value varies depending on the IMD order, but it can be seen that there is no difference based on the IMD index within the same IMD order. Therefore, the IMD index does not need to be taken into account.

Observation 2: The simplest way to address this issue is to assume a specific value in the absence of a default MSD. For example, if we assume this value to be 0, the HPUE MSD could be represented as 0 + delta MSD. However, the actual default MSD value can vary depending on the band combination, so a single default MSD may not be sufficient. If we were to define a default MSD considering all band combinations, it would lead to the definition of too many default MSDs, increasing complexity.

Proposal 5: Agree on using the simplified equation for HPUE MSD

- PCx MSD = PC3 MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

Proposal 6: Support option 2

- Option 2: RAN4 keeps on specifying two PC2 requirements:

  - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB,

  - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB,

Proposal 7: Consider PCx values

- PC21Tx ≦ 3 dB

- PC22Tx ≦ 9 dB

- PC1.51Tx ≦ 6 dB

Proposal 8: It is necessary to specify different  MSD according to the default MSD value

Proposal 9: Consider similar HPUE MSD equation with 1UL case

- HPUE MSD(PC2, PC1.5) = Default MSD +  MSDpc2IMDX or 1.5IMDX

Proposal 10: It is necessary to consider simple equation for deriving MSD value and further discussion on details is needed.

Proposal 11: It is not necessary to consider the IMD index for the  MSD value.

Proposal 12: Consider simplifying power class configuration as shown in the table below

| case # |  | 1 Baseline | 2 | 3 |
| --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 | PC2 | PC1.5 |
|  | PCmax | 23 | 26 | 29 |

Proposal 13: It is necessary to study how to define the default MSD value in the absence of a default MSD for the HPUE MSD simplification.

- References

[1] R4-2420329, “WF on HPUE RF requirements”, Samsung

[2] R4-2420380, “WF on MSD rule”, Skyworks, Murata, Nokia, Qualcomm

[3] R4-2417793, Observations on HPUE 2UL IMD MSDs (skyworks)

[4] R4-2417784, Further discussion on HPUE MSD rules (Murata)

[5] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT