3GPP TSG-RAN WG4 Meeting #114	R4-2501750

Athens, Greece 17th – 21st February, 2025

Source:	Qualcomm Incorporated

Title:	MPR for FR1 intra-band UL CA

Agenda item:	7.1.1.2.2

Document for:	Approval

# 1.	Introduction

In RAN4#113 there was much discussion on FR1 intra-band ULCA. In this paper we further present our views on the remaining open issues on this topic.

# 2. 	Discussion

In RAN4#113 a WF was created to further discuss the open issues related to intra-band contiguous and non-contiguous CA with a single cell activated [2].

It was agreed in RAN4#112-bis that for FR1 there will be no LO retuning for either contiguous or non-contiguous ULCA where only 1 CC is activated [1] as captured below:

#### Issue 2-1-4: Whether UL interruption is allowed for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA.

- Agreement in AH

  - Proposal 1

#### Issue 2-2-1: Applicable MPR for FR1 intra-band non-contiguous UL CA

- Proposals

Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements

  - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

  - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements

  - Proposal 3: single CC MPR applied with single CC emission requirements:

Option 2: Single CC MPR/SEM applied on indication of dualPA IE

  - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE

  - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported.

Option 3: Possible spec impact on LO retuning

  - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation

- WF

  - FFS on whether for PC2 and PC3, no MPR requirements to be changed.

  - FFS PC1.5 when NC CA requirements are available

  - FFS clarification for the applicable emission requirements whether they are single CC based or composite based

Agreement in main session

  - Drop the discussion for LO retuning since it is out of WI scope

    - FFS for FR2

Observation #1: For FR1 no LO retuning for either contiguous or non-contiguous ULCA when only 1 CC is activated.

In RAN4#113 a WF was created to further discuss the open issues related to intra-band contiguous and non-contiguous CA with a single cell activated [2].

#### Issue 2-2-1: Applicable MPR and corresponding spectrum requirements for PC2/PC3 intra-band NC CA

Agreement in main session:

- No change of applicable MPR requirement in current spec but with clarification that applicable emission requirements are composite based regardless of dualPA IE indication.

  - FFS on whether update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability.

  - Note: Composite emissions means combination of individual sub-block emissions.

In our opinion for NC CA it has already been agreed that LO retuning is not considered in this WI. The applicable MPR for intra-band non-contiguous UL CA when only 1 CC is activated was left for further discussion in the WF created in RAN4#113 [2]. From [3] we see that according to section 6.2A.2.2.0 the following is mentioned for UEs indicating and not indicating dual PA-Architecture:

#### 6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

##### 6.2A.2.2.0	General

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively.

Else

MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

For UE without indicating dualPA-Architecture supported

If OR( LCRB1 = 0, LCRB2 = 0 )

For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used;

For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used;

For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

According to [3] for a UE indicating or not indicating dualPA-Architecture for intra-band non-contiguous CA when either LCRB1=0 or LCRB2=0 the single CC MPR tables are used. As the RB determination occurs at scheduling it is seen that for the non-contiguous CA the standard [3] already accounts for the scenario where only 1 CC is scheduled. In the above text the exception stated for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement. These exceptions were added based on measurement work done in [4]

Observation#2: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal#1: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes or updates to the standard are required.

Observation#3: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# Conclusion

In this paper we present further views on MPR applicability for FR1 intra-band UL CA and make the following observations and proposals:

Observation #1: For FR1 no LO retuning for either contiguous or non-contiguous ULCA when only 1 CC is activated.

Observation#2: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal#1: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes or updates to the standard are required.

Observation#3: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# References

[1] [R4-2417204, WF on power domain enhancements, RAN4#112bis, October 14th – October 18th, 2024

[2] R4-2420527, WF on power domain enhancements, RAN4#113, November 17th – November 22nd, 2024

[3] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[4] R4-2204225, 1CC fall-back MPR for NC UL CA with 1LO architecture, February 21st – March 3rd, 2022