3GPP TSG-RAN WG4 Meeting #114 	    R4-2501759

Athens, Greece, 17  – 21 February 2025

Agenda Item:	7.21.1

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

# Introduction

Starting from RAN4#112bis meeting, discussion on issues related to AI/ML for mobility has started in RAN4. Discussions on requirement and testability are presented in our companion papers and general aspects related to AI/ML for mobility are discussed in this paper.

# Discussion

Requirement for prediction

To reduce the measurement overhead and measurement delay for network triggered L3-based mobility, prediction of RRM measurements in time domain, frequency domain and spatial domain is being studied in Rel. 19. Before a detail analysis of impact on RAN4 requirements due to different prediction types for RRM measurement for mobility could start, it is important to establish a common understanding of the terms “prediction” and “predicted measurement/event” to be used in RAN4 discussions.

The term “prediction” should refer to the process of using a trained AI/ML model for producing output(s). And the term “predicted measurement/event” should refer to the output of the trained AI/ML model reported by the UE to facilitate network triggered L3-based mobility. A high-level overview of prediction and predicted measurement/event is provided in Figure 1. Within this framework, the focus/scope of setting the requirements should not be on the details of the AI/ML model deployed to support mobility functionality rather the focus/scope of setting the requirements should be on the measurements performed by the UE before the prediction could start, the output produced by the trained AI/ML model or predicted measurement/event reported to the network, and the delay incurred while reporting the measurement and predictions to the network.

Figure 1. Elaboration of prediction and predicted measurement/event.

Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality.

Requirements for measurements used for prediction

UE needs to perform measurements that it can feed to its trained AI/ML model before the prediction of RRM measurements for mobility can be performed. As per RAN2 evaluations, measurements defined for the legacy L3-based mobility are used for the prediction of the RRM measurements for mobility. For such legacy measurements, requirements and side-conditions under which the requirements are to be met by the UE are well-defined in the existing RAN4 specification. Therefore, when the legacy measurements are used for the RRM measurement prediction for mobility purposes, the UE should at least meet the requirements for the legacy measurements.

Observation 2: Measurements defined for legacy L3-based mobility are used as model input for the prediction of RRM measurement for mobility.

Proposal 1: Requirements and side-conditions for legacy L3-based mobility measurements are applicable to the measurements performed by the UE for RRM measurement prediction for mobility purposes.

Impacts of measurement error on prediction accuracy

In the last meeting, impact of error in measurements used for prediction over the prediction accuracy was discussed and the following options were identified FFS [1].

Candidate options:

- Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

- Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase

- Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

- Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

Way Forward:

Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event.

So far, the evaluations done by RAN2 highlight that the RRM measurement prediction can provide gain in terms of the measurement overhead and delay reductions in comparison to the legacy approach. This gain should however not come at a cost of degradation in the accuracy of the measurement/event reported to the network for the L3-based mobility when the RRM measurements reported by the UE to the network are predicted measurements and not the actual measurement. The accuracy of measurements/events predicted using an AI/ML model heavily depends on the accuracy of the measurements provided as an input to the AI/ML model. Impact of the error in the measurements used for RRM measurement prediction on the accuracy of the predicted measurements/event therefore needs to be evaluated. This is important not only to make a fair conclusion on the applicability of AI/ML model for RRM measurement prediction for mobility at the end of the SI, but also to make a fair judgement on the scope of the normative work to be done during the WI phase. Due to the limited TU budget allocated to RAN4 to study the impact of AI/ML based mobility on requirements, it does not seem feasible to do evaluations in RAN4 that can give us an insight on the impact of error in measurement on the accuracy of the predicted measurements/events. As many companies in the last meeting expressed their concerns on the simulation assumptions made by RAN2 and understand the relevance of the issue, RAN4 should therefore send an LS to RAN2 requesting to at least consider the impact of error in the measurement used as an input to the AI/ML model for RRM measurement prediction in temporal, frequency, and spatial domains. A draft of LS to RAN2 is provided in the Annex of this document.

Observation 3: Impact of the error in the measurements used as input for the AI/ML model for RRM measurement prediction on the accuracy of predicted measurements/event needs to be evaluated.

Proposal 2: RAN4 to send an LS to RAN2 to at least consider impact of error on measurement (for example due to interference from another cell, inaccuracy of the measurement etc.) used for prediction on the accuracy of the predicted measurement/event.

# Summary

In this contribution, general issues that have an impact on different prediction types being studied are highlighted. The discussions presented in this paper are summarized in terms of observations and proposals below.

Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality.

Observation 2: Measurements defined for legacy L3-based mobility are used as model input for the prediction of RRM measurement for mobility.

Proposal 1: Requirements and side-conditions for legacy L3-based mobility measurements are applicable to the measurements performed by the UE for RRM measurement prediction for mobility purposes.

Observation 3: Impact of the error in the measurements used as input for the AI/ML model for RRM measurement prediction on the accuracy of predicted measurements/event needs to be evaluated.

Proposal 2: RAN4 to send an LS to RAN2 to at least consider impact of error on measurement (for example due to interference from another cell, inaccuracy of the measurement etc.) used for prediction on the accuracy of the predicted measurement/event.

# References

- R4-2420095, WF on AI/ML Mobility, Moderator (Nokia).

# Annex: Draft of RAN4 LS to RAN2

3GPP TSG-RAN WG4 Meeting # 114		R4-25xxxxx

Athens, GR, 17 – 21 February 2025

Title:	On consideration of impact of error in measurement on prediction accuracy of RRM measurement and event for mobility

Response to:	N/A

Release:	Rel-19

Work Item:	FS_NR_AIML_Mob

Source:	RAN WG4

To:	RAN WG2

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments: N/A

1. Overall Description:

Based on the discussions held in RAN4#114 meeting, it is RAN4 understanding that the impact of the error in the measurements used as input for the AI/ML model for RRM measurement and event prediction on the accuracy of predicted measurements/event needs to be evaluated. RAN4 would therefore like to request RAN2 to consider the following in RAN2 evaluations:

- Impact on the accuracy of RRM measurement prediction and event prediction provided the accuracy of measurement used as an input to the AI/ML model follows the accuracy requirements defined in clauses 10.1.2, 10.1.3, 10.1.4, and 10.1.5 of TS 38.133.

- Simulate the impact of interference when UE is performing measurements for model input on the accuracy of RRM measurement prediction and event prediction.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to take above feedback into consideration while evaluating the performance of AI/ML model for RRM measurement and event predictions for mobility.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114bis			07 – 11 April 2025				Wuhan, CN

TSG RAN WG4 Meeting #115				19 – 23 May 2025				Malta, MT