3GPP TSG-RAN WG4 Meeting #114						    R4-2501762

Athens, Greece, 17th -22nd February, 2025

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#113 there was a new WF [1] to further study power domain enhancements for NR single carrier. In this paper we present additional views on this topic.

# 2. 	Discussion

In RAN4#113 there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier. Some of the unresolved issues are noted below:

In the last meeting there was much discussion on how the UE can either calculate or be informed of how many free RBs it has on either side of the UE-specific CBW. The two main proposals that were discussed for determining the number of free RBs were either by indication through NW signalling or by UE calculation based on information received during RRC configuration and via SIB information. We think that using UE-specific CBW which is assigned to the UE at RRC configuration in conjunction with the cell-specific carrier bandwidth obtained via the SIB1 information should be used to determine the number free RBs on either side of the UE CBW.

Proposal 1: UE-specific CBW assigned to the UE at RRC configuration in conjunction with the cell-specific carrier bandwidth obtained via the SIB1 information should be used to determine the number of RBs available for ‘extension’ on each side of the UE CBW

In this contribution, ‘RBs available for ‘extension’ on each side of the UE CBW’ are referred to as ‘free RBs’ for compactness.

In the above mentioned WF it was proposed that companies attempt to formulate the appropriate wording to describe the inner RB region when the UE CBW is extended. In figure 1 the original MPR region comprising of inner and outer regions are shown in the left-side plot and the inner and outer regions for the extended bandwidth scenario is given in the right-side plot. The intersection of the original MPR region with the inner region of the extended bandwidth, indicated in the right-side plot in light green, is the new inner region in the extended bandwidth scenario.

Figure 1: MPR regions for original and extended bandwidth scenarios [2]

Therefore, the MPR region of the original UE-specific CBW which can use inner RB MPR inside the extended UE-specific bandwidth is the intersection of the inner MPR region of the extended UE-specific CBW with the original MPR region of the original UE-specific CBW. Putting this in words makes the description unwieldy:

The MPR region inside the original UE-specific CBW which can use inner RB MPR inside the extended UE bandwidth is given by the intersection of the inner MPR region of the extended UE-specific CBW with the MPR region of the original UE-specific CBW.

Moreover, re-using existing MPR definitions implies limited change to applicable SEM requirements. There is however not sufficient clarity on this aspect currently.

Observation 1: Postpone wording to describe the inner RB region when the UE CBW is extended as it can be problematic.

The UE-specific channel bandwidth expansion should not exceed the BS bandwidth as any region outside the BS BW may not belong to the same operator and expansion of BW into those regions may not be allowed.

Proposal 2: The UE-specific extended channel should be contained within the cell-specific carrier spectrum (i.e. bandwidth and location).

It was earlier agreed that bandwidth extension can be symmetric or asymmetric on either side of the UE-specific CBW [3]. There are several ways that this BW expansion feature can be implemented. As illustrated in figure 2 one way is to add a certain number of RBs, denoted as p and q to one or both sides of the UE-specific CBW based on the number of free RBs on each side of the spectrum. This method gives the greatest flexibility for BW expansion. Here NRB is the maximum number of RBs in the original UE-specific CBW.

Figure 2: UE-specific CBW extension

Note the UE-specific CBW occupies a slightly larger BW than NRB as it includes the guard band on both sides of the NRB. The extra virtual RBs, p and q are added starting from the left of RB(1) and the right of RB(NRB) and there is no guard band between the extra RBs and NRB.

It is not possible to talk about MPR regions and MPR without first defining all the applicable emissions requirements. The next aspect to consider therefore is what emissions requirements would apply to the UE when it is enabled to use the ‘extended BW’ concept. The specific requirements are ACLR, SEM and SE.

The figure below shows various BW parameters together when ‘extension’ is enabled

We evaluate the 3 OOB emissions criteria based on this figure.

SE:

ITU in SM.329 define SE based on the ‘necessary bandwidth’ (NB), which in turn is defined as:

For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Since NB is unchanged despite the extension, the frequency ranges of applicability of the ITU requirements also remain unchanged, at 2*NB away from channel edge. UE-specific CBW can be used as proxy for NB.

Proposal 3: SE starts no further than 2x UE-specific CBW from channel edge when network enables UE to assume BW extension

The UE-specific CBW extension, X on either side should be configured such that the frequency offset from where the ITU emissions start should be greater than the frequency offset from where 3GPP SE limits apply. This means that:

(2*UE-specific CBW) ≥ (X+UE-specific CBW + 5MHz).

i.e. X ≤ (UE-specific CBW - 5MHz)

Observation 2: The maximum allowed extension on each side is limited to UE-specific CBW - 5MHz

SEM:

For SEM, the choices (beyond trivial case of no relaxation) are whether the extended BW is used as the basis or the SEM mask is slid over by the extension. Using a mask meant for a narrow BW is the more conservative assumption, except for the first bin. Note however that neither MPRs nor definition of inner change even though the first bin limits change as a function of BW, so it follows that the standard is already compliant to the tightest configuration of the first SEM bin.

Proposal 4: when network enables UE to assume BW extension The SEM mask is derived from the legacy SEM but offset in frequency (slid over) by the extension value

ACLR:

ACLR does not have direct regulatory exposure except for regulators that adopt 3GPP requirements. ACLR is a requirement that is self-imposed by 3GPP to enable like-RAT neighbor channel operation. A reasonable proposal is to retain this requirement from the native un-extended BW, but to move the integrated BW by the extension.

Proposal 5: when network enables UE to assume BW extension the ACLR is derived from the legacy ACLR requirement but offset in frequency by the extension value

With these proposals in place it is possible to set expectations for the UE’s UL power because the exercise of determining inner or outer is a matter of merely renumbering RBs to account for the extensions.

The equations for finding the inner RBs are now modified as follows:

NRB’=NRB+p+q

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB=NRB+p+q-RBStart,Low-LCRB

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

The above equations can be used to determine the new inner region based on p and q. The SEM region starts p RBs before the left edge and q RBs beyond the right edge of the UE-specific CBW, in the frequency domain. This approach can be used as alternative to defining in words, the inner RB region when the UE CBW is extended

Proposal 6: An allocation belongs to the inner RB region when the UE-specific CBW is extended when it satisfies the following inequality:

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB

Where:

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE-specific CBW respectively

While this scheme allows p and q to take on any value subject to observation 2 (i.e. allows maximum flexibility for adding RBs on either side of the UE-specific CBW), it requires enormous effort to validate therefore making it impractical for actual implementation.

Observation 3: Extending the UE specific CBW by an arbitrary number of RBs on one or both sides of the spectrum leads to many RB combinations which would take enormous effort to validate

To reduce the validation effort it is proposed to limit the bandwidth extension to NRB/2 based on the number of free RBs. If the number of free RBs is greater than NRB/2 then the UE-specific CBW extension is limited to NRB/2. If the number of free RBs is less than NRB/2 then no CBW extension is allowed.

Proposal 7: If the number of free RBs is greater than NRB/2 then the UE-specific CBW extension is limited to NRB/2 if the number of RBs in the BW = (UE-specific CBW - 5MHz) is greater than or equal to NRB/2   . If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE-specific CBW - 5MHz) is less than NRB/2 then no CBW extension is allowed.

In our opinion it makes sense to use IBE from the edges of the original UE-specific CBW to the edges of the extended UE-specific CBW. Also, that IBE in this region should be an extension of the IBE in the original UE-specific CBW region as indicated in figure 3.

Proposal 8: IBE should be used from the edges of the UE-specific CBW to the edges of the extended UE-specific CBW

Figure 3: Extension of IBE mask

Proposal 9: The IBE from the edges of the UE-specific CBW and the edges of the extended UE-specific CBW should be an extension of the IBE for the signal within the UE-specific CBW.

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: UE-specific CBW assigned to the UE at RRC configuration in conjunction with the cell-specific carrier bandwidth obtained via the SIB1 information should be used to determine the number of RBs available for ‘extension’ on each side of the UE CBW

Observation 1: Postpone wording to describe the inner RB region when the UE CBW is extended as it can be problematic.

Proposal 2: The UE-specific extended channel should be contained within the cell-specific carrier spectrum (i.e. bandwidth and location).

Proposal 3: SE starts no further than 2x UE CBW from channel edge when network enables UE to assume BW extension

Observation 2: The maximum allowed extension on each side is limited to  UE-specific CBW - 5MHz

Proposal 4: when network enables UE to assume BW extension The SEM mask is derived from the legacy SEM but offset in frequency (slid over) by the extension value

Proposal 5: when network enables UE to assume BW extension the ACLR is derived from the legacy ACLR requirement but offset in frequency by the extension value

Proposal 6: An allocation belongs to the inner RB region when the UE-specific CBW is extended when it satisfies the following inequality:

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB

Where:

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE-specific CBW respectively

Observation 3: Extending the UE specific CBW by an arbitrary number of RBs on one or both sides of the spectrum leads to many RB combinations which would take enormous effort to validate

Proposal 7: If the number of free RBs is greater than NRB/2 then the UE-specific CBW extension is limited to NRB/2 if the number of RBs in the BW =(UE-specific CBW - 5MHz) is greater than or equal to NRB/2   . If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE-specific CBW - 5MHz) is less than NRB/2 then no CBW extension is allowed.

Proposal 8: IBE should be used from the edges of the UE-specific CBW to the edges of the extended UE-specific CBW

Proposal 9: The IBE from the edges of the UE-specific CBW and the edges of the extended UE-specific CBW should be an extension of the IBE for the signal within the UE-specific CBW.

# References

[1] R4-2420527, WF on power domain enhancements, RAN4#113, November 17th – November 22nd, 2024

[2] R4-2418015, Redefining allocation types for improved MPR with BS BW larger than UE CBW, USA, 18 – 22 November 2024

[3] R4-2417204, WF on power domain enhancements, RAN4#112-bis, October 12th – October 18th , 2024