3GPP TSG-RAN WG4 Meeting #114	R4-2501787

# Athens, Greece, February 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on the methodology of NR SCM for demodulation

Agenda item:	7.16.2

Document for:	Discussion

# Introduction

In the RAN4#113 meeting, companies agreed on the simulation parameters of interested test cases to compare different methodologies and further align [1].

In this contribution, the open issues on the impact of antenna configuration, beamforming, and CSI port mapping will be continually analyzed according to different test cases. The proposed methods, such as CDL extension methodology and dual-directional beam steering TDL methodologies, etc., are further compared by derivation limitation and simulation results.

# Discussion

## 2.1	Common for all methodologies

#### Cases for SU-MIMO PDSCH

Way Forward (from RAN4#113):

Random PMI as a baseline, FFS Fixed.

Interested companies invited to choose based on most common PMI in PMI histogram.

The general PDSCH requirement is defined based on random PMI, which would let the channel estimation performance be independent of the BS behavior and UE PMI reporting performance (follow PMI from UE or not). This leads to no spatial preference and corresponding performance degradation, but it might be an understandable way for TDL model simulation since no spatial preference is shown in the TDL model. In that case, some companies propose to consider follow PMI under the CDL model and checking the PMI histogram based on result statistics. It would help to show a “high possibility” UE behavior under such a channel model. Then, the most common PMI configuration could be extracted from the histogram, and a simulation could be launched based on this highly probable PMI, referred to as a “fixed” PMI configuration. Moreover, a fixed most preferred PMI configuration might check the performance of UE product(s) on channel estimation and whether it is aligned with the priori channel information in theoretical speaking.

But the methodology above still has unclear parts, such as PMI type (e.g., wideband or sub-band?) and target SNR/throughput range, which would impact the comparison results. In our simulations, we use wideband PMI and get PMI histogram at high SNR/throughput (>70%) range.

Theoretically, a fixed most preferred PMI test could reflect UE channel estimation performance against a priori channel information through simulations.

Proposal 1	The principle of how to get the fixed preferred PMI should be clarified.

Compared to the random PMI configuration, the follow PMI configuration is expected to have better performance if UE has good enough channel estimation, and the fixed PMI configuration might also show relative better performance than random PMI but a bit worse performance than the follow PMI (i.e., lacking adaption for angular spread). Based on our simulation, the expected performance trend (follow > fixed > random) is seen for 4Tx4Rx with 4 layers; see figure 2.2.1.2-1 in [2]. But a different trend is observed in the results with 8 layers, where, random > follow > fixed; see figure 2.1-1 below. In that case, a suspicious conclusion could be reached that random PMI is the best choice for 8 layers transmission under FR1 Uma CDLC channel.

(a) 128AE FC 4Tx4Rx 4 layers sub-band PMI                                  (b) 128AE FC 8Tx8Rx 8 layers wideband PMI

Figure 2.1-1 Performance comparison between random PMI and fixed PMI with FR1 Uma CDLC

The SU-MIMO PDSCH with 4 layers shows better performance with fixed PMI than random PMI, but 8 layers show worse performance with fixed PMI.

By observing the per CW performance (see figure 2.2.1.1-1 (b) in [2]), the CW1 gets better performance with fixed PMI when it is transmitted by 1st and 2nd strongest beams, and CW0 gets much worse performance with fixed PMI when it is transmitted by 3rd and 4th strongest beams, see Figure 2.1-2 below. Compared to 1st and 2nd strongest beam directions, beamforming gain at the 3rd and 4th strongest beam directions have 3dB ~ 7dB lower gain at the peak direction and potential high inter-layer interference caused by sidelobes. Additionally, channel fading at the 3rd and the 4th beam directions is also higher based on the channel model profile. In that case, the receiver side would get a much lower gain for layers transmitted along the 3rd and the 4th beam directions.

Therefore, it might be more helpful to search and use other 3rd and 4th strongest beam directions, for example switching them from 100o vertical angle plane to 90o plane. Regarding the vertical traffic beam width could be 13o for 128AE (8, 8, 2) full connected with 0.5 lamda vertical separation , the inter-beam interference could be high when beam grid only have 10o separation. By increasing vertical AE separation to 0.7 lamda would reduce vertical beam width to 9o and might improve the total performance.

Figure 2.1-2 channel spatial power and beam gain of top 4 strongest beams by 128AE FC

The CW transmitted along current 3rd and 4th strongest beam directions with followed PMI under FR1 Uma CDLC channel would lead to much worse performance than random PMI.

When a requirement is considered, it could be possible to use fixed-PMI configuration to under a deterministic CDL channel model. But the precondition is high PMI preference can be seen based on follow-PMI simulations with a certain PMI hypothesis. Based on the simulation results summary of Table 2.2.1.2-2 in [2], the fixed PMI could achieve ~2dB gain than random PMI for most of PDSCH with 4 layers cases, but negative gain on 8 layers transmission. Thus, it could be furtherly investigated if “fixed PMI” would be used for PDSCH performance tests.

#### Receiver Type

Agreement:

IRC Baseline for SU-MIMO;

Interested Companies to use IRC and E-IRC for MU-MIMO and LMMSE for SU-MIMO

Regarding random PMI is used, the inter-layer interference would be averaged along spatial domain no matter what kind of receiver is used. In that case, the performance of IRC, LMMSE or MRC could be close in SU-MIMO scenario. Figure 2.1-3 show the comparison results below.

Figure 2.1-3  Normalized throughput for different receiver types

No obvious performance difference between IRC and LMMSE receiver for SU-MIMO PDSCH with random PMI configuration.

#### Cases for MU-MIMO

Agreement:

Interested companies to bring to RAN4#114 views on the following cases:

PDSCH

2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

Interested companies are encouraged to assess 2+2 layers with IRC and E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE)

In a general view, MU-MIMO scenario focus on cell/network level performance more than link level performance because the feature heavily depends on how to schedule UE pairs to achieve higher cell throughput. At the same time, there would be intra-cell inter-user interference when beamforming can’t be fully orthogonal among UEs. Then the UE capability of interference cancelation would also impact the cell performance, but this ability cannot be dominating issue for MU-MIMO scenario like network scheduling. Thus, network normally would try to select UE pair with smaller intra-cell interference, such as selecting UE with good enough SINR, larger departure/arrival angle difference etc., which the scheduling algorithm depends on vendors’ implementations.

The MU-MIMO scenario focus more on cell/system level throughput which would be dominated by network scheduling algorithm more than UE capability of interference cancelation.

The existing MU-MIMO PDSCH requirements focus on intra-cell inter-user interference cancelation. The whole test focus on target UE performance and there is no aggressor UE is used during the test. In that case, one of the key issues here is how to derive the interference profile. Rel-18 and Rel-19 NR reuse LTE interference profile to derive interference power level and use TDL model to define requirements. In corresponding LTE SI, the system level simulation campaign was setup to have large scale SINR distribution, then specific DIP values were chosen for interference profile. The final IRC receiver performance is verified by normalized PDSCH throughput with random PMI configuration. For small scale propagation per aggressor, its spatial distribution might not be so important here since what we care is the total interference power level at receiver of target UE. Meanwhile, the random PMI configuration will average interference in spatial domain. So, if the target UE has been checked with SCM under SU-MIMO case and the legacy IRC performance with TDL model, it’s IRC receiver would be expected to perform well with a proper SCM either.

In that case, it seems no necessary to derive SCM for MU-MIMO case especially if only target UE performance is cared about.

The spatial domain distribution of interference might not be important due to random PMI is used in current MU-MIMO IRC test setup.

Some companies deliver a procedure for MU-MIMO CDL model derivation based on “spatial consistency” in 38.901 [3]. There are two procedures A and B are introduced.

- The procedure A derive different channel models recursively based on UE initial state and its trajectory. The derived channel heavily depends on the previous state channel, so it lacks stochastic changes regarding real multiple user scenario. Furthermore, the updated distance between UT/BS is limited within 1m. To have larger distance, multiple drops (initial channel) might be needed based on our understanding, then the correlation with previous drop might be reconsidered every time.

- The procedure B derive all UE link channels separately but consider correlation function between links according to inter-user distance. This method would have better stochastic than procedure A but worse consistency.

If we would use these procedures to derive channel model for other users on the map, several issues could be considered respecting to real condition.

- Random UE antenna conditions (coordination, types, radiation directions etc.). It is not reasonable to assume all UE have same conditions for MU-MIMO scenario, such as all isotropic, all at same height and all pointing to BS. The channel correlation between two close UE could also be low due to very different antenna radiation pattern, then network would also schedule them simultaneously.

- Random multipath conditions (including delay spread, angle spread, ray coupling etc.). It is not reasonable to assume exactly same multipath parameters for all users. Of course, the derivation effort would be very huge if we want to fix all random variables for different users.

If we ignore the randomness of UE condition in real network, then the justification is lacking for artificial configuration of aggressor UEs. In theoretical, a system level simulation would also be needed but it could be impossible.

Based on analysis above, it seems that companies should make it clear about the MU-MIMO SCM derivation purpose at the first, and then should address the justification and benefit of this derivation comparing to SU-MIMO SCM derivation.

Proposal 2	Companies should make it clear about the MU-MIMO SCM derivation purpose at the first, and then should address the justification and benefit of this derivation comparing to SU-MIMO SCM derivation.

#### Metrics for comparison

Contributions welcome at RAN4#114.

In previous meeting, it was agreed that companies bring full throughput curves for comparison. It could be helpful to see the channel model implementation difference between companies and result diversity level between different channel models.

If the comparison purpose is to check the performance difference between different companies, full curves could show a whole picture, but it could be also useful to check performance under a specific metric (e.g., metric in legacy requirement) to be aligned or not from demodulation requirement perspective. Following metrics could be considered:

- SU-MIMO PDSCH

  - For both random PMI and fixed PMI

    - SNR at 70% normalized and combined average throughput.

    - For 8 layers, SNR at 70% normalized per CW throughput and the gap between CWs.

  - For fixed PMI only

    - Preferred PMI based on PMI histogram.

- SU-MIMO PMI

  - SNR at 90% normalized throughput with followed PMI.

  - Gamma value to random PMI configuration.

If the comparison purpose is to make decision on which SCM would be used as output of this SI, then we could use following metrics:

- Channel model availability which impacts the effort for model derivation.

- Practicability which shows deployment characters (antenna, environment, etc.)

- Performance expectation, such as align ability among companies, reasonability of per CW performance (for 8 layers), PMI preference etc.

- Testability which impacts the model profile and test setup.

Proposal 3	Consider following metrics for simulation results or channel model comparison.

- Comparison among companies’ results for alignment.

  - SU-MIMO PDSCH

    - SNR at 70% normalized and combined average throughput. For 8 layers, SNR at 70% normalized per CW throughput and the gap between CWs.

    - Preferred PMI based on PMI histogram for fixed PMI.

  - SU-MIMO PMI

    - SNR at 90% normalized throughput with followed PMI.

    - Gamma value to random PMI configuration.

- Comparison among channel model methodologies.

  - Channel model availability.

  - Practicability.

  - Performance expectation.

  - Testability.

## 2.2 	TDL based methodologies

#### Tap Delay Line Parameters

Contributions welcome at RAN4#114.

The number of taps in TDL channel models were discussed in RAN4 from the beginning of NR. It was agreed that the number should be less or equal to 12 and the resolution between taps should be less than 5ns to reduce the complexity of TE implementation. But in previous RAN4#112bis and RAN4#113 meeting, TE vendors seem fine with the CDL channel model with more than 12 clusters, but no clear view on the delay resolution. It could be better to invite TE vendors give more clarifications on it.

By checking FR1 Uma CDLC model in 38.827, there are 24 clusters, and the minimum delay separation is <5ns within one strong cluster and minimum delay separation is <1ns between clusters. Regarding there are also 20 rays in one cluster, the complexity of CDL profile would be very complex. Once it is turned to a TDL model, the straightforward way is to convert all angle spread and antenna relevant parameters to correlation matrix. But per-tap correlation matrix could be very large if BS have a very large antenna array and the total number of taps is high. Furthermore, two taps might have very close absolute delay (i.e., <1ns) even their correlation matrix are different. In that case, it might be possible to combine taps with very similar delay but consider how to combine their correlation matrices furtherly.

In some investigation articles, equivalent per channel correlation matrix could be derived to replace per-tap correlation matrix to reduce complex [5]. It could be considered to improve the existing correlation matrix for high layer transmission if it is feasible.

TE vendors seem fine with CDL models in TR38.827 which have up to 24 clusters.

The minimum delay separation between different clusters could be <1ns in FR1 Uma CDLC model in TR38.827.

It could be possible to use per-channel correlation matrix rather than per-tap correlation matrix together with reduced number of taps to simplify TDL extension model.

Proposal 4 	Invite TE vendors give confirmation on the minimum delay separation between clusters.

Proposal 5	Investigating the derivation of per-channel correlation matrix based on per-tap correlation matrices if TDL extension and tap combination are used for SCM.

#### TDL Model Extensions

Contributions welcome at RAN4#114.

There are three TDL model based methodologies are proposed in previous meetings:

Method 1 (R4-2412762):

Covert CDL model to TDL model with per tap correlation matrix. This method is indicated by TR38.901 which would have quite similar spatial channel fading as CDL model in theoretical analysis. But the problem of it is also obvious that the channel profile (especially correlation matrix) would be very large and complicated depending on BS antenna size and the number of taps. Considering no TE vendor ever implemented similar model for now, its testability is doubtable. Regarding the timeline of this SI, it would be very challenging to evaluate SCM based on this method.

It will be very challenging to evaluate methodology that TDL model with per-tap correlation matrix in Rel-19 timeline.

Method 2 (R4-2415382):

Reuse TDL model, correlation matrices and dual-clusters Tx beam steering method in existing demodulation specification (generally used for multiple PMI tests) but apply Rx beam steering matrices additionally to distinguish arrival angles at receiver side. With this approach, multiple cluster beams could be created by applying Tx-Rx beam steering matrices on multiple independent TDL channels with correlation matrix. The power split among beams and the steering angles are configurable.

Based on this method, all beams have similar inner tap statistical distributions, such as tap delay and relative tap power. Furtherly, the beam steering angles are normally orthogonal and evenly separated between each other. The same correlation matrix is applied for all beams/clusters which could be OK if channel of beams/clusters are independent, but the issue of existing correlation matrix, such as not practical for high layer transmission, is just circumvented. Using independent channel between beams would be similar as MU-MIMO condition that different BS-UE links could be considered as independent.

The pros of this method include that easily adjusted beam angle and power, reduce correlation matrix impact among beams and the test setup could be simple. The cons of this method include treating beams as independent channels (not so practical for SU-MIMO) with same channel model, beam directions are relative ideal.

As for method in R4-2415382,

- Pros: easily adjusted beam angle and power, reduce correlation matrix impact among beams and the test setup could be simple.

- Cons: treating beams as independent channels with same channel model which seems not so practical for SU-MIMO; departure and arrival beam directions are evenly separated.

Method 3 (R4-2419895):

On top of idea of R4-2415382, split one TDL channel taps into separate sets to represent different beams and adding beam directions by applying Tx-Rx beam steering matrices separately.

Based on this method, different beams would have different number of taps, different RMS delay and different relative power. Since tap delay in TDL model are increment, so the beam delays are also increment. The beam angles are also orthogonal and evenly separated between each other. But it is not clear that how to choose taps for a beam, for example number of taps. Furthermore, the correlation matrix impact is also not clear.

The pros of this method are similar as R4-2415382, but only using one channel for a link which would be more reasonable in SU-MIMO scenario. The cons of this method also include relative ideal beam configurations (equivalent separation and relative even power split). Furthermore, there is no general procedure description to adaptive this method to all cases.

As for method in R4-2419895:

- Pros: easily adjusted beam angle and power, reduce correlation matrix impact among beams and the test setup could be simple. One channel for a link.

- Cons: departure and arrival beam directions are evenly separated. Power of beams are evenly split. No general procedure description.

In summary, TDL model based extension methods proposed by companies are more artificial model aiming to simplify demodulation tests. Compared to the existing CDL model, it looks a bit more ideal if we tend to use SCM in 6G. However, there were no many discussions on these methods during previous meetings. It would be worthy for companies to check and bring ideas to improve it if possible.

Proposal 6	Companies to check and bring ideas to TDL model based methodologies if possible.

## 2.3	CDL based methodologies

#### Angles of Departure and Angles of Arrival

Way Forward:

Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

Set mf,desired based on the LOS direction between the TX and RX

Option 2: Following 38.827 (i.e., mf,desired = mf,model).

The purpose of Option 1 is to have new CDL channel model under new desired departure and arrival angles which could fulfill the PMI test setup. To verify PMI performance, multiple PMI configurations should be tested so that the channel model and beamforming directions should be updated accordingly.  In current PMI test, multiple angles with TDL model are implemented by predefined beam steering matrix. For CDL model, multiple channel models need to be derived regarding multiple departure and arrival angles. It could need very high effort for channel model profile alignment, simulation results alignment and model implementations in test equipment.

Option 2 just use one angle configuration which is better for PDSCH performance verification rather than PMI test. Or just use Option 2 for model comparison and simulation result and alignment to simplify the scenario.

Proposal 7	Consider Option 1 for SU-MIMO PMI performance verification.

Proposal 8	Consider Option 2 for model comparison and simulation result alignment only.

#### SNR Definition with CDL

Agreement:

Define normalized receive signal power as the following:

The sum of signal power is normalized across all Rx RF chains using a single normalization factor. The normalization factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain].

The normalization factor will be aligned and can be included in TR 38.753.

Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

Some discussions on the SNR definition regarding beamforming is used in 38.827 models and also possibly used in 38.753 model. The key issue here seems the beamforming will “enhance” receiving signal power at the receiver side, then might lead to quite SNR definition compared to existing one in specification.

The received signal could be simply expressed as y=Hx+n, where H is the channel and n is the noise. Compared to legacy test setup with TDL channel, the H would be power normalized no matter CDL model and/or beamforming are considered or not. The beamforming with normalized weight itself won’t add power to Tx signal, and the gain might be different per departure angles. In that case, the sum of signal power across all Rx Rf chains should be the same as before.

It could be noted that, the formula (7.2-8) for BF weight in TR38.827 seems incorrect on power normalization. The original formula is

α1,1⋯αMe,Ne=1MeNeexp-j2πrtx,maxT∙dtx,Me,Neλ0∈C1×MeNe

Where the power normalization factor is 1MeNe  which should be 1MeNe to keep the i=1, j=1i=Me, j=Neαi,j2=1.

The sum of received signal power is the same no matter beamforming is used or not if beamforming weight and channel power are normalized.

The power normalization factor should be 1MeNe in (7.2-8) in TR38.827.

Proposal 9	No need to consider normalization factor because AAV and beamforming weights are generally power normalized.

## 2.4	Comparison of methodologies

In this chapter, we would summarize the observations from our simulations [2] based on agreed parameters in WF [1].

### 2.4.1 SU-MIMO PDSCH

In our simulations, different AAV approaches (full connected, subarray virtualization and default single AE) are compared. As some companies proposed, we also derive the most preferred PMI based on histogram of follow PMI results, and furtherly compare performance with random PMI. FRI Uma CDLC model and TDLC300-100 model are compared in some cases.

#### 8Tx8Rx with 8 layers results

The simulation results are summarized in Table 2.2.1.1-1 and Table 2.2.1.1-2 in [2] which are copied here.

Table 2.4.1-1 Results summary of preferred PMI for 8Tx8Rx 8 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI histogram |
| --- | --- | --- | --- |
| 128AE (8,8,2, 8, 8) | FC | FR1 Uma CDLC |  |
|  |  | TDLC300-100 |  |
| 128AE (8,8,2, 8, 2) | SV | FR1 Uma CDLC |  |
|  |  | TDLC300-100 |  |
| 64AE (8,4,2, 8, 4) | FC | FR1 Uma CDLC |  |
| 64AE (8,4,2, 8, 1) | SV | FR1 Uma CDLC |  |
| 8AE (1,4,2,1,1) | 1 to 1 | FR1 Uma CDLC |  |
|  |  | TDLC300-100 |  |

Table 2.4.1-2 Results summary of SNR at 70% normalized throughput for 8Tx8Rx 8 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI configuration | SNR at 70% NThp. [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CW0 | CW1 | Combined Average |
| 128AE (8,8,2, 8, 8) | FC | FR1 Uma CDLC | Random | 20.5 | 15 | 17.5 |
|  |  |  | Fixed | 27 | 7.5 | 25.1 |
| 128AE (8,8,2, 8, 2) | SV |  | Random | 14 | 22.5 | 20.7 |
|  |  |  | Fixed | 0.5 | 28.3 | 26.5 |
| 64AE (8,4,2, 8, 4) | FC | FR1 Uma CDLC | Random | NA | 17 | NA |
|  |  |  | Fixed | NA | 1 | NA |
| 64AE (8,4,2, 8, 1) | SV |  | Random | 27 | 27.5 | 27.2 |
|  |  |  | Fixed | 17.3 | 31.7 | 30.5 |
| 8AE (1,4,2,1,1) | 1 to 1 | FR1 Uma CDLC | Random | 33.2 | 33.8 | 33.5 |
|  |  |  | Fixed | 25 | 37.5 | 36.5 |

On PMI preference:

With larger antenna array (128 AE), full connected method could show obvious PMI preference on both CDLC and TDLC models, but TDLC model also show bias compared to CDLC. By subarray virtualization, the preferred PMI would be biased a bit on CDLC model, but TDLC model almost show no clear preference.

Reducing antenna array (64 AE), especially on horizontal direction, would greatly impact the PMI preference due to less beam power and direction precision.

With 8AE antenna without full connection, both CDLC and TDLC model show no clear PMI preference.

On throughput performance:

Compared to random PMI, fixed PMI would increase performance of CW sent along strong channel directions but reduce performance of CW sent along weak channel directions on all antenna configurations. The averaged throughput is normally degraded.

Reducing antenna array from 128AE to 64AE, especially on horizontal direction, would greatly degrade the performance both per CW and average. Random PMI show similar per CW performance.

With 8AE antenna without full connection, random PMI show similar performance on both CW.

#### 4Tx4Rx with 4 layers results

The simulation results are summarized in Table 2.2.1.2-1 and Table 2.2.1.2-2 in [2] which are copied here.

Table 2.4.1-3 Results summary of preferred PMI for 4Tx4Rx 4 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI histogram |
| --- | --- | --- | --- |
| 128AE (8, 8, 2, 8, 8) | FC | FR1 Uma CDLC |  |
| 128AE (8, 8, 2, 8, 4) | SV |  |  |
| 64AE (8, 4, 2, 8, 4) | FC | FR1 Uma CDLC |  |
| 64AE (8, 4, 2, 8, 2) | SV |  |  |
| 4AE (1, 2, 2,1,1) | 1 to 1 | FR1 Uma CDLC |  |
|  |  | TDLC300-100 |  |

Table 2.4.1-4 Results summary of SNR at 70% normalized throughput for 4Tx4Rx 4 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI configuration | SNR at 70% NThp. [dB] |
| --- | --- | --- | --- | --- |
| 128AE (8, 8, 2, 8, 8) | FC | FR1 Uma CDLC | Random | -1.2 |
|  |  |  | Fixed | -2 |
| 128AE (8, 8, 2, 8, 4) | SV |  | Random | 1.7 |
|  |  |  | Fixed | -0.1 |
| 64AE (8, 4, 2, 8, 4) | FC | FR1 Uma CDLC | Random | 6.5 |
|  |  |  | Fixed | 3.2 |
| 64AE (8, 4, 2, 8, 2) | SV |  | Random | 3.9 |
|  |  |  | Fixed | 2.5 |
| 4AE (1, 2, 2,1,1) | 1 to 1 | FR1 Uma CDLC | Random | 18.3 |
|  |  |  | Fixed | 16.8 |

On PMI preference:

By using same PMI hypothesis as 8 layers, large antenna array (128AE) with 4 layers transmission shows less PMI preference on specific PMI value due to oversampling on spatial domain. But the preference could be seen on a “PMI cluster”.

Reducing antenna array from 128AE to 64AE, the obvious PMI preference could be seen especially on full connected method.

With 8AE antenna, CDLC model can show obvious PMI preference than TDLC model.

On throughput performance:

Fixed PMI performance can show better performance than random PMI as expected for all antenna configurations.

When antenna array is reduced to 64AE, performance of subarray virtualization is better than full connected.

### 2.4.2	SU-MIMO PMI

#### 2.4.2.1 Type-I codebook

The simulation results are summarized in Table 2.2.2.1-1 in [2] which are copied here.

Table 2.4.2.1-1 Results summary of PMI results for Type-I codebook

| BS antenna Config. (M,N,P,Ms,Ns) | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp. [dB] | NThp. of Random PMI | Gamma |
| --- | --- | --- | --- | --- | --- | --- |
| 128AE FC (8,8,2, 8, 8) SV (8,8,2, 8, 2) | 4 | FC | FR1 Uma CDLC | 5 | 77% | 1.16 |
|  |  | SV |  | 1.8 | 52% | 1.73 |
|  | 2 | FC |  | -4.5 | 78% | 1.15 |
|  |  | SV |  | -5.5 | 47% | 1.91 |
| 64AE FC (8,4,2, 8, 4) SV (8,4,2, 8, 1) | 4 | FC | FR1 Uma CDLC | 7.5 | 62% | 1.45 |
|  |  | SV |  | 6.2 | 50% | 1.8 |
|  | 2 | FC |  | -3.9 | 62% | 1.45 |
|  |  | SV |  | -5.6 | 35% | 2.57 |
| 8AE (1,4,2,1,1) | 4 | 1 to 1 | FR1 Uma CDLC | 14 | 48% | 1.88 |
|  |  |  | TDLC300-100 | 18 | 54% | 1.67 |
|  | 2 |  | FR1 Uma CDLC | 2.5 | 34% | 2.65 |
|  |  |  | TDLC300-100 | 2.5 | 34% | 2.65 |

The full connected AAV approach would get small gamma value compared to subarray virtualization approach because subarray virtualization can achieve better performance under follow PMI but worse performance under random PMI.

Similar gamma values are achieved between 4 layers and 2 layers.

With legacy 8 AE BS antenna configuration, TDLC channel could achieve similar performance as FR1 Uma CDLC model.

#### 2.4.2.2	Comparison between eType-II and Type-I codebooks with wideband PMI precoder granularity

The simulation results are summarized in Table 2.2.2.2-1 in [2] which are copied here.

Table 2.4.2.2-1 Results summary of PMI results using wideband eType-II codebook.

| BS antenna Config. (M,N,P,Ms,Ns) | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp [dB] | NThp. of Random PMI | Gamma Type-II codebook | Gamma Type-I codebook |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128AE FC (8,8,2, 8, 8) SV (8,8,2, 8, 2) | 4 | FC | FR1 Uma CDLC | 4.99 | 78% | 1.15* | 1.16 |
|  |  | SV |  | 2.84 | 57% | 1.57* | 1.73 |
|  | 2 | FC |  | -8.58 | 47% | 1.91 | 1.15 |
|  |  | SV |  | -8.99 | 36% | 2.5 | 1.91 |
| 64AE FC (8,4,2, 8, 4) SV (8,4,2, 8, 1) | 4 | FC | FR1 Uma CDLC | 6.59 | 59% | 1.52 | 1.45 |
|  |  | SV |  | 6.59 | 52% | 1.73* | 1.8 |
|  | 2 | FC |  | -6.07 | 50% | 1.8 | 1.45 |
|  |  | SV |  | -6.48 | 31% | 2.9 | 2.57 |
| 8AE (1,4,2,1,1) | 4 | 1 to 1 | FR1 Uma CDLC | 14.98 | 52% | 1.73* | 1.88 |
|  |  |  | TDLC300-100 | 19.18 | 56% | 1.6 | 1.67 |
|  | 2 |  | FR1 Uma CDLC | 2.77 | 36% | 2.5* | 2.65 |
|  |  |  | TDLC300-100 | 2.33 | 34% | 2.64 | 2.65 |
| * Here eType-II codebook yield is less than Type-I codebook. |  |  |  |  |  |  |  |

From the above table, follow PMI with eType-II and Type-I codebooks perform better than random PMI in all cases.

Compared to the default 8AE configuration with rank 2, with FC scenario (using all four strongest beams) and SV scenario (using the strongest beam direction for the four subarrays), we started seeing gain of eType-II codebook over Type-I codebook whether with 64AEs or 128AEs.

However, for rank 4, we see mix behavior. That is, with FC for 64 AE eType-II codebook marginally performs better than Type-I codebook, while for FC with 128 AE shows no gain. On the other hand, for SV with 64 and 128 AEs Type-I codebook is marginally better than eType-II codebook.

In the default 8AE configuration, we do not see any gain in terms of Normalized Throughput of using eType-II over Type-I codebook irrespective of the rank. The reason could be that eType-II codebook is not benefiting from the spatial properties of the channel due to the absence of beamforming using large antenna panel.

#### 2.4.2.3	Comparison between eType-II and Type-I codebooks with sub-band PMI precoder granularity

The simulation results are summarized in Table 2.2.2.3-1 in [2] which are copied here.

Table 2.4.2.3-1 Results summary of PMI results using sub-band eType-II codebook.

| BS antenna Config. | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp [dB] | NThp. of Random PMI | Gamma eType-II codebook | Gamma Type-I codebook |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128AE FC (8,8,2, 8, 8) SV (8,8,2, 8, 2) | 4 | FC | FR1 Uma CDLC | 1.92 | 59% | 1.52 | 1.16 |
|  |  | SV |  | 0.59 | 48% | 1.875 | 1.73 |
|  | 2 | FC |  | -9.33 | 43% | 2.09 | 1.13 |
|  |  | SV |  | -9.41 | 33% | 2.72 | 1.8 |
| 64AE FC (8,4,2, 8, 4) SV (8,4,2, 8, 1) | 4 | FC | FR1 Uma CDLC | 5.01 | 53% | 1.69 | 1.47 |
|  |  | SV |  | 4.56 | 46% | 1.95 | 1.76 |
|  | 2 | FC |  | -6.26 | 49% | 1.83 | 1.47 |
|  |  | SV |  | -6.74 | 29% | 3.1 | 2.57 |
| 8AE (1, 4, 2, 1, 1) | 4 | 1 to 1 | FR1 Uma CDLC | 12.87 | 46% | 1.95 | 1.76 |
|  |  |  | TDLC300-100 | 16.54 | 52% | 1.73 | 1.76 |
|  | 2 |  | FR1 Uma CDLC | 1.56 | 30% | 3 | 2.5 |
|  |  |  | TDLC300-100 | 2.33 | 34% | 2.64 | 2.64 |

In all scenarios, eType-II codebook with sub-band PMI precoding granularity consistently outperforms Type-I codebook in terms of the normalized throughput using FR1 Uma CDLC channel.

The use of TDLC300-100 channel model, irrespective of the PMI precoder granularity, does not lead to any performance gain of e-Type-II codebook over Type-I codebook.

### 2.4.3	Other comparison simulations

In this clause, we capture our observations for simulations other than ones in WF, see clause 2.3 in [2].

#### 2.4.3.1	BS antenna variants and AAV

In this section, full connected with different antenna array configurations are compared for SU-MIMO 8Tx8Rx with 8 layers PDSCH. Some new antenna array configurations are used for simulation, see figure 2.3.1-1. The motivation is to show more information of antenna array impact on different cases, such as layer number, beam direction and channel character etc., which would improve the parameters configuration for SCM channel model derivations.

The full connected with subarray virtualization is newly raised which might be more practical regarding real product implementation. For large antenna array, digital beamforming complexity could be greatly reduced by subarray virtualization. To have better beamforming performance under higher layer transmission scenario, full connected on top of subarray ports could be a balanced way for product. In that case, it could be worthy to check the performance.

Table 2.4.3.1-1 PDSCH performance for 8Tx8Rx with 8 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI configuration | Average SNR at 70% NThp. [dB] |
| --- | --- | --- | --- | --- |
| 128AE (8,8,2, 8, 8) | Full connected | FR1 Uma CDLC | Random | 18 |
| 64AE (8,4,2, 8, 4) |  |  |  | NA |
| 64AE (4,8,2, 4, 8) |  |  |  | 20.6 |
| 128AE (8, 8, 2, 2, 1) | Full connected with Subarray virtualization |  |  | 21.4 |

More horizontal antenna elements could help for higher layer transmitted mainly on horizontal plane case.

Subarray with full connected show small degradation compared to full connected with AE.

#### 2.4.3.2 Doppler impact

The FR1 Uma CDLC model use fixed UE velocity (30km/h) which looks a bit high for 8 layers transmission. To compare with lower Doppler, the UE velocity 3km/h is compared under 128AE with full connected and subarray virtualization configuration.

Table 2.4.3.2-1 PDSCH performance for 8Tx8Rx with 8 layers

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI configuration | UE velocity [km/h] | SNR at 70% NThp. [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CW0 | CW1 | Average |
| 128AE (8,8,2, 8, 8) | FC | FR1 Uma CDLC | Random | 30 | 20.5 | 15 | 18.2 |
|  |  |  |  | 3 | 18.2 | 13.7 | 17.1 |
| 128AE (8,8,2, 8, 2) | SV |  |  | 30 | 14 | 22.5 | 20.7 |
|  |  |  |  | 3 | 20.6 | 14 | 18.8 |

Around 2dB performance difference between 30km/h and 3km/h UE velocity under FR1 Uma CDLC channel.

#### 2.4.3.3 Tx beam directions

In TR38.827, different beam directions are indicated for different layer transmission with full connected AAV. By using subarray virtualization without full connect, all layers are transmitted long same direction, such as boresight of subarray or max received power direction (if subarray have enough columns). Once the subarrays have multiple columns (separation >=1 lamda), the correlation between layer channels would become much lower even all layers use same direction.

In this section, one case with full connected and same beam directions (max received power direction) for all layers are used for comparison. Regarding very high correlation between layer channels, the performance is expected to be very poor, and the results prove it.

| BS antenna Config. (M,N,P,Ms,Ns) | Port mapping | Channel model | PMI configuration | Beam directions | SNR at 70% NThp. [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CW0 | CW1 | Average |
| 128AE (8,8,2, 8, 8) | FC | FR1 Uma CDLC | Random | 4 directions | 20.5 | 15 | 18.2 |
|  |  |  |  | Max received power direction | NA | NA | NA |
| 128AE (8,8,2, 8, 2) | SV |  |  |  | 14 | 22.5 | 20.7 |

Full connected with same direction for all layers (maximum received power direction) would cause very high inter-layer interference due to high channel correlation.

Subarray virtualization (without full connected) with same direction (maximum received power direction) for all layers would have small correlation between layers once the separation between subarray is larger enough (i.e., >=1 lamda).

### 2.4.4	Analysis and proposals

PMI preference

For a known channel model, such as FR1 Uma CDLC, all departure and arrival angles are fixed with a certain spread, so a clear PMI preference should be seen when “follow PMI” strategy is used, the received SNR is good enough and BS antenna have obvious directional radiation capability (e.g., large array, codebook matched beamwidth). In our simulations, different antenna array sizes are compared. The PMI preference extraction is normally at very high normalized throughput or SNR point to make sure the channel state at UE side is good enough.

Besides Observation 9~11 and 15~17, TDLC channel under large directional antenna array also can show preference with offsets sometimes, such as 8Tx8Rx 128AE FC case.

Comparing 8 layers and 4 layers under same PMI hypothesis by type-I codebook, spatial oversampling phenomenon could be seen in 4 layers cases which means several adjacent PMI would show similar preference, especially on 128AE FC case.

Random PMI vs. fixed PMI

Random PMI is normally used in legacy performance requirements which intend to let UE side performance independent from BS behavior. But the issue of this method for high layer reception is the inter-layer interference would be averaged in spatial domain which can’t show UE capability on interference cancelation.

Fixed PMI method use PMI with highest probability from PMI report during follow PMI simulations. That means this chosen PMI could be the one matches channel spatial power if UE channel estimation is good enough on spatial domain. In that case, relative better performance might be seen by a fixed PMI than random PMI at relatively high normalized throughput or SNR range like our simulations show.

During 8Tx8Rx 8 layers transmission, we can only see this trend easily on one CW (which is along stronger channel directions), but the opposite trend on the other CW (which is along much weaker beam directions). On the worse CW, the random PMI is the best choice which lead to a suspect on the transmission feasibility of this CW. We also  see a possibility to improve this by changing beam directions with current FR1 Uma CDLC model.

In 4Tx4Rx 4 layers cases, performance gain of fixed PMI to random PMI shows in all cases, in even in 4AE case (all AE are directional not isotropic). The reason behind might be that all 4 layers are transmitted by the strong channel directions.

From the demodulation perspective, follow PMI method might be more practical than random PMI method regarding to a fixed spatial channel model is used in the test. But using a “fixed most preferred” PMI could be a compromising way if companies think follow PMI would involve BS behavior impact. As we pointed out in Proposal 1, the whole methodology should be aligned. Otherwise, it is hard to get alignment between companies results.

Proposal 10 	Companies to check if followed PMI or fixed PMI could be considered for PDSCH performance test under SCM with high spatial preference.

AAV approaches comparison

For higher layer transmission, larger antenna array is expected to migrate spatial channel fading and enhance channel capacity. It is one of key evolutions of 5G technology. Our simulations show this trend for all cases. There are many BS antenna virtualization configurations are discussed in the study.

Full connected

TR38.827 use full connected method to utilize full antenna array for beamforming to match channel fading well. The beamforming weight for different layers would be applied to layers separately, then different layers of signal will be summed and transmitted. From the propagation perspective, full connected method would achieve the best beamforming precision and highest angle flexibility but relative high correlation at BS side. In that case, very high path correlation will be observed if same beamforming weight is applied for different layers. To support higher layer transmission, full connected method shows better performance than other approaches under most of cases if the beam directions per layer could achieve lower correlation. Unfortunately, this method is not a typical implementation which would lead to very high complexity when very large antenna is used.

Subarray virtualization

Once subarray virtualization is used, the basic intention is to reduce the digital beamforming complexity for massive MIMO antenna so all subarrays would create same radiation pattern in typical implementation. Normally, user distribution in horizontal plan would have much larger range than in vertical plan. In that case, beamforming capability in horizontal plan is much more important. But multiple columns in one subarray would reduce the flexibility to use all array for beamforming, then only one column per subarray is normally used in real product implementation. It could be fine to use multiple column subarray just for simulation, but the performance difference should be noted comparing to typical one-column subarray. If no full connected method is applied, one or multiple subarrays could be used for one layer transmission and all layers use different subarray sets. The corresponding correlation between layers depends on the distance between subarray (sets).

To support higher layer transmission with better performance and low implementation complexity, full connected with subarray virtualization could be considered. Take 4 layers transmission under 128AE antenna panel with FR1 Uma CDLC channel as an example, the strongest beam directions are along same vertical plane. In that case, all vertical AE per column in antenna panel could be allocated into one subarray and there will be 16 subarrays in horizontal directions. By full connected, 4 layers could be transmitted by two directions and two polarizations. To support 8 layers, more subarray sets could be considered to have more flexibility on vertical dimension.

Default one-to-one port mapping

The default one to one port mapping with TDL model ignores antenna array configuration but consider it by correlation matrix. Once we use it with CDL model, the channel correlation simply depends on AE separation since no beamforming can be used for transmitting along different directions. In our simulations, limited improvement is applied that directional AE is used rather than isotropic. However, it still shows unacceptable performance with one-to-one mapping on high layer transmission compared to large antenna array.

We summarize BS antenna virtualization variants used in simulations and give comments on its pros and cons in table below. Some proposals are delivered after the table.

Table 2.1-1 Observations for different BS antenna virtualizations

| AAV types | Antenna configuration (M,N,P,Ms,Ns) | Beam directions | Observations |
| --- | --- | --- | --- |
| Full connected |  | Different directions per layer | - Not practical for large antenna array - Best beamforming precision and flexibility for each layer - Possible low path correlation if good beamforming directions are chosen. - Reduce horizontal AE would impact horizontal BF performance. |
| Subarray virtualization |  | Same direction for all subarrays or different directions per subarray | - Not practical implementation - Possible but limited BF capability per subarray, e.g., (8,8,2,8,4) could generate different beam directions with large angle separation. - Low path correlation between subarrays. Might achieve good performance. |
|  |  | Boresight direction for all subarrays | - Practical for implementation - No beamforming capability if no full connected is used. - High path correlation between subarrays. |
| One-to-one mapping |  | Same direction for all subarrays | - Not practical BS antenna configuration - No beamforming capability. - High path correlation between subarrays. |
| Full connected with subarray virtualization |  | Different directions per layer | - Practical implementation for large antenna array - High BF precision and flexibility according to deployment character. - Possible low path correlation if good beamforming directions are chosen. |

Proposal 11	Consider full connected with AE or practical subarray virtualization for scenario with large BS antenna array and high layer transmission.

Channel model analysis

As observed in previous simulation results for 8Tx8Rx with 8 layers PDSCH, FR1 Uma CDLC model could show obvious performance difference between CWs which is expected to be observed in real network, but legacy TDLC300-100 show very similar performance due to no spatial fading included in the channel. But one thing should be noticed that the very bad performance on one CW might not be reasonable because network would reduce layer number to secure the average performance.

From channel model perspective, some parts might need improved or further investigated for FR1 Uma CDLC model including:

- Channel rank and angle configuration: All transmissions are fixed to same horizontal plane which might lead to lower channel rank than expectation (i.e., 8 layers).

- Tx beam directions: Current Tx beam directions are also fixed on one half of horizontal plane (0o ~ -90o) which might also cause worse performance for layers transmitted by weak beams.

- Codebook configuration: Current only (N1,N2,O1,O2) = (4,1,4,1) in type-I codebook is used, but (N1,N2,O1,O2) = (2,2,4,4) can be also considered if ZoD/ZoA could be adjusted to other directions.

From practical scheduling perspective, FR1 Uma CDLC model have limitations to support 8 layers due to very bad channel conditions on one CW.

Proposal 12	Companies furtherly investigate how to improve channel capability on supporting higher layer transmission.

## 2.5	Requirements and Other

#### Definition of New Requirements

Contributions welcome at RAN4#114

Some requirements, such as SU-MIMO PDSCH with 8 layers, SU-MIMO PMI with 4 layers, were proposed by some companies for requirement definition with FR1 Uma CDLC model. Based on above observations, it seems existing FR1 Uma CDLC model configuration seems have obvious limitation on 8 layers support. For PMI tests, there is no obvious gamma value difference between FR1 Uma CDLC model and TDLC300-100 model. Thus, more investigations would be needed on current model improvement. It looks too early to consider requirement for demodulation in Rel-19.

Proposal 13	Don’t consider demodulation requirements with SCM in Rel-19.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Theoretically, a fixed most preferred PMI test could reflect UE channel estimation performance against a priori channel information through simulations.

Observation 2	The SU-MIMO PDSCH with 4 layers shows better performance with fixed PMI than random PMI, but 8 layers show worse performance with fixed PMI.

Observation 3	The CW transmitted along current 3rd and 4th strongest beam directions with followed PMI under FR1 Uma CDLC channel would lead to much worse performance than random PMI.

Observation 4	No obvious performance difference between IRC and LMMSE receiver for SU-MIMO PDSCH with random PMI configuration.

Observation 5	The MU-MIMO scenario focus more on cell/system level throughput which would be dominated by network scheduling algorithm more than UE capability of interference cancelation.

Observation 6	The spatial domain distribution of interference might not be important due to random PMI is used in current MU-MIMO IRC test setup.

Observation 7	TE vendors seem fine with CDL models in TR38.827 which have up to 24 clusters.

Observation 8	The minimum delay separation between different clusters could be <1ns in FR1 Uma CDLC model in TR38.827.

Observation 9	It could be possible to use per-channel correlation matrix rather than per-tap correlation matrix together with reduced number of taps to simplify TDL extension model.

Observation 10	It will be very challenging to evaluate methodology that TDL model with per-tap correlation matrix in Rel-19 timeline.

Observation 11	As for method in R4-2415382,

	Pros: easily adjusted beam angle and power, reduce correlation matrix impact among beams and the test setup could be simple.

	Cons: treating beams as independent channels with same channel model which seems not so practical for SU-MIMO; departure and arrival beam directions are evenly separated.

Observation 12	As for method in R4-2419895:

	Pros: easily adjusted beam angle and power, reduce correlation matrix impact among beams and the test setup could be simple. One channel for a link.

	Cons: departure and arrival beam directions are evenly separated. Power of beams are evenly split. No general procedure description.

Observation 13	The sum of received signal power is the same no matter beamforming is used or not if beamforming weight and channel power are normalized.

Observation 14	The power normalization factor should be 1MeNe in (7.2-8) in TR38.827.

Observation 15	With larger antenna array (128 AE), full connected method could show obvious PMI preference on both CDLC and TDLC models, but TDLC model also show bias compared to CDLC. By subarray virtualization, the preferred PMI would be biased a bit on CDLC model, but TDLC model almost show no clear preference.

Observation 16	Reducing antenna array (64 AE), especially on horizontal direction, would greatly impact the PMI preference due to less beam power and direction precision.

Observation 17	With 8AE antenna without full connection, both CDLC and TDLC model show no clear PMI preference.

Observation 18	Compared to random PMI, fixed PMI would increase performance of CW sent along strong channel directions but reduce performance of CW sent along weak channel directions on all antenna configurations. The averaged throughput is normally degraded.

Observation 19	Reducing antenna array from 128AE to 64AE, especially on horizontal direction, would greatly degrade the performance both per CW and average. Random PMI show similar per CW performance.

Observation 20	With 8AE antenna without full connection, random PMI show similar performance on both CW.

Observation 21	By using same PMI hypothesis as 8 layers, large antenna array (128AE) with 4 layers transmission shows less PMI preference on specific PMI value due to oversampling on spatial domain. But the preference could be seen on a “PMI cluster”.

Observation 22	Reducing antenna array from 128AE to 64AE, the obvious PMI preference could be seen especially on full connected method.

Observation 23	With 8AE antenna, CDLC model can show obvious PMI preference than TDLC model.

Observation 24	Fixed PMI performance can show better performance than random PMI as expected for all antenna configurations.

Observation 25	When antenna array is reduced to 64AE, performance of subarray virtualization is better than full connected.

Observation 26	The full connected AAV approach would get small gamma value compared to subarray virtualization approach because subarray virtualization can achieve better performance under follow PMI but worse performance under random PMI.

Observation 27	Similar gamma values are achieved between 4 layers and 2 layers.

Observation 28	With legacy 8 AE BS antenna configuration, TDLC channel could achieve similar performance as FR1 Uma CDLC model.

Observation 29	From the above table, follow PMI with eType-II and Type-I codebooks perform better than random PMI in all cases.

Observation 30	Compared to the default 8AE configuration with rank 2, with FC scenario (using all four strongest beams) and SV scenario (using the strongest beam direction for the four subarrays), we started seeing gain of eType-II codebook over Type-I codebook whether with 64AEs or 128AEs.

Observation 31	However, for rank 4, we see mix behavior. That is, with FC for 64 AE eType-II codebook marginally performs better than Type-I codebook, while for FC with 128 AE shows no gain. On the other hand, for SV with 64 and 128 AEs Type-I codebook is marginally better than eType-II codebook.

Observation 32	In the default 8AE configuration, we do not see any gain in terms of Normalized Throughput of using eType-II over Type-I codebook irrespective of the rank. The reason could be that eType-II codebook is not benefiting from the spatial properties of the channel due to the absence of beamforming using large antenna panel.

Observation 33	In all scenarios, eType-II codebook with sub-band PMI precoding granularity consistently outperforms Type-I codebook in terms of the normalized throughput using FR1 Uma CDLC channel.

Observation 34	The use of TDLC300-100 channel model, irrespective of the PMI precoder granularity, does not lead to any performance gain of e-Type-II codebook over Type-I codebook.

Observation 35	More horizontal antenna elements could help for higher layer transmitted mainly on horizontal plane case.

Observation 36	Subarray with full connected show small degradation compared to full connected with AE.

Observation 37	Around 2dB performance difference between 30km/h and 3km/h UE velocity under FR1 Uma CDLC channel.

Observation 38	Full connected with same direction for all layers (maximum received power direction) would cause very high inter-layer interference due to high channel correlation.

Observation 39	Subarray virtualization (without full connected) with same direction (maximum received power direction) for all layers would have small correlation between layers once the separation between subarray is larger enough (i.e., >=1 lamda).

Observation 40	From practical scheduling perspective, FR1 Uma CDLC model have limitations to support 8 layers due to very bad channel conditions on one CW.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The principle of how to get the fixed preferred PMI should be clarified.

Proposal 2	Companies should make it clear about the MU-MIMO SCM derivation purpose at the first, and then should address the justification and benefit of this derivation comparing to SU-MIMO SCM derivation.

Proposal 3	Consider following metrics for simulation results or channel model comparison.

	Comparison among companies’ results for alignment.

o	SU-MIMO PDSCH

	SNR at 70% normalized and combined average throughput. For 8 layers, SNR at 70% normalized per CW throughput and the gap between CWs.

	Preferred PMI based on PMI histogram for fixed PMI.

o	SU-MIMO PMI

	SNR at 90% normalized throughput with followed PMI.

	Gamma value to random PMI configuration.

	Comparison among channel model methodologies.

o	Channel model availability.

o	Practicability.

o	Performance expectation.

o	Testability.

Proposal 4 	Invite TE vendors give confirmation on the minimum delay separation between clusters.

Proposal 5	Investigating the derivation of per-channel correlation matrix based on per-tap correlation matrices if TDL extension and tap combination are used for SCM.

Proposal 6	Companies to check and bring ideas to TDL model based methodologies if possible.

Proposal 7	Consider Option 1 for SU-MIMO PMI performance verification.

Proposal 8	Consider Option 2 for model comparison and simulation result alignment only.

Proposal 9	No need to consider normalization factor because AAV and beamforming weights are generally power normalized.

Proposal 10 	Companies to check if followed PMI or fixed PMI could be considered for PDSCH performance test under SCM with high spatial preference.

Proposal 11	Consider full connected with AE or practical subarray virtualization for scenario with large BS antenna array and high layer transmission.

Proposal 12	Companies furtherly investigate how to improve channel capability on supporting higher layer transmission.

Proposal 13	Don’t consider demodulation requirements with SCM in Rel-19.

# References

[1] 	R4-2419782, Way Forward for [113][320] FS_NR_demod_SCM, Nokia, RAN4#113

[2]	R4-2501788, Simulation results for Rel-19 SCM for demodulation, Ericsson, RAN4#114

[3]	R4-2418043, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#113