3GPP TSG-RAN WG4 Meeting # 114							R4-2501794

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.24.2.3

Source: 	OPPO

Title: 	RF requirements for CW

Document for:	Approval

- Introduction

The new WID [1] for ambient IOT has been approved in the RAN#106 meeting. In this paper we try to give discussion on the RF requirements for device 1 as per the WID request and also based on the outcome of the study item as captured in the TR 38.769.

- Discussion

During the Rel-19 study item phase, it has been discussed the CW cancellation capability and captured in the TR 38.769. However further requirement for the CW has not been discussed. Some RF parameter for co-existence study has been captured in the TR and shown below.

Table 6.6.2.2-4: CW RF parameters

| CW parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power (dBm) | If UL spectrum is used, UE Tx power is assumed, i.e. 23dB If DL spectrum is used, A-IoT reader Tx power is assumed, i.e. 33 dBm | Intermediate UE Tx power is assumed. |
| Antenna gain | Same as A-IoT reader | Same as intermediate UE |

.

Only TX power has been considered but the in-band/out-of-band emission is not considered during the simulation stage.

Observation 1：Only TX power has been considered for CW in the co-existence study.

While in clause 6.8 Characteristics of carrier-wave waveform of the TR, it has captured the CW characteristics and captured the potential requirement as:

The following CW waveform characteristics which would need control of the CW node(s) are identified:

-	When CW is transmitted or not transmitted

-	Transmission Power

-	Frequency resources, e.g. frequency location(s) for waveform (e.g. waveform 1, waveform 2)

Other CW waveform characteristics which would need control of the CW node(s), if any, can be further identified in a later stage.

Observation 2: The CW characteristics identified by RAN1 is the transmission time, transmission power and transmission frequency.

Furthermore, in TR 38.769 sub-clause 6.6.3.4 the impact from CW to NR has captured the reason why the CW impact is not modelled.

Observation 3: During the co-existence study phase, RAN4 doesn’t model the CW impact.

6.6.3.4	Impact from CW to NR

In 6.6.3.2, the SINR definition includes the impact from CW to D2R which is treat as intra-system interference. However, Whether and how the CW will influence the NR system also needs to be studied. In [156], the impact of CW signal under different assumptions are investigated and the evaluation results are shown in figure 6.6.3.4-1.

Figure 6.6.3.4-1: Impact from CW to NR system [156]

The results shows that when the CW is orthogonal with NR subcarrier, i.e., α=0, the influence of interference is much lower and also easier to be removed after FFT, only if the power of CW interference does not exceed the dynamic range of receiver.

Based on the analysis above, the impact from CW to NR is not modelled in the co-existence evaluation for simplicity.

With the above observations, we believe the most important characteristics for CW is the transmission power and corresponding frequency accuracy, which guarantee the CW is generated at the correct frequency.

Proposal: To define the transmission power and frequency accuracy requirement for CW.

3	Conclusions

In this paper, we give discussion on the CW RF requirement, the observations and proposals are as below:

Observation 1：Only TX power has been considered for CW in the co-existence study.

Observation 2: The CW characteristics identified by RAN1 is the transmission time, transmission power and transmission frequency.

Observation 3: During the co-existence study phase, RAN4 doesn’t model the CW impact.

Proposal: To define the transmission power and frequency accuracy requirement for CW.

4 References

[1] RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR