3GPP TSG-RAN WG4 Meeting # 114							R4-2501795

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.24.2.2

Source: 	OPPO

Title: 	RF requirements for device 1

Document for:	Approval

- Introduction

The new WID [1] for ambient IOT has been approved in the RAN#106 meeting. In this paper we try to give discussion on the RF requirements for device 1 as per the WID request and also based on the outcome of the study item as captured in the TR 38.769.

- Discussion

During the Rel-19 study item phase, the UE RF requirement for ambient device 1 has been extensively discussed and captured in TR 38.769. The big TP [2] to TR has been agreed in the RAN4#113 meeting.

Firstly, for the transmitted power, it has been captured in the TR as:

Transmitter power

For device 1, D2R backscattering is transmitted on a carrier wave provided externally, backscattering power and power backscattering loss are potentially needed to be defined to ensure the coverage range between reader and device. Maximum output power and other transmit output power requirements are potentially not needed.

The minimum backscattered modulated power can be specified for device 1. The backscatter power is the amount of power reflected back from the device when a successful command sequence is completed at the device at a specific sensitivity level.

The backscatter loss due to the modulation can be defined as scattering cross-section or modulation efficiency in equation (4). From equation (4), it can be observed that the modulation efficiency is defined as the ratio of the power radiated by the device to the power density incident on its antenna.

As per the TR discussion, there might be two requirements as the backscattering power and the power backscattering loss. These two requirements are to verify the device uplink coverage and the capability backscattering. When look to the RF ID regulation, we see there is only one output power requirement defined. The requirement is based on a certain test range between the CW and device and to test the exact output power level. From our point of view, this kind of requirement has already included both the exact output power level and the backscattering power loss.

Observation 1: The RF ID regulation output power has already considered the exact backscattering power level and the backscattering power loss.

Proposal 1: Only one RF output power requirement which is similar to RF ID output power will be defined.

For transmitted signal quality, the conclusion in TR is captured as below.

Although it has been recommended that the frequency error is not needed since the device 1 will not have its own LO, still we believe the frequency error is a quite necessary requirement for the device to backscatter the CW properly. Furthermore, small frequency shift is under discussion in RAN1 and how precise the frequency shift can reach is absolutely a necessary requirement to be checked.

Proposal 2: To define the frequency error requirement and also consider the RAN1 conclusion on the small frequency shift.

Transmitted signal quality

Transmitted signal quality includes frequency error, carrier leakage, in-band emission and EVM.

As D2R backscattering is transmitted on a carrier wave provided externally, the requirements for frequency error, carrier leakage and in-band emission are potentially unnecessary.

The waveform for D2R could be OOK or BPSK. OOK signal is not mapped by the legacy constellation. EVM may not be suitable for D2R OOK waveform, and whether EVM testing is feasible for BPSK waveform needs to be discussed.  Requirement to guarantee the modulation quality is potentially needed. How to define modulation quality requirements will be discussed in WI.

For the occupied bandwidth, the D2R channel bandwidth will be defined in RAN1 as stated and captured below. For the SEM and spurious emission, as from RF ID regulation part, the spurious emission is defined. For the SEM, since currently there is only one device to one reader and hence the influence of different devices within the reader bandwidth range is not considered. In such case, we believe the SEM requirement is not needed.

Output RF spectrum emissions

Output RF spectrum emissions includes occupied bandwidth, out of band emission (SEM, ACLR), spurious emissions and transmit intermodulation.

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Occupied bandwidth requirement is potentially needed. The specific value is dependent on the physical layer design of D2R, which has not yet been finalized at this stage.

SEM and spurious emissions are related to co-existence study. Whether to define these requirements need to be discussed in WI. Based on the co-existence study, ACLR is not needed.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. However, device1 is low complexity and low cost, and has no active components at the transmitter. SEM, spurious emissions may cover this function and requirement. Transmit intermodulation requirement is unnecessary.

Proposal 3: To define the spurious emission requirement based on the regulation.

Proposal 4: The SEM requirement is not needed for device 1.

For the RX part, we believe the simplest requirement should be defined for device 1. Hence we propose only the REFSENS requirement to be defined so that the coverage is guaranteed.

Proposal 5: For RX requirement, only REFSENS to be defined.

3	Conclusions

In this paper, we give discussion on the RF requirement for device 1, the observations and proposals are as below:

Observation 1: The RF ID regulation output power has already considered the exact backscattering power level and the backscattering power loss.

Proposal 1: Only one RF output power requirement which is similar to RF ID output power will be defined.

Proposal 2: To define the frequency error requirement and also consider the RAN1 conclusion on the small frequency shift.

Proposal 3: To define the spurious emission requirement based on the regulation.

Proposal 4: The SEM requirement is not needed for device 1.

Proposal 5: For RX requirement, only REFSENS to be defined.

4 References

[1] RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2] R4-2420372	TP to TR38.769: RAN4 part for Ambient-IoT, CMCC