3GPP TSG-RAN WG4 Meeting #113							R4-2501797

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.5.2.2

Source: 	OPPO

Title: 	On UE capability and behavior for type 4a and 4b

Document for:	Approval

- Introduction

In RAN4#113 meeting, the UE capability, behaviour and the BS signalling for type 4a and 4b has been fully discussed and a WF[1] has been agreed. In this paper we give further discussion on these topics.

Discussion

The WF agreement have been captured as below.

<Issue 2-2-2:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

Way forward:

- For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

- For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

<Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

Way forward:

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- FFS in the next meeting.

  - When maxMIMO-layers = 4

- FFS in the next meeting.

NOTE: maxMIMO-layers means the existing BS signaling.

Firstly, for the BS signalling, although it has been agreed that one new BS signalling is needed from type 4 to type 1 4L or type 1 larger than 4L per CC for both NR-CA and EN-DC, respectively, we have to keep in mind that the type 1 larger than 4 layer such as 6 layer or 8 layer is optional feature as agreed in previous meeting.

Observation 1: It has been agreed that the type 1 more than 4 layer is an optional capability for UE.

In such case, we might need to tell RAN2 about this BS signalling that 6 layer and 8 layer type 1 is optional. We can leave RAN2 for further clarification when defining such BS signalling and that the base station will not signal the UE for the capability that it doesn’t have.

Proposal 1: To also inform RAN2 that the 6 layer and 8 layer type 1 is optional capability for UE.

Secondly, for UE behaviour, it has been agreed that when maxMIMO-layers=2, the Rel-18 behaviour should be followed in Rel-19. But for maxMIMO-layers = 4 and greater than 4, still further discussion is needed.

For the case that maxMIMO-layer = 2 in Rel-18, a BS signalling is introduced to differentiate the collocated and non-collocated scenario and the “nonCollocatedTypeNR-CA-r18” has been captured as below.

– CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels

with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the

configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::= SEQUENCE {

nonCollocatedTypeNR-CA-r18 ENUMERATED { true } OPTIONAL, -- Need R

uplinkTxSwitchingMoreBands-r18 SetupRelease { UplinkTxSwitchingMoreBands-r18 } OPTIONAL -- Need M

]]

In Rel-19, when maxMIMO-layer = 4, UE can work under non-collocated and collocated scenario with 4 layers. In this case, similar to type 2 UE, a further NW signalling is needed to differentiate the collocated and non-collocated deployment as well as the requirement. As discussed in the first part, the BS signalling has been agreed. So the BS signalling +  the maxMIMO-layer=4 can tell the UE is type 1 or type 4.

One more point needs to be emphasized is that as also discussed in first part, the BS signalling will differ UE from not only 4 layer but also 6 layer and 8 layer. Hence similar approach for maxMIMO-layer = 4 can also apply which is both the maxMIMO-layer and the BS signalling together to define the UE behavior.

Proposal 2: For both the case maxMIMO-layer =4 or greater than 4, the new BS signalling as well as the maxMIMO-layer together define the UE behaviour.

Similar as Rel-18, the detail of BS signalling design is left to RAN2.

3	Conclusions

In this paper, we give further discussion on the UE behaviour and network signalling and the observations and proposals are as below:

Observation 1: It has been agreed that the type 1 more than 4 layer is an optional capability for UE.

Proposal 1: To also inform RAN2 that the 6 layer and 8 layer type 1 is optional capability for UE.

Proposal 2: For both the case maxMIMO-layer =4 or greater than 4, the new BS signalling as well as the maxMIMO-layer together define the UE behaviour.

4 References

[1] R4-2420399 	WF on non-collocated intra-band EN-DC and NR-CA, KDDI