3GPP TSG-RAN WG4 Meeting #114                                      R4-2501803

Athens, Greece, February 17th – February 21st, 2025

Source: 	Huawei, HiSilicon

Title: 	On RF requirements of fragmented carriers

Agenda Item:	7.6.3

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104. During RAN4#113, it was agreed to use a link budget template to provide the DL performance evaluation.

In this contribution, we provide further evaluation results based on the agreements.

Discussion

REFSENS

As discussed in previous meetings, the REFSENS performance of Single Rx RF Chain will be impacted by the Tx leakage when the UL signal is close to the DL carrier as shown in below Figure 2.1-1.

Figure 2.1-1 REFSENS on band n3 with Tx leakage

Taking band n3 as an example, the estimated SNR on PCC and SCC could be derived as in Table 2.1-1 based on the agreed link budget format using different assumption of the key parameters. To achieve the -1dB demodulation threshold, extra 2.5~7.5dB relaxation would be required on PCC, and 1~4.1dB relaxation is required on SCC.

Table 2.1-1 Estimated SNR on band n3 with a strong Tx leakage

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 | 50 | 55 | 55 |
| LPF attenuation @Tx Fc |  | 12 | 12 | 12 | 12 |
| NF increase due to AGC adjustment |  | 5 | 5 | 2 | 2 |
| ADC (12bit) |  | -12dBFS | -6dBFS | -12dBFS | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -8.5 | -6.8 | -4.8 | -3.5 |
|  | REFSENS (SCC) | -5.1 | -3.9 | -2.7 | -2 |

Observation 1: When the UL signal is close to DL carrier, the REFSENS performance is impacted by the Tx leakage. The estimated SNR varies with different assumptions of key parameters, including the duplexer isolation @Tx, the LPF attenuation @Tx, the NF increase due to AGC adjustment and the ADC setting.

Observation 2: On band n3, extra 2.5~7.5dB relaxation would be required on PCC, and 1~4.1dB relaxation is required on SCC to achieve the -1dB demodulation threshold with the existence of a strong Tx leakage.

Another example band n7 is also studied. The TX–RX frequency separation on band n7 is larger than that of band n3, therefore the LPF is expected to provide larger attenuation on the Tx leakage.

Figure 2.1-2 REFSENS on band n7 with Tx leakage

The estimated SNR on PCC and SCC could be derived as in Table 2.1-2 based on the same logic. To achieve the -1dB demodulation threshold, extra 0~2.1dB relaxation would be required on both PCC and SCC.

Table 2.1-2 Estimated SNR on band n7 with a weak Tx leakage

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 | 50 | 55 | 55 |
| LPF attenuation @Tx Fc |  | 30 | 30 | 30 | 30 |
| NF increase due to AGC adjustment |  | 2 | 2 | 0 | 0 |
| ADC (12bit) |  | -12dBFS | -6dBFS | -12dBFS | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -3.1 | -3.0 | -1.1 | -1.0 |
|  | REFSENS (SCC) | -3.1 | -3.0 | -1.1 | -1.0 |

Observation 3: On band n7, extra 0~2.1dB relaxation would be required on PCC and SCC to achieve the -1dB demodulation threshold with the existence of a weak Tx leakage.

The REFSENS performance is highly related to the LPF attenuation, which is dependent on the gap between the UL signal and the DL carrier. The usual RAN4 process would be to evaluate and specify the required MSD per each band. An alternative way of specifying a general mapping table between MSD value and the gap could be considered for simplifying the requirement derivation.

Proposal 1: RAN4 to consider an alternative way of specifying a general mapping table between MSD value and the gap between the UL signal and the DL carrier instead of evaluating the MSD per each band.

ACS/IBB

According to the WF in last RAN4 meeting, the evaluation of ACS/IBB test cases need to consider the location of Rx image of the in-gap interference. In this clause we provide the evaluation results for both cases.

Rx Image does not overlap with the wanted carrier

When the Rx image doesn’t overlap with wanted carrier, the ACS/IBB performance is evaluated for band n3 and n7 representing the cases with a strong or weak Tx leakage respectively as in Table 2.1.1-1 and Table 2.1.1-2. The test case requiring the largest relaxation is highlighted in red font.

Table 2.1.1-1 Estimated SNR on band n3 with a strong Tx leakage

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 | 50 | 55 | 55 |
| LPF attenuation @Tx Fc |  | 12 | 12 | 12 | 12 |
| NF increase due to AGC adjustment |  | 5 | 5 | 2 | 2 |
| ADC (12bit) |  | -12dBFS | -6dBFS | -12dBFS | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -8.5 | -6.8 | -4.8 | -3.5 |
|  | REFSENS (SCC) | -5.1 | -3.9 | -2.7 | -2 |
|  | ACS case 1 (PCC) | -1.7 | -1.5 | -1.2 | -1.1 |
|  | ACS case 1 (SCC) | 2.7 | 2.9 | 3.2 | 3.3 |
|  | ACS case 2 (PCC) | -1.0 | -1.0 | -1.0 | -1.0 |
|  | ACS case 2 (SCC) | -1.0 | -1.0 | -1.0 | -1.0 |
|  | IBB case 1 (PCC) | -4.4 | -3.3 | -2.3 | -1.8 |
|  | IBB case 1 (SCC) | -0.6 | 0.3 | 1.1 | 1.4 |
|  | IBB case 2 (PCC) | -4.4 | -3.3 | -2.4 | -1.8 |
|  | IBB case 2 (SCC) | -0.6 | 0.3 | 1 | 1.4 |

Table 2.1.1-2 Estimated SNR on band n7 with a weak Tx leakage

|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| Duplexer isolation @Tx Fc |  | 50 | 50 | 55 | 55 |
| LPF attenuation @Tx Fc |  | 50 | 50 | 50 | 50 |
| NF increase due to AGC adjustment |  | 2 | 2 | 0 | 0 |
| ADC (12bit) |  | -12dBFS | -6dBFS | -12dBFS | -6dBFS |
| SNR (dB) | REFSENS (PCC) | -3.0 | -3.0 | -1.0 | -1.0 |
|  | REFSENS (SCC) | -3.0 | -3.0 | -1.0 | -1.0 |
|  | ACS case 1 (PCC) | -1.1 | -1.1 | -1.0 | -1.0 |
|  | ACS case 1 (SCC) | -1.1 | -1.1 | -1.0 | -1.0 |
|  | ACS case 2 (PCC) | -1.0 | -1.0 | -1.0 | -1.0 |
|  | ACS case 2 (SCC) | -1.0 | -1.0 | -1.0 | -1.0 |
|  | IBB case 1 (PCC) | -1.6 | -1.6 | -1.0 | -1.0 |
|  | IBB case 1 (SCC) | -1.6 | -1.6 | -1.0 | -1.0 |
|  | IBB case 2 (PCC) | -1.7 | -1.6 | -1.1 | -1.0 |
|  | IBB case 2 (SCC) | -1.7 | -1.6 | -1.1 | -1.0 |

Observation 4: Generally, REFSENS is the most demanding test case. If an extra relaxation is needed for REFSENS, the relaxation would also be enough for ACS/IBB test cases.

Observation 5: If no relaxation is needed for REFSENS, a minor relaxation (~1dB) might be needed for ACS/IBB test cases, especially IBB case 2.

Proposal 2: RAN4 to evaluate and specify the requirements of ACS/IBB for Single RF Rx Chain without considering the Tx leakage. The impact of Tx leakage is reflected in requirements of REFSENS, which will implicitly apply to ACS/IBB test cases.

Rx Image overlaps with the wanted carrier

When the Rx image overlaps with the wanted carrier, SNR of victim carrier could be roughly estimated by

SNR ≈ PWanted – (PBlocker - IRR)

where PWanted and PBlocker are receive power of the wanted carrier and the in-gap blocker interference, and IRR is the Image Rejection Ratio, which is assumed as 25dB in RAN4.

The evaluation in [2] shows the PSD difference between cells of different operators could be up to 20dB, including 10dB difference due to antenna orientation, and 10dB difference due to heterogeneous network deployment.

The largest possible power difference PBlocker – PWanted needs also consider the bandwidth difference between the wanted carrier and the interference carrier. At RAN4#112, the typical value of gap between fragmented carriers are provided by operators as below:

- n2/n25: 10 MHz, 15 MHz, 25 MHz

- n7: 20 MHz and 30 MHz

- n66: 15 MHz

Therefore, the maximum power difference PBlocker – PWanted is 20dB+10*log10(30MHz/5MHz) ≈ 28dB. As a result, the worst SNR could be estimated as -3dB, which could be considered as a corner case.

Observation 6: With 20dB PSD difference, the maximum power difference between in-gap interference and wanted carrier is 28dB. As a result, the worst SNR of wanted carrier due Rx image is -3dB.

If the power difference between operators is further limited as lower than 25dB, then the -1dB demodulation threshold could be achieved even with the existence of Rx image.

Proposal 3: For inter-operator collocated scenario, assume the power difference between operators as lower than 25dB. The power difference means the difference between the overall receive power within the gap and the lower power of two wanted carriers.

With the constraint in Proposal 3, there will be no Rx performance impact observed due to Rx image. Specification update is not needed.

Proposal 4: Specification update is not needed for Rx image scenario.

Conclusion

In this contribution we evaluated the RF performance based on previous agreements. According to the analysis, we have the following proposals:

Observation 1: When the UL signal is close to DL carrier, the REFSENS performance is impacted by the Tx leakage. The estimated SNR varies with different assumptions of key parameters, including the duplexer isolation @Tx, the LPF attenuation @Tx, the NF increase due to AGC adjustment and the ADC setting.

Observation 2: On band n3, extra 2.5~7.5dB relaxation would be required on PCC, and 1~4.1dB relaxation is required on SCC to achieve the -1dB demodulation threshold with the existence of a strong Tx leakage.

Observation 3: On band n7, extra 0~2.1dB relaxation would be required on PCC and SCC to achieve the -1dB demodulation threshold with the existence of a weak Tx leakage.

Proposal 1: RAN4 to consider an alternative way of specifying a general mapping table between MSD value and the gap between the UL signal and the DL carrier instead of evaluating the MSD per each band.

Observation 4: Generally, REFSENS is the most demanding test case. If an extra relaxation is needed for REFSENS, the relaxation would also be enough for ACS/IBB test cases.

Observation 5: If no relaxation is needed for REFSENS, a minor relaxation (~1dB) might be needed for ACS/IBB test cases, especially IBB case 2.

Proposal 2: RAN4 to evaluate and specify the requirements of ACS/IBB for Single RF Rx Chain without considering the Tx leakage. The impact of Tx leakage is reflected in requirements of REFSENS, which will implicitly apply to ACS/IBB test cases.

Observation 6: With 20dB PSD difference, the maximum power difference between in-gap interference and wanted carrier is 28dB. As a result, the worst SNR of wanted carrier due Rx image is -3dB.

Proposal 3: For inter-operator collocated scenario, assume the power difference between operators as lower than 25dB. The power difference means the difference between the overall receive power within the gap and the lower power of two wanted carriers.

Proposal 4: Specification update is not needed for Rx image scenario.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- RP-2415804, “On methods for reducing the number of UE Rx chain”, Huawei, RAN4#112bis Meeting, October 2024

- R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, RAN#113 Meeting, November 2024