3GPP TSG-RAN WG4 Meeting#114                                R4-2501883

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.23.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

- Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz) Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. - Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported - Issue 1-1-4: Transmission bandwidth configuration for SBFD Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues. - Issue 1-1-5: DL/UL subband configurations support for SBFD Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case. Sub-topic 1-2: Other general issues for SBFD requirements - Issue 1-2-1: BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

Discussion

- Issue 1-1-3: Specification implementation of operating bands support for SBFD

Agreement:

- The following TP is adopted for clause 5.2 in TS 38.104

  - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

- FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

- In the last RAN4 meeting, there is one open issue for the necessity of additional note to indicate the NR TDD bands not applicable to SBFD operation. From our understanding, the exceptional NR TDD bands not supporting SBFD operation has already clarified in the main body, therefore we don’t see the additional or duplicated information to be captured in the band table 5.2-1.

- Proposal 1: propose not to specify the additional note in band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

- Issue 1-3-1: Clarification on declaration and flexibility

Agreement:

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - FFS on how to capture this in the specification.

  - This restriction shall at least be applied to FR1 WA BS

  - FFS other BS classes

    - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations.

      - FFS how to capture this in the specifications

- Regarding the declaration and flexibility, it is still open on how to capture it in the specification. From our understanding, this is up to the additional manufacture declaration for SBFD operation in the TS38.141-2. BS capable of SBFD operation need to declare its supported SBFD configuration similar as legacy BS indicating the supported channel bandwidth (e.g. D.7). In addition, regarding the test signal used to build testing configuration for SBFD operation, there would be two different approach:

- FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

- For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

Table 4.7.2.1-1: Signal to be used to build NR TCs for BS type 1-H and BS type 1-O

| Operating band characteristics |  | FDL_high – FDL_low < 100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note) | 20 MHz (Note) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |  |
| NOTE: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

- Proposal 2: regarding the declaration and flexibility for SBFD operation, we propose to define the conformance testing as following way:

- FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

- For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

- Issue 1-3-2: DL/UL subband configurations support for SBFD

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

- Regarding the maximum specific UL sub-band size, it is still open. From our understanding, we don’t see the necessity to restrict to 2 specific UL sub-band. From the standardization perspective, to specify the maximum UL sub-band size as 3 could leave more flexibility for manufacture declaration if it is capable to support.

- Proposal 3: regarding the DL/UL subband configurations support for SBFD, to set the maximum UL sub-band size as 3.

- Issue 1-3-6: Other restriction on DL subband size

Way Forward:

- FFS the following restriction on DL subband size:

  - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

  - FR2 minimum CBW to support SBFD is 100MHz.

- The above proposal is mainly motivated by sub-band DL symbols overlapping with SSB symbols in the time domain, if this is overlapped, then such minimum channel bandwidth restriction is needed, however SBFD symbols could also overlap with other DL symbols. In short, we don’t see the necessity to mandate such kind of restriction for DL sub-band. This should be totally up to the manufacture declaration.

- Proposal 4: propose not to mandate the additional restriction for DL sub-band band.

- Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way Forward:

- FFS the RF requirement impact, if any, due to UL resource muting introduced in RAN1.

- The above proposal might have the impacts on RAN4 MPR evaluation if UL resource muting introduced in RAN1.

- Proposal 5: propose to further discuss the corresponding MPR requirement for UL resource muting solution if specified by RAN1.

- Issue 2-1-3: Location of transient period between SBFD and non-SBFD

Based on the discussions in the previous meetings, the location of transient period between SFBD and non-SBFD symbols is not determined yet and the necessity of this requirement is also debatable so far. At least from our understanding, the transition period should be located within SBFD symbols instead of non-SBFD symbols to avoid the negative impacts on the legacy UE reception. It is well known that, during the BS Tx OFF-ON or ON-OFF, the transmitter signal quality within the transition period cannot be guaranteed properly. Usually from UE receiver demodulation perspective, the corrupted soft information should be dropped before feeding into the LPDC/Polar decoder. For the SBFD aware UE, UE could know the location of the transition period by the TDD DL-UL configuration and additional SBFD configuration in RRC inf, then the corrupted soft information could be dropped within the estimated window length and baseband performance could be ensured somehow. If the transition period is located within the non-SBFD symbols or across the non-SBFD symbols and SBFD symbols, then the performance degradation for non-SBFD symbols will be expected. In short, we propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Figure 2.1.3-1. the location of transition period between SBFD and non-SBFD symbols

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Conclusions

In this contribution, we want to share some further views on general aspects for SBFD BS and proposals/observations are made as following:

- Proposal 1: propose not to have the additional note in band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

Proposal 2: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

- Proposal 3: regarding the DL/UL subband configurations support for SBFD, to set the maximum UL sub-band size as 3.

- Proposal 4: propose not to mandate the additional restriction for DL sub-band band.

- Proposal 5: propose to further discuss the corresponding MPR requirement for UL resource muting solution if specified by RAN1.

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General, Approved.

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General

- R4-1801031, WF on NR BS RF FRCs, approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.