3GPP TSG-RAN WG4 Meeting #114	R4-2501962

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	On RRM Requirement Impacts of AIML Mobility

Agenda item:	7.21.2

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#113 in the scope of Rel-19 SI ‎[1]. Agreements achieved during this meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements, as listed below:

- Performance metrics

- Impact on beam/cell level measurements

- Impacts of measurement errors

- Impacts on Core requirements

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

Generalization, test coverage, and testing setup related aspects are discussed in our accompanying paper.

Discussion

Performance metrics

This section addresses open issues related to the performance metrics for AI/ML-based measurement prediction. From the previous meeting, following agreement and open issue were identified regarding the definition of absolute and relative RSRP accuracy, which requires further discussion ‎[2].

Absolute RSRP Accuracy

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

In this regard, in RAN4 AI/ML for beam management WI following agreement has made regarding the definition of the definition of the ground truth of L1-RSRP for FR1 ‎[3].

Agreements in RAN4 #111 AI/ML for beam management meeting Agreement: The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP FFS on impact of multiple-AoA test setup FFS on the channel condition FFS on whether the ground truth will be changing or not - Other solutions are not precluded

The ground truth for the predicted FR2 L1-RSRP is the ideal measurement of RSRP on the predicted Tx beam. In our perspective, for FR2 and the ground truth definition of L3-RSRP prediction, RAN4 can align with the agreed-upon ground truth definition for L1-RSRP prediction and consider UE-reported measured L3-RSRP as the ground truth for evaluation. Furthermore, in specific sub-use cases where the UE initially predicts L1-RSRP and subsequently derives L3-RSRP predictions based on the predicted L1 samples, adhering to the L1 ground truth definition could simplify both the prediction process at the UE and the evaluation procedure at the network side. This approach is also consistent with the legacy FR2 definition of absolute RSRP measurement. However, other potential options with higher accuracy can also be investigated.

RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction.

For the ground truth definition of FR1 measurement prediction, since there is no significant impact of beamforming gain or other parameters/impairments affecting UE measurements and predictions (which are derived based on these measurements), conducted tests in a controlled environment can be utilized for evaluation. In this case, the FR1 ground truth can be defined as the ideal L3-RSRP measurement (i.e., transmitted power) as the reference, which can be considered the ground truth for FR1 L3-RSRP prediction ‎[4].

RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power).

Relative RSRP Accuracy

Ensuring high network performance, maintaining required QoS, and delivering an optimal user experience requires achieving high levels of relative accuracy in both measurements and predicted measurements. This is particularly critical for Radio Resource Management (RRM) decisions, such as handovers, where high relative accuracy is essential to enable seamless transitions between cells, minimize radio link failures and ping-pong effects, conserve network resources, and maintain uninterrupted connectivity for users. Consequently, it is imperative to define both absolute and relative accuracy to ensure seamless interoperability between the network and diverse devices.

RAN4 to consider both absolute and relative accuracy for measurement prediction.

The legacy definition of relative accuracy states [TS 38.133]: “The SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell, or between any two SS-RSRP levels measured on the same cell.” This definition, depending on the scenario, applies to both intra-frequency and inter-frequency cases in FR1 and FR2 frequency bands.

In mobility scenarios, the UE may predict the RSRP for one cell, which then needs to be compared with the RSRP measured or predicted for another cell. Alternatively, the UE might need to compare two predicted RSRP values directly.

Depending on the scenario, the UE may be required to compare the predicted RSRP of one cell or beam with either the measured or predicted RSRP of another cell or beam.

RAN4 to define relative accuracy for the comparison between the predicted RSRP of one cell with either the measured or predicted RSRP-level of another cell or same cell.

Regarding measurement prediction especially in the domain of temporal L3 RSRP prediction as presented in  Figure 1, the applied filtering coefficient necessitates that the UE may or may not combine and reports both predicted and measured samples as one value. These combined samples must then be compared with reports from other cells or beams to ensure accurate performance assessment. Given this process, it is crucial that the requirements for predicted relative accuracy are clearly defined and aligned with the established definitions and requirements for legacy relative accuracy specified in [TS 38.133]. This alignment ensures consistency and reliability in performance metrics, facilitating seamless integration and comparison across different network elements and technologies. Additionally, maintaining this alignment helps in optimizing network performance and enhances the overall user experience by ensuring that the predicted values are as close to the actual measured values as possible.

Figure 1: Examples of temporal domain predictions.

Definition and requirements for predicted relative accuracy need to be defined and aligned with the established definitions and requirements for legacy relative measurement accuracy specified in TS 38.133.

It is expected Case-B temporal domain prediction to comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

Based on the above discussions, we believe that at least for Case-B temporal domain prediction, which focuses on measurement reduction, the definition of relative accuracy should be consistent for both predicted and measured values, or a combination of both. This consistency is essential to ensure that the performance metrics are reliable and comparable. The requirement value for relative accuracy can then be defined based on various prediction parameters such as the observation window, prediction window, and measurement reduction rate.

For at least temporal domain AI/ML mobility measurement prediction, RAN4 to define relative accuracy as “reported SS-RSRP level of one cell compared to another reported SS-RSRP level from another cell or the same cell, where the reported level is predicted or measured or a combination of both predicted and measured samples”.

For other types of prediction scenarios e.g., frequency domain prediction, RAN4 may consider studying other KPI metrics reusing agreed relative accuracy metrics by RAN4 AI/ML for beam management. One option is to define relative accuracy as measured SS-RSRP of (the best) predicted cell compared to the highest measured SS-RSRP of another cell or the same cell.

RAN4 may study a different relative accuracy definition for frequency domain prediction aligning with definitions of RAN4 AI/ML for beam management.  One option is to define relative accuracy as ground truth SS-RSRP of (the best) predicted cell compared to the ground truth of the highest SS-RSRP of another cell or the same cell.

Impacts on beam/cell level measurements

In addition, as outlined in below from RAN4#112bis ‎[4], further investigation is needed on the performance metrics used to evaluate the effectiveness of AI/ML measurement predictions, as discussed in the previous meeting.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Accordingly, RAN4 has agreed to focus on studying L3 cell-level RSRP (Point C) for evaluating measurement prediction. Additionally, two other options—L1 beam-level (Point A1) and L3 beam-level (Point E)—remain open for further discussion. For L1 measurements, as outlined in sub-use cases 1 and 3, the UE can either derive L3 cell-level measurements from predicted L1 measurements or directly predict cell-level measurements based on actual L1 measurements. Since L3 cell-level measurements and predictions are expected to be reported by the UE or used by the network for RRM and handover decisions, it is logical to consider these as the primary evaluation point for performance metrics. As L3 beam-level measurements could further optimize mobility, they can be examined after the study of cell-level measurements or undergo additional evaluation by RAN2.

Figure 2: 3GPP TS 38.300 Measurement Model.

Additionally, considering both L3 beam-level and L3 cell-level measurements, RAN2#127 and RAN2#127bis have reached agreements prioritizing L3 cell-level measurements for simulation and performance evaluation.

RAN2#127 FS_NR_AIML_Mob - Release 19 Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2#127bis FS_NR_AIML_Mob - Release 19 Agreements - To keep two filtering options on the table and up to company to report. - One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. - In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

Beam-level L3 predictions are deprioritized in RAN2 FS_NR_AIML_Mob - Release 19.

RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

For L3 cell-level RSRP (Point C) prediction, as agreed in RAN4, measurement prediction can be based on three sub-use cases: (i) predicted L1 beam-level measurement results derived from actual L1 measurements (Sub-use case 1), (ii) actual L3 cell-level measurement results (Sub-use case 2), or (iii) actual L1 beam-level measurement results (Sub-use case 3). Since the accuracy of AI/ML model outputs is highly sensitive to the choice of sub-use case as well as the quality, quantity, and characteristics of the inputs, it is essential to study and potentially prioritize one of the agreed sub-use cases. Based on system-level simulation evaluations conducted in RAN2, the majority of companies have indicated that cell-level prediction based on actual L3 cell-level measurements provides the highest accuracy in Mean Absolute Error (MAE) performance.

The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy.

RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Impacts of measurement errors

RAN4 has discussed the impact of measurement errors for AI/ML Mobility evaluation and identified a number of options on measurement errors.

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Way Forward: Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event

In mobility scenarios, it is unavoidable that measurement errors will occur when using data as input or ground truth for AI/ML models. These errors can significantly impact network performance by causing inaccurate predictions, which consequently can affect mobility management, handover optimization, and RRM.

When conducting simulations, it is imperative to consider measurement errors to define the inputs of AI/ML models. One option involves utilizing a Gaussian distribution, similar to the RAN4 measurement error requirements for both RF and baseband. The parameters of an error distribution can be selected to capture the characteristics of normal or extreme conditions for the simulation assumption. Alternatively, the different distributions for baseband and RF can be assumed, as discussed in the RAN4 AI/ML for Beam Management. In this scenario, baseband errors may be generated using a Gaussian distribution, or they can be modeled through link-level simulations. For RF errors, generation can occur via a uniform or Gaussian distribution, or they can be assigned a constant value. Moreover, when incorporating RF errors in simulations, RAN4 can also consider whether each TX beam has the error independently.

RAN4 can discuss how to capture and model the measurement error with the following options.

  - Option 1: Both RF and baseband errors follow a Gaussian distribution.

  - Option 2: Baseband and RF errors follow different distributions.

    - Baseband error is modeled as:

      - Option A: Gaussian distribution

      - Option B: link-level simulation results

    - RF error is modeled as:

      - Option A: uniform distribution

      - Option B: Gaussian distribution

      - Option C: truncated Gaussian distribution

      - Option D: fixed value

    - How to apply RF error:

      - Option A: Independent for each TX beam

      - Option B: Same for each Tx beam

      - Option C: Independent for each module/panel

      - Option D: Same for each module/panel

  - Option 3: Other options are not precluded.

RAN4 should agree that both RF and baseband errors follow a Gaussian distribution (Option 1).

When evaluation is conducted in the high-priority scenarios, a study is needed to evaluate the impacts of these measurement errors on prediction accuracy. Currently, this issue is under discussion in RAN2. RAN4 may consider waiting for RAN2's agreement and then initiate discussions based on RAN2's evaluations. Also, RAN2 and RAN4 are studying the impact of error based on the different assumptions. For instance, RAN4 performance metrics assumes a measurement error following a Gaussian distribution. RAN2 is however studying a mean square error without having an assumption on measurement errors. Therefore, by considering the aforementioned measurement errors in a RAN2 study, it becomes feasible to evaluate their impacts based on both RAN4 and RAN2 metrics.

RAN4 should send an LS to RAN2, suggesting that RAN2 considers measurement errors in its evaluation.

We have provided a draft LS in Annex I.

Impacts on Core requirements:

From TR 38.843: The life cycle management (ac) of AI/ML model (e.g., model training, model deployment, model inference, model monitoring, model updating) and AI/ML functionality are characterized.

The following aspects, including the definition of components (if needed) and necessity, are studied in LCM:

-	Data collection

-	Note: 	This also includes associated assistance information, if applicable.

-	Model training

-	Functionality/model identification

-	Model delivery/transfer

-	Model inference operation

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it)

-	Functionality/model monitoring

-	Model update

-	UE capability

LCM procedures related to RRM for mobility involve in particular procedures on model activation, deactivation, switching to another model as well fallback operation (including to non-ML option). These different operations will impact the testing procedure as part of RAN4 (including test scenario definition and requirements) and should be investigated and aligned with RAN2 progress.

RAN4 should investigate requirements related to LCM procedures (in particular model activation, deactivation, switching and fallback operation), aligned with RAN2 progress on this topic.

LCM framework agreed in AIML air interface work item will be used as baseline for any further enhancement for AIML mobility in RAN4.

Impacts from observation and prediction windows

Over the different identified scenarios in AIML mobility, both Observation Window (OW) and Prediction window (PW) should be specified as part of the ML based prediction procedure. The observation window will therefore impact the past measurements to collect and use as input of the trained ML model. Whereas the prediction window indicates the time ahead for the predicted measurements.

Testing the different scenarios with different assumptions (e.g. inter & intra frequency, beam/cell levels) will require to investigate impacts from several factors including:

- DRX cycle: UE being in DRX state can impact its measurement ability and therefore OW and PW should be adjusted according to DRX parameters

- Measurement gaps: which may affect the delay and reporting of predicted measurements for scenarios involving inter frequency and inter cell (scenario 3). The configuration of measurement gaps should be investigated to evaluate the impact on prediction window specifically.

RAN4 should investigate impacting factors on the selection of observation and prediction windows for RAN4 requirements and test cases specification.

Inter-frequency prediction scenario

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency.

By utilizing measurement data from two frequency layers, an ML model can be trained to understand the correlation between radio measurements across these layers. Specifically, the model can either predict first-layer measurements based on second-layer measurements or determine if a UE connected to a macro layer can connect to a micro layer. This may involve a clustering approach, where measurements from multiple cells serve as inputs for prediction. A time series of measurements, along with additional inputs, may also be considered. Consequently, for inter-frequency measurement prediction, both the input data and model performance metrics (e.g., accuracy) differ from those used in temporal domain predictions

The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility scenario and RRM requirements to be studied in RAN4 working group.

The following Observations and Proposals were made:

Proposal 1: RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction.

Proposal 2: RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power).

Proposal 3: RAN4 to consider both absolute and relative accuracy for measurement prediction.

Observation 1: Depending on the scenario, the UE may be required to compare the predicted RSRP of one cell or beam with either the measured or predicted RSRP of another cell or beam.

Proposal 4: RAN4 to define relative accuracy for the comparison between the predicted RSRP of one cell with either the measured or predicted RSRP-level of another cell or same cell.

Observation 2: Definition and requirements for predicted relative accuracy need to be defined and aligned with the established definitions and requirements for legacy relative measurement accuracy specified in TS 38.133.

Observation 3: It is expected Case-B temporal domain prediction to comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

Proposal 5: For at least temporal domain AI/ML mobility measurement prediction, RAN4 to define relative accuracy as “reported SS-RSRP level of one cell compared to another reported SS-RSRP level from another cell or the same cell, where the reported level is predicted or measured or a combination of both predicted and measured samples”.

Proposal 6: RAN4 may study a different relative accuracy definition for frequency domain prediction aligning with definitions of RAN4 AI/ML for beam management.  One option is to define relative accuracy as ground truth SS-RSRP of (the best) predicted cell compared to the ground truth of the highest SS-RSRP of another cell or the same cell.

Observation 4: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

Observation 5: Beam-level L3 predictions are deprioritized in RAN2 FS_NR_AIML_Mob - Release 19.

Proposal 7: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

Observation 6: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

Observation 7: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy.

Proposal 8: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Observation 8: RAN4 can discuss how to capture and model the measurement error with the following options.

- Option 1: Both RF and baseband errors follow a Gaussian distribution.

- Option 2: Baseband and RF errors follow different distributions.

  - Baseband error is modeled as:

    - Option A: Gaussian distribution

    - Option B: link-level simulation results

  - RF error is modeled as:

    - Option A: uniform distribution

    - Option B: Gaussian distribution

    - Option C: truncated Gaussian distribution

    - Option D: fixed value

  - How to apply RF error:

    - Option A: Independent for each TX beam

    - Option B: Same for each Tx beam

    - Option C: Independent for each module/panel

    - Option D: Same for each module/panel

- Option 3: Other options are not precluded.

Proposal 9: RAN4 should agree that both RF and baseband errors follow a Gaussian distribution (Option 1).

Proposal 10: RAN4 should send an LS to RAN2, suggesting that RAN2 considers measurement errors in its evaluation.

Proposal 11: RAN4 should investigate requirements related to LCM procedures (in particular model activation, deactivation, switching and fallback operation), aligned with RAN2 progress on this topic.

Proposal 12: LCM framework agreed in AIML air interface work item will be used as baseline for any further enhancement for AIML mobility in RAN4.

Proposal 13: RAN4 should investigate impacting factors on the selection of observation and prediction windows for RAN4 requirements and test cases specification.

Observation 9: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

Proposal 14: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2420095, WF on AI/ML Mobility, Nokia, 18th – 22nd November 2024, Orlando, USA.

- R4-2401570, WF on AI/ML, Qualcomm Incorporated, Fukuoka city, Japan , May 20th – 24th, 2024.

- R4-2416855, WF on AI/ML Mobility, Nokia,  Hefei, Anhui, China, 14th – 18th October 2024.

- Annex I (draft LS)

3GPP TSG-RAN WG4 Meeting #114	R4-25XXXXX

Athens, Greece, 17th – 21st February 2025

Title:	LS on impacts of measurement errors on RAN2 evaluation

Response to:

Release:	Rel-19

Work Item:	FS_NR_AIML_Mob

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 has agreed in RAN4#114 on the necessity of considering the impacts of measurement error in simulations; for example, the measurement error in RF and baseband is modelled as a Gaussian distribution. The agreement on simulation assumption is captured in WF [1].

With that understanding in RAN4, it is vital for RAN2 to discuss and adopt the measurement error in the simulation for performance evaluation.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to consider the impacts of measurement errors in the simulations, so that the input to AIML model captures the measurement error.

3. Reference:

- R4-24XXXXX, “WF on AI/ML mobility,” 3GPP RAN4#114, Feb. 17-21, 2025.

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114-bis   		April 07 – April 11, 2025			Wuhan, China

TSG RAN WG4 Meeting #115	   		May 19 – May 23, 2025			Malta, MT