# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501985

# Athens, Greece, 17 – 21 February 2025

# Title:	PC2 RSD for the FD-FDD RedCap UE

# Agenda Item:	7.7.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document discusses the PC2 RSD requirements for the FD-FDD RedCap UE.

# Discussion

For the FD-FDD RedCap UE, the 1Rx REFSENS requirements are specified that the 2Rx REFENS requirements increase by parameter R1R, specified in Table 7.3I.2-1, based on LTE Cat-1bis agreements for 5, 10, 15, 20MHz CBW.

To our limited understanding on how the R1R requirements were derived for LTE Cat-1bis, it is unclear if at that time, the impact of Tx noise self-desense in bands with large CBW relative to the Tx-Rx gap (eg. Band n8 20MHz CBW) were accounted for. We do not intend to challenge the legacy agreements, but it may be worth cross-checking if applying a “constant” R1R across all applicable FDD bands is valid for CBW where the UE may be affected by Tx noise leakage self-desense.

Observation: It is not clear if the LTE Cat-1bis “constant” 2Rx REFSENS derating using R1R is valid for CBW in FDD bands that may be affected by Tx noise leakage self-desense.

Under the assumption that R1R can be applied to all FDD bands and all CBW, it can be assumed that the 1Tx RSD currently specified for 2Rx can also be applied to 1Rx REFSENS requirement.

Proposal: The PC2 FD-FDD 1Rx REFSENS requirement is specified as PC3 FD-FDD 1Rx REFSENS increased by RSD specified in Table 7.3.2-1c. An example of core-requirement changes is proposed as a starting point below.

“For a power class 3 RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. For a power class 2 RedCap UE equipped with 1 Rx antenna port, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed and the reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a shall be modified by the amount given in Table 7.3.2-1c for the applicable operating bands.”

# Conclusion

This contribution summarizes our views regarding the RSD requirements for the PC2 FD-FDD RedCap UE.

Observation: It is not clear if the LTE Cat-1bis “constant” 2Rx REFSENS derating using R1R is valid for CBW in FDD bands that may be affected by Tx noise leakage self-desense.

Proposal: The PC2 FD-FDD 1Rx REFSENS requirement is specified as PC3 FD-FDD 1Rx REFSENS increased by RSD specified in Table 7.3.2-1c. An example of core-requirement changes is proposed as a starting point below.

“For a power class 3 RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. For a power class 2 RedCap UE equipped with 1 Rx antenna port, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed and the reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a shall be modified by the amount given in Table 7.3.2-1c for the applicable operating bands.”