# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501987

# Athens, Greece, 17 – 21 February 2025

# Title:	On near-miss for Rx harmonic mixing

# Agenda Item:	6.3.3

# Source:	Skyworks Solutions, Inc., Anritsu, Qualcomm France, Oppo.

# Document for:	Approval

# Introduction

This document clarifies the MSD test point rules for bands affected by near-miss Rx harmonic mixing interference. These proposals may also serve a companion to a revision of any maintenance CR presented at this meeting. Alternatively, these changes may be captured at meeting #114-bis.

# Discussion

## Background on near-miss Rx harmonic mixing.

CA_n7A-n26A is the only band combination for which a near-miss Rx harmonic MSD test point has been specified for Band n26. We reproduce below the text proposal which discussed the MSD test point selection and the MSD evaluation for this combination from [1].

“For CA_n7A-n26A, the worst case n26 MSD may not be met if the Band n7 is configured to its minimum CBW of 5 MHz because the Rx harmonic would, at worst, only mix with ACLR2. We therefore propose to adopt the smallest n7 UL CBW that ensures Rx harmonic mixing occurs with the highest interference level, i.e., with ACLR1.

The new MSD template facilitates this task since the UL RB allocation start position is specified. The simplified intermodulation distortion (IMD) sketch of Figure 1 shows that these worst-case conditions may be met for:

- Band n26 5MHz CBW, lowest carrier DL frequency, Fc= 861.5MHz. The third Rx harmonic is 2584.5MHz;

- Band n7 25MHz CBW (SCS15kHz), highest UL carrier frequency, Fc = 2557.5MHz; and

- Band n7 UL RB allocation set to Lcrb=25RB104. The IMD3 center frequency of modulated RB with its image is 2584.5MHz, ie. centered on the third order Rx harmonic.”

Figure 1: Simplified IMD landscape for CA_n7A-n26A UL1/DL3 Rx harmonic mixing test point.

The MSD test point of Table 1 was proposed and later implemented into the TS. This test point ensures that Band n26 is affected by Rx harmonic mixing of the UL1 ACLR1. A new “Note X” was proposed to specify the UL/DL carrier frequencies of this test point.

Table 1: n26 UL1/DL3 near-miss Rx harmonic mixing MSD for CA_n7A-n26A

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n26 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest EARFCN or NR ARFCN of the affected DL (lower) band, and for the highest EARFCN or NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

However, we realized that the MSD test selection rule proposed for CA_n7A-n26A (highlighted in blue above) was never captured officially, neither in a Way Forward, nor into the guidelines of TR 38.846.

We also observe in Table 2 that during the huge initiative of cleaning up the 1UL Rx harmonic mixing MSD test points, the UL CBW was changed from 25MHz to 5MHz and the UL RB allocation “RBstart” was modified according to the new NOTE 13 “clean-up” rules that apply to direct-hit MSD test points.

Table 2: Band n26 UL1/DL3 near-miss Rx harmonic mixing MSD for CA_n7A-n26A from Rel 18.7.0 Table 7.3A.4-4.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n263 | 25 5 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE 10 | UL1/DL3 near-miss |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

Under these new test conditions, the Band n26 is now affected by Rx harmonic mixing of UL1 ACLR~3. The specified MSD level of 2.0dB is no longer valid.

Observation 1: The MSD test point selection rules for near-miss Rx harmonic mixing were never officially captured at RAN4. In addition, the original Band n26 2dB MSD level is no longer valid under the new UL CBW / UL RB allocation rules that were used to “clean-up” the 1UL MSD test points.

Observation 2: The difference between “direct-hit” and “near-miss” for harmonic mixing may be summarized as:

- Direct-hit: when one of the harmonics of the DL band LO overlaps with one RE of the UL band signal (its fundamental “UL1”) or with one the UL band harmonics (UL2, UL3, etc..)

- Near-miss: when one of the harmonics of the DL band LO overlaps with the ACLR1 of the UL band signal.

This clarification may be captured in the RAN4 PRD together with the set of agreed rules on valid UL/DL harmonic orders for harmonic mixing.

Based on observation 1, we propose to adopt into the RAN4 PRD the following exception rules for MSD test point due to near-miss Rx harmonic mixing. To avoid breaking the new “Note 13” rule, we propose that the UL RB allocation is configured fully allocated. These “exception” rules are justified by the fact that such interference collision scenarios are by nature also “exceptional” cases, i.e. the number of affected band combinations is very small.

Proposal 1: harmonic MSD requirements are specified only for the UL1/DLx cases, where x=2,3,4,5,

- A maximum of one near-miss MSD test point may be specified:

  - For the smallest DL affected band (lower band) CBW,

  - For the smallest UL (higher band) CBW that ensures Rx harmonic mixing of the UL1 ACLR 1 lower channel ("ACLR1_low") or Rx harmonic mixing of the UL1 ACLR 1 higher channel ("ACLR1_high"),

  - For full UL band RB allocations.

- The near-miss harmonic mixing test conditions for UL/DL carrier frequencies are specified using a footnote:

  - For DLx Rx harmonic mixing with the UL1 ACLR1_high:

    - “Note X” (eg. Table 7.3A.4-4 Note 10) applies: "The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band".

  - For DLx Harmonic mixing with the UL1 ACLR1_low:

    - A new “Note Y” is needed: "The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band”.

Based on Proposal 1, it is proposed to keep Band n26 MSD at 2.0dB and bring the following corrections.

Proposal 2: Adopt the following corrections to the CA_n7A-n26A near-miss UL1/DL3 MSD test point.

Table 3: Proposed corrections to Band 26 PC3 UL1/DL3 near-miss MSD requirements.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n263 | 25 | 15 | 128 | 5 | 2.0 | NOTE 10 | UL1/DL3 near-miss |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

## Correction for CA_n28A-n78A

It appears that the MSD test point for CA_n28A-n78A has been copied from the test point specified for CA_n28A-n77A for which direct-hit UL1/DL5 collision occurs, leading to 31dB of the Band n28 5MHz CBW. However, direct-hit test conditions can not be met for CA_n28A-n78A. At best, it is possible to ensure that Band n28 is affected by Rx harmonic mixing of the Band n78 ACLR 1. Since this case is like that of CA_n7-n26, we propose the following MSD test point corrections based on the rules of proposal 1.

MSD re-evaluation:

The minimum Band n78 CBW that ensures Rx harmonic mixing of the Band n28 lowermost channel 5MHz local-oscillator 5th harmonic (“DL5”) with the Band n78 ACLR1 is 10MHz as illustrated in Figure 2-left. The Band n28 5th LO harmonic “falls” at 3802.5MHz, i.e. within the band n78 ACLR1 that ranges from 3800 to 3810MHz.

Figure 2: Left: simplified IMD landscape for CA_n28A-n78A UL1/DL5 near-miss Rx harmonic mixing. Right: PC3 n78 10MHz CBW, Lcrb=50RB PA interference measurements for Band n28 5MHz CBW.

The Band n28 MSD is re-evaluated by calculating the UL1 direct-hit interference level which leads to 31dB MSD. We assume a 6dB Main/Diversity interference imbalance based on 1UL MSD simplification assumptions from [2].  We then measure the near-miss interference level in 5MHz CBW at the Band n28 DL5 frequency of 3802.5MHz frequency using a PC3 Band n78 10MHz CBW LCRB=50RBs power amplifier (PA). The measured near-miss interference using the “green” band power markers of of Figure 2-right and Figure 3 is ~27dB, -43dB, -48.8dB and -63.2dB smaller than the direct-hit interference level respectively for 5, 10, 15, 20MHz CBW. The decay of the band n28 near-miss interference is due to down-mixing of increasing ACLR order as shown in Figure 3.

- n28 5MHz is affected by ACLR1,

- n28 10MHz is affected by ACLR2,

- n28 15MHz is affected by ACLR3,

- n28 20MHz is affected by ≥ACLR4.

Figure 3: PC3 n78 10MHz CBW, Lcrb=50RB PA interference measurements for Band n28 10,15,20MHz CBW.

Table 4 captures the band n28 MSD for 5, 10, 15, 20MHz CBW where we assume 6dB interference imbalance based on the MSD simplification proposals of [2]. This near-miss cases may only affect band n28 5MHz CBW with 6.1dB MSD.

Table 4: PC3 Band n28 near-miss UL1/DL5 MSD evaluation.

|  | n28 5MHz Direct-hit |  | n28 5MHz Near-miss |  | n28 10MHz Near-miss |  | n28 15MHz Near-miss |  | n28 20MHz Near-miss |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 DL2 near-miss interference relative to direct-hit (dB) | n-a |  | -27 |  | -43 |  | -48.8 |  | -63.2 |  |
|  | Prim. | Div. | Prim. | Div. | Prim. | Div. | Prim. | Div. | Prim. | Div. |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -62.0 | -68.0 | -89.0 | -95.0 | -105.0 | -111.0 | -110.8 | -116.8 | -125.2 | -131.2 |
| n 28 ideal REFSENS (dBm/MBW) | -98.5 |  | -98.5 |  | -95.3 |  | -93.4 |  | -92.2 |  |
| n28 REFSENS MRC degradation (dB) | 31.0 |  | 6.1 |  | 0.4 |  | 0.1 |  | 0 |  |

Observation 3: The band n28 MSD due to near-miss UL1/DL5 Rx mixing interference in CA_n28A-n78A may be ignored for CBW >= 10MHz. The band n28 PC3 MSD for 5MHz CBW is evaluated at 6.1dB.

Based on observation 2, we evaluate the Band n28 5MHz CBW PC2 and PC1.5 MSD requirements in Table 6 using the Table 5 simplification look-up table reproduced below from [2] where 6dB Main/Diversity 1Tx interference imbalance is assumed.

Table 5: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

Table 6: PC2 and PC1.5 Band n28 5MHz CBW near-miss UL1/DL5 MSD.

| Near-miss UL1/DL5 Rx harmonic HPUE MSD for CA_n28_n78 | PC 2 n28 5MHz | PC1.5 n28 5MHz |
| --- | --- | --- |
| PC3 MSD (dB) | 6.1 |  |
| PC2/PC1.5 (dB) | 2.1 | 6.9 |
| PC2/PC1.5 MSD (dB) | 8.2 | 13.0 |

The MSD levels reported in Table 6 are captured in proposal 3 (P3) for Rel-15 & Rel-16, in P4 for Rel-17 and in P5 for Rel-18 & Rel-19 as text proposal for correction CRs.

Based on proposal 1:

- New footnotes are proposed.

- Only one test point is specified for these “exception” near-miss cases. We notice that 2 test points are specified for HPUE, hence we propose to remove the 2nd test point.

Proposal 3: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-15 and in Rel-16.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | n284 | 6.1 |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1.

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n28 | 15 | 50 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-17.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 8.2 | NOTE 6 | UL1/DL5 near-miss |
| NOTE 6: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Proposal 5: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-18 and in Rel-19.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 8.2 | NOTE 15 | UL1/DL5 near-miss |
| NOTE 15: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 13.0 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

## New near-miss Rx harmonic MSD test points

### CA_n1A-n79A near-miss UL1/DL2

Band n1 may be affected by UL1/DL2 near-miss Rx harmonic mixing of Band n79 ACLR1 low channel as depicted in Figure 4-left. It is found that the smallest Band n79 CBW for which this condition is met is 80MHz for which the measured near-miss ACLR1 interference that may affect band n1 is -46.8dB smaller than the direct-hit UL1 interference – cf  Figure 4-right.

Figure 4: Left: Simplified IMD landscape for CA_n1A-n79A UL1/DL2 near-miss Rx harmonic mixing. Right: Measured Band n79 PC3 PA emissions at MPR0 for Lcrb=216RB0 CBW=80MHz SCS30kHz.

The band n1 MSD analysis is evaluated in Table 8 with parameters similar to those of [3] of Table 7.

Table 7: RF Front-end parameters for CA_n1-n79 UL1/DL2 near-miss PC3 MSD analysis.

| Parameter | Value | Unit |
| --- | --- | --- |
| Primary to Diversity antenna isolation | 10 | dB |
| PCB isolation from n79 PA to band n1 LNA | 70 | dB |
| PC3 n79 antenna referred output power | 23 | dBm |
| Primary/Diversity RX front-end and path loss | 4 | dB |
| PC3 Near-miss to direct-hit interference ratio | -46.8 | dB |
| PC2 Near-miss to direct-hit interference ratio | -49.3 | dB |
| Total conductive rejection (DPX/DRX filter + antenna multiplexer) | 65 | dB |
| 2nd Harmonic mixing rejection | 50 | dB |
| SNR requirement for QPSK | -1 | dB |

Table 8: CA_n1-n79 PC3 near-miss UL1/DL2 MSD analysis.

| Conducted Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity | Unit |
| n79 PA to Primary LNA referred to antenna | -130.8 | N/A | dBm |
| n79 PA to Diversity LNA referred to antenna | N/A | -148.8 | dBm |
| PCB Coupling Path |  |  |  |
| Parameter | Main | Diversity |  |
| Path number 1 and 2 referred to antenna | -135.8 | -135.8 | dBm |
| Combined |  |  |  |
| Total interference power referred to antenna | -129.6 | -135.8 | dBm |
| PC3 MSD after MRC (n1 RX CBW of 5MHz) | 0 |  | dB |

There is no MSD for Band n1 near-miss UL1/DL2 for PC3 and for PC2 operation.

Observation 4: Band n1 may be affected by near-miss UL1/DL2 Rx mixing interference for CA_n1-n79. Band n79 PA measurements indicate there is no MSD for both PC3 and PC2 operation.

### CA_n14A-n30A near-miss UL1/DL3

Band n14 may be affected by UL1/DL3 near-miss Rx harmonic mixing of band n30 ACLR1 low channel as depicted in Figure 5-left. It is found that the smallest Band n30 CBW for which this condition is met is 10MHz for which the measured near-miss ACLR1 interference that may affect band n14 is -38.8dB smaller than the direct-hit UL1 interference – cf  Figure 5-right.

Figure 5: Left: Simplified IMD landscape for CA_n14A-n30A UL1/DL3 near-miss Rx harmonic mixing. Right: Measured Band n30 PC3 PA emissions at MPR0 for Lcrb=50RB0 CBW=10MHz SCS15kHz.

The band n14 MSD analysis is evaluated in Table 10 with parameters of Table 9. The Band n14 may be affected by 1.6dB MSD for 5MHz CBW.

Table 9: RF Front-end parameters for CA_n14-n30 UL1/DL3 near-miss PC3 MSD analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Primary to Diversity antenna isolation | 10 | dB |
| PCB isolation from n30 PA to band n14 LNA | 70 | dB |
| PC3 n30 antenna referred output power | 23 | dBm |
| Primary/Diversity RX front-end and path loss | 4 | dB |
| PC3 Near-miss to direct-hit interference ratio | -38.8 | dB |
| Total conductive rejection (DPX/DRX filter + antenna multiplexer) | 65 | dB |
| 3rd Harmonic mixing rejection | 25 | dB |
| SNR requirement for QPSK | -1 | dB |

Table 10: CA_n14-n30 PC3 near-miss UL1/DL3 MSD analysis.

| Conducted Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity | Unit |
| n30 PA to Primary LNA referred to antenna | -97.8 | N/A | dBm |
| n30 PA to Diversity LNA referred to antenna | N/A | -115.8 | dBm |
| PCB Coupling Path |  |  |  |
| Parameter | Main | Diversity |  |
| PA to Primary/Diversity LNA via PCB coupling | -102.8 | -102.8 | dBm |
| Combined |  |  |  |
| Total interference power referred to antenna | -96.6 | -102.6 | dBm |
| PC3 MSD after MRC (n14 RX CBW of 5MHz) | 1.6 |  | dB |

Proposal 6: Introduce in Rel-19 the following Band n14 PC3 MSD test point for CA_n14A-n30A with new Note “Y”.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n30 | n14 | 10 | 15 | 50 | 5 | 1.6 | NOTE Y | UL1/DL3 near-miss |
| NOTE Y: The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

# Conclusion

This paper proposes a clear set of rules for MSD test point due to near-miss Rx harmonic mixing interference. Based on these rules, we propose corrections for CA_n7A-n26A, CA_n28A-n78A. A new test point is proposed for CA_n14A-n30A.

Observation 1: The MSD test point selection rules for near-miss Rx harmonic mixing were never officially captured at RAN4. In addition, the original Band n26 2dB MSD level is no longer valid under the new UL CBW / UL RB allocation rules that were used to “clean-up” the 1UL MSD test points.

Observation 2: The difference between “direct-hit” and “near-miss” for harmonic mixing may be summarized as:

- Direct-hit: when one of the harmonics of the DL band LO overlaps with one RE of the UL band signal (its fundamental “UL1”) or with one the UL band harmonics (UL2,UL3, etc..)

- Near-miss: when one of the harmonics of the DL band LO overlaps with the ACLR1 of the UL band signal.

This clarification may be captured in the RAN4 PRD together with the set of agreed rules on valid UL/DL harmonic orders for harmonic mixing.

Proposal 1: harmonic MSD requirements are specified only for the UL1/DLx cases, where x=2,3,4,5,

- A maximum of one near-miss MSD test point may be specified:

  - For the smallest DL affected band (lower band) CBW,

  - For the smallest UL (higher band) CBW that ensures Rx harmonic mixing of the UL1 ACLR 1 lower channel ("ACLR1_low") or Rx harmonic mixing of the UL1 ACLR 1 higher channel ("ACLR1_high"),

  - For full UL band RB allocations.

- The near-miss harmonic mixing test conditions for UL/DL carrier frequencies are specified using a footnote:

  - For DLx Rx harmonic mixing with the UL1 ACLR1_high:

    - “Note X” (eg. Table 7.3A.4-4 Note 10) applies: "The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band"

  - For DLx Harmonic mixing with the UL1 ACLR1_low:

    - A new “Note Y” is needed: "The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band”.

Proposal 2: Adopt the following corrections to the CA_n7A-n26A near-miss UL1/DL3 MSD test point.

Table 3: Proposed corrections to Band 26 PC3 UL1/DL3 near-miss MSD requirements.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n263 | 25 | 15 | 128 | 5 | 2.0 | NOTE 10 | UL1/DL3 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

Observation 3: The band n28 MSD due to near-miss UL1/DL5 Rx mixing interference in CA_n28A-n78A may be ignored for CBW >= 10MHz. The band n28 PC3 MSD for 5MHz CBW is evaluated at 6.1dB.

Proposal 3: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-15 and in Rel-16.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | n284 | 6.1 |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1.

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n78 | n28 | 15 | 50 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-17.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 8.2 | NOTE 6 | UL1/DL5 near-miss |
| NOTE 6: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Proposal 5: Adopt the following corrections to the Band n28 near-miss UL1/DL5 MSD requirements for CA_n28A-n78A in Rel-18 and in Rel-19.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 8.2 | NOTE 15 | UL1/DL5 near-miss |
| NOTE 15: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 13.0 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Observation 4: Band n1 may be affected by near-miss UL1/DL2 Rx mixing interference for CA_n1-n79. Band n79 PA measurements indicate there is no MSD for both PC3 and PC2 operation.

Proposal 6: Introduce in Rel-19 the following Band n14 PC3 MSD test point for CA_n14A-n30A with new Note “Y”.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n30 | n14 | 10 | 15 | 50 | 5 | 1.6 | NOTE Y | UL1/DL3 near-miss |
| NOTE Y: The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

# References

- R4-2215516 MSD for CA_n7A-n26A, 3GPP TSG-RAN WG4 Meeting #104-bis-e, electronic meeting, Skyworks Solutions, Inc.

- R4-2501955 1UL MSD simplification rules for Rel-19, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia.

- R4-1911161 The 2nd Harmonic Mixing MSD for ENDC B3_n77/n78, 3GPP TSG-RAN WG4 Meeting #92-bis, Chongqing, China, MediaTek Inc., Murata.