# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501989

# Athens, Greece, 17 – 21 February 2025

# Title:	HPUE MSD for CA_n2A-n77A and CA_n14A-n77A

# Agenda Item:	6.8.4

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document proposes PC2 single UL (1UL) MSD requirements for CA_n2A-n77A and CA_n14A-n77A requested in [1] using the 1UL PC21Tx/PC22Tx MSD simplification look-up table concept of [2].

# Discussion

## Background

The recent request for 1UL PC2 operation in FDD bands n2 and n14 triggers the following MSD analyses:

- CA_n2A-n77A: band n77 MSD due to UL2/DL1 harmonic interference

- CA_n14A-n77A: band n77 MSD due to UL5/DL1 harmonic interference

Table 2 summarizes the band n77 PC2 MSD requirements evaluated using the Table 1 PC21Tx/PC22Tx simplification look-up table reproduced from [2] where 6 dB main/diversity 1Tx interference imbalance is assumed.

Table 1: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

Table 2: Band n77 PC2 MSD requirements for CA_n2A-n77A and CA_n14A-n77A.

|  | CA_n2A-n77A UL2/DL1 |  |  |  | CA_n14A-n77A UL5/DL1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band n77 10MHz 1st MSD test point |  | Band n77 100MHz 2nd MSD test point |  | Band n77 10MHz 1st MSD test point |  | Band n77 10MHz 2nd MSD test point |  |
|  | PC21Tx | PC22Tx | PC21Tx | PC22Tx | PC21Tx | PC22Tx | PC21Tx | PC22Tx |
| Table 7.3A.4-1 PC3 MSD (dB) | 23.9 |  | 13.8 |  | 10.4 |  | 2.9 |  |
| PC21Tx/PC22Tx (dB) | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 1.6 | 2.7 |
| PC21Tx/PC22Tx MSD (dB) | 26.9 | 29.9 | 16.8 | 19.8 | 13.4 | 16.4 | 4.5 | 5.6 |

Based on Table 2, proposal 1 and proposal 2 capture the 1UL PC2 MSD requirements for band n77 in CA_n2A-n77A and CA_n14A-n77A.

Proposal 1: Adopt the following PC2 MSD requirements for band n77 in CA_n2A-n77A.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the following PC2 MSD requirements for band n77 in CA_n14A-n77A.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n14 | n77 | 5 | 15 | 12 | 10 | 13.4 | 16.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 12 | 100 | 4.5 | 5.6 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: Void NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

# Conclusion

This paper proposes 1UL PC2 MSD requirements for band n77 in CA_n2A-n7AA and CA_n14A-n77A using the 1UL MSD simplification PC21Tx/PC22Tx look-up table concept of [2] where 6 dB main/diversity interference imbalance is assumed.

Proposal 1: Adopt the following PC2 MSD requirements for band n77 in CA_n2A-n77A.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n2 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the following PC2 MSD requirements for band n77 in CA_n14A-n77A.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n14 | n77 | 5 | 15 | 12 | 10 | 13.4 | 16.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 12 | 100 | 4.5 | 5.6 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: Void NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

# References

- RP-242865 Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL), 3GPP TSG RAN Meeting #106, Madrid, Spain, China Telecom, China Unicom.

- R4-2501955 1UL MSD simplification rules for Rel-19, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia.