3GPP TSG-RAN WG4 Meeting # 114	R4-2502888

Athens, Greece, 17th – 21st February 2025

Source:	Ericsson

Title:	TP for TR 37.719-11-11 adding DC_106A_n48A, DC_106A_n48C and DC_106A_n48(2A)

Agenda item:			6.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_106A_n48A, DC_106A_n48C and DC_106A_n48(2A) as defined in WID [1].

# ---Start of changes---

### 6.1.X	DC_106_n48

#### 6.1.X.1	Configuration for DC

Table 6.1.X.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_106A_n48A | DC_106A_n48A | No |
| DC_106A_n48C | DC_106A_n48A | No |
| DC_106A_n48(2A) | DC_106A_n48A | No |

#### 6.1.X.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_106A_n48A | 23 | +2/-3 |

#### 6.1.X.3	Spurious emission band UE co-existence for DC

There is no need to include this combination in UE co-existence table.

#### 6.1.X.4	Co-existence analysis for DC

Table 6.1.X.4-1 Band 106 and Band n48 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 106 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 896 | 1792 | 2688 | 3584 | 4480 |  |
| n48 | flow | fhigh | 901 | 1802 | 2703 | 3604 | 4505 |  |
| DL1 | 3550 | 3700 | N/A | - | - | - |  | UL harmonic |
| DL23 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  | - | - | N/A | - | N/A |  |
| DL4 |  |  | - | N/A | N/A | N/A | N/A |  |
| DL54 |  |  | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 4th harmonic of band 106 may fall into RX frequencies of band n48. |  |  |  |  |  |
| UL/DL harmonics |  | n48 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| 106 | flow | fhigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 935 | 940 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1870 | 1880 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2805 | 2820 | - | - | N/A | - | N/A |  |
| DL4 | 3740 | 3760 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4675 | 4700 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing issues are identified |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.X.4-3: Band 106 and Band n48 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2804 – 2649 |  | 4446 – 4601 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1908 – 1748 |  | 6199 – 6504 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5342 – 5502 |  | 7996 – 8301 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1012 - 847 |  | 9749 - 10204 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5608 – 5298 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6238 – 6403 |  | 11546 - 12001 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8892 – 9202 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13904 – 13299 |  | 54 - 116 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9308 - 8848 |  | 4397 - 4712 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15096 - 15701 |  | 7134 - 7304 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12442 -- 12902 |  | 9788 - 10103 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.X.4-3, there are

- 4th order IMD products generated by band n48 and band 106 uplink may fall into own Rx of band 106.

#### 6.1.X.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_8-48 as specified in TS 36.101.

Table 6.1.X.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_106_n48 | 0.6 | 0.8 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.X.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_106_n48 | 0.2 | 0.5 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.1.X.6	Analysis of MSD requirements

The REFSENS requirement for DC_106_n48 due to UL harmonic is reused from DC_5_n78 and are shown in table 6.1.X.6-1:

Table 6.1.X.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 106 | n48 | 3 | 15 | 6 | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| 106 | n48 | 3 | 15 | 6 | 100 | 2.9 | NOTE 4 | UL4/DL1 direct-hit |

MSD values for band 106 is reused from DC_8_n78.

Table 6.1.X.6-2: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_106A_n48A DC_106A_n48C DC_106A_n48(2A) | 106 | 898.5 | 3 | 25 | 937.5 | [10] | IMD4 |
|  | n48 | 3633 | 10 | 50 | N/A | N/A | N/A |

---End of changes---

# Reference

[1] 	 RP-243257, Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia