# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501993

# Athens, Greece, 17 – 21 February 2025

# Title:	On simultaneous RxTx requirements

# Agenda Item:	6.12.2

# Source:	Skyworks Solutions, Inc., Nokia.

# Document for:	Approval

# Introduction

This document proposes to simplify the NR-CA simultaneous Rx/Tx requirements based upon prior work done in [1,2]. Testing considerations are also discussed based on prior work [3]. A companion CR is presented at this meeting [4].

# Discussion

The proposals of previous contributions [1,2] may be grouped into two “categories”:

- proposals related to removing redundant footnotes,

- proposals related to missing 1UL MSD analyses/requirements.

This paper focuses on cleaning up the usage of redundant footnotes. Regarding MSD, we fully support proposal 3 and 4 from [2] which state that:“RAN4 shall always conduct MSD study and if needed define MSD when Note 1(P3)/ when Note 9(P4) for simultaneous Rx/Tx is used”

Also, further to the observations and proposals of [3], we would like to clarify how test conditions are created to verify a band combination for which the UE has the mandatory requirement to support simultaneous Rx/Tx.

## On cleaning-up redundant footnotes related to simultaneous Rx/Tx.

We reproduce and we slightly augment the list of notes used in TS 38.101-1 related to simultaneous Rx/Tx in Table 1 from [1] where we highlight:

- in blue footnotes related to “mandatory simultaneous Rx/Tx capability” or equivalent meaning,

- in yellow footnotes related to “without simultaneous Rx/Tx” or “non simultaneous Tx/Rx” or equivalent meaning,

- in green when footnotes have been voided.

Table 1: Use of Notes in TS 38.101-1 related to simultaneous Rx/Tx from [1] + additional footnotes from clause 7.3A

| Table 5.2A.1-1 Table 5.2A.1-2 | NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |
| --- | --- |
| Table 5.2A.2.1-1 | NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |
|  | NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. |
|  | NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. |
|  | NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |
|  | NOTE 13: voided since [1] |
|  | NOTE 15: voided since [1] |
| Table 5.2A.2.2-1 | NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability NOTE 5: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx |
| Table 5.2A.2.3-1 Table 5.2A.2.4-1 | NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |
| Table 6.2A.4.2.3-1 | NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |
|  | NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |
|  | NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |
| Table 6.2A.4.2.4-1 | NOTE 7: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 10: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. |
| Table 7.3A.3.2.1-1 | NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |
| Table 7.3A.5-2 Table 7.3A.5-2a Table 7.3A.5-2b Table 7.3A.6-1 Table 7.3A.6-1a-1 Table 7.3A.6-1b | NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |

Observation 1:

- 19 footnotes related to simultaneous RxTx are captured in 16 different tables across several clauses: 5.2A, 5.5A, 6.2A, 7.3A. It is difficult to clarify the exact status of a given band combination, a single point of entry into TS 38.101-1 would be welcome.

- Many notes are redundant.

- According to the clause 5.2A.2 core requirements, it seems that once specified for a lower order band combination, the requirements on this capability are no longer needed for any higher order band combination built upon this lower order “fall-back” combination.

Special case of CA_n77-n78, CA_n77-n79 and CA_n78-n79:

These combinations are tagged with the following footnotes:

- CA_n77-n78 & CA_n77-n79:

  - Table 5.2A.2.1-1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration

  - Table 6.2A.4.2.3-1, 6.2A.4.2.4-1, 6.2A.4.2.5-1, 6.2A.4.2.6-1:

    - no entry for CA_n77-n78,

    - no footnote for CA_n77-n79.

  - Table 7.3A.3.2.1-1: no entries.

  - Subsequent tables of sub-clause 7.3A (e.g. 7.3A.5-2): no entry for higher order combinations using CA_n77-n78, no footnote for higher order combinations using CA_n77-n79.

- CA_n78-n79:

  - Table 5.2A.2.1-1: For UEs supporting Band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration.

  - Table 6.2A.4.2.3-1, 6.2A.4.2.4-1: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation.

  - Table 7.3A.3.2.1-1: no footnote for higher order combinations using CA_n78-n79

  - Subsequent tables of sub-clause 7.3A (e.g. 7.3A.5-2). The following footnote is captured for higher-order combination that use CA_n78-n79: “The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation.”

Observation 2:  Based on observation 1 relative to the core requirements of clause 5.2A.2, the following simplifications are sufficient:

- For CA_n77-n78, CA_n77-n79. For Table 5.2A.2.1: “The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration”

- For CA_n78-n79:

  - Note 5 of Table 5.2A.2.1 could be simplified to “The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers for UEs supporting Band 78 with a n77 implementation. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration”.

  - We observe a discrepancy between this Note 5 and the footnotes of tables 6.2A.x and 7.3A.x. It seems that the following wording is missing in Note 5 of Table 5.2A.2.1 and should be added: “The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability”.

In our view, the requirements for this capability should be captured only in a single point of entry: Table 5.2A.2.1-1.

Based on these observations we make the following simplification proposal:

Proposal 1: To simplify and clarify the requirements on the simultaneous Rx/Tx conditional mandatory capability:

- Void all related footnotes from all tables of TS 38.101-1,

- Remove all footnotes from CA combinations. Example remove Notes 1,5,7,9,13,15 from column “NR CA Band” in Table 5.2A.2.1-1,

- Capture the requirements using a new column in Table 5.2A.2.1-1 entitled “UE support for simultaneous Rx/Tx (Simultaneous RxTx)”, similar to the column that capture the “DL interruption allowed” requirements.

Regarding the wording for this new Table 5.2A.2.1-1 column:

We propose so use a simple wording that summarizes without ambiguity the four remaining footnotes:

- For NOTE 1:	“Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.”

We propose to adopt the entry “Mandatory SimRxTx”

- For NOTE 5:	“For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration.”

We tentatively propose to resolve the discrepancies between the notes of table 5.2A and those of clauses 6.2A and 7.3A by adopting the following entry “[Mandatory SimRxTx except for band 78 supported with n77 implementation]”.

- For NOTE 7: “The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration.”

We propose to adopt the entry “Without SimRxTx”

- For NOTE 9:	“Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.”

We propose to adopt the entry “Without SimRxTx”

Proposal 2:

- Normalize the entries to new column “UE support for simultaneous Rx/Tx (SimRxTx)” in Table 5.2A.2.1-1 to only four cases:

  - “Empty or blank” for optional conditional SimRxTx support

  - “Mandatory Simultaneous RxTx” for note 1

  - Tentatively “[Mandatory Simultaneous RxTx except for band 78 supported with n77 implementation]” for CA_n78A-n79A. Companies are encouraged to resolve the discrepancies of observation 2.

  - “Simultaneous RxTx is not supported” for note 5, 7 and note 9.

- Introduce the following core requirement text in clause 5.2A.2.

“5.2A.2 Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, Table 5.2A.2.2-1, Table 5.2A.2.3-1, Table 5.2A.2.4-1 and Table 5.2A.2.5-1, where all operating bands are within FR1.

The mandatory simultaneous Rx/Tx capability is specified in Table 5.2A.2.1-1. CA configurations which do not have mandatory simultaneous Rx/Tx specified and for which the UE may signal optional support for simultaneous Rx/Tx shall be captured with an empty entry. If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.”

An illustration of the simplification scheme is presented in Table 2 and companion CRs are proposed at this meeting.

Table 2: Example of new column in Table 5.2A.2.1-1 to specify clearly the simultaneous Rx/Tx requirements.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) | UE support for simultaneous Rx/Tx (Simultaneous RxTx) |
| --- | --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |  |
| CA_n1-n5 | n1, n5 |  |  |
| … | … | … | … |
| CA_n1-n18 | n1, n18 |  |  |
| CA_n1-n411 | n1, n41 |  | Mandatory Simultaneous RxTx |
| CA_n1-n46 | n1, n46 |  |  |
| … | … | … | … |
| CA_n1-n75 | n1, n75 |  |  |
| CA_n1-n771 | n1, n77 | No | Mandatory Simultaneous RxTx |
| CA_n1-n781 | n1, n78 | No | Mandatory Simultaneous RxTx |
| … | … | … | … |
| CA_n34-n399 | n34, n39 |  | Simultaneous RxTx is not supported |
| CA_n34-n40 | n34, n40 |  |  |
| … | … | … | … |
| CA_n41-n74 | n41, n74 |  |  |
| CA_n41-n771 | n41, n77 |  | Mandatory Simultaneous RxTx |
| … | … | … | … |
| CA_n77-n787 | n77, n78 |  | Simultaneous RxTx is not supported |
| CA_n77-n797 | n77, n79 |  | Simultaneous RxTx is not supported |
| … | … | … | … |
| CA_n78-n795 | n78, n79 |  | [Mandatory Simultaneous RxTx except for band 78 supported with n77 implementation] |
| … | … | … | … |
| NOTE 1: void. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: The frequency range below 2506 MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: void. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: void. NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: void. NOTE 10 The frequency range in band n77 is restricted for this band combination to 3520-3560 MHz, 3700-3800 MHz, 4000-4100 MHz. NOTE 11: The frequency range in band n78 is restricted for this band combination to 3520 -3560 MHz and 3700– 3800 MHz. NOTE 12: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 13: Void NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: Void NOTE 16: The minimum requirements for intra-band non-contiguous CA/DC apply for CA_n46-n96 or CA_n46-n102 and related higher order CA/DC configurations. NOTE 17: The combination is not used alone as fall back mode of other band combinations in which UL in Band 48 is not used. NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. NOTE 19: Void NOTE 20: The combination is not used alone as fall-back mode of other band combinations in which UL in Band n78 is not used. |  |  |  |  |

## On verifying 1UL and 2UL MSD requirements for TDD-TDD with mandatory simultaneous Rx\Tx.

In this section, we follow up proposal 2 from [3] based on the following observations. For TDD-TDD combinations with mandatory support for simultaneous Rx\TX, RAN4 considers two modes of transmissions:

- Mode 1: single UL carrier transmissions and corresponding 1UL CC MSD requirements are specified.

- Example: PC3 Band n41 UL2/DL3 harmonic mixing MSD due to Band n77 1UL CC transmission for CA_n41A-n77A (Table 7.3A.4-4).

- Mode 2: dual-UL inter-band CA transmissions and corresponding 2UL MSD requirements are specified.

- Example: PC3 Band n1 IMD4 MSD due to 2UL Band n41 + n77 transmission for CA_n41A-n77A (Table 7.3A.5-2).

It is not clear from reading the RMC in the Annex of TS 38.101-1, or by reading the TS 38.521-1, how either mode of transmission is configured. It seems no agreements were reached on say, a dedicated slot structure for each mode of transmission. It is not clear either if special scheduling configurations were agreed, hereby raising questions on how are the 1UL and 2UL MSD requirements verified for such band combinations (example CA_n41-n77 and CA_n1-n41-n77).

Observation 3: For inter-band CA TDD-TDD combinations with mandatory support for simultaneous Rx\TX, RAN4 considers two UL configurations:

- 1UL CC configurations and the corresponding 1UL CC MSD requirements should be always analysed and specified when applicable [1,2]. In this mode, it is assumed the UE is configured so that only single carrier transmissions occur in one band at any moment of time,

- 2UL inter-band CA configurations, a mode in which it is assumed that the UE is configured so that dual uplink transmissions occur simultaneously all the time. The corresponding 2UL MSD requirements should be systematically analysed and when applicable specified.

- It is not clear how both this UE capability and the corresponding MSD requirements are verified using RAN4 specified RMC or by RAN5 test procedures.

In [3] it was proposed to send an LS to RAN5 to clarify the testing procedure and TDD patterns. Here we re-iterate the need to invite interested companies to verify how this UE capability is verified and how the corresponding 1UL and 2UL MSD requirements are verified.

Proposal 3: Interested companies are invited to check how the UE mandatory support for simultaneous Rx\TX for TDD-TDD band combinations can be verified using the existing TS 38.101-1 RMC and how the corresponding 1UL CC MSD and 2UL inter-band MSD requirements are verified. Consider sending an LS to RAN5 to clarify the testing procedures.

# Conclusion

This contribution summarizes our views on clarifying and simplifying the requirements for support of the simultaneous Rx\TX UE capability based on observations from [1,2]. We also re-iterate the question from [3] on how such capability is verified using either RAN4 RMC or RAN5 testing procedures. Beyond the verification of this UE capability, the question is also to understand how are the 1UL CC MSD requirements and the 2UL inter-band MSD requirements verified. A companion CR is presented at this meeting [4].

Observation 1:

- 19 footnotes related to simultaneous Rx\Tx are captured in 16 different tables across several clauses: 5.2A, 5.5A, 6.2A, 7.3A. It is difficult to clarify the exact status of a given band combination, a single point of entry into TS 38.101-1 would be welcome.

- Many notes are redundant.

- According to the clause 5.2A.2 core requirements, it seems that once specified for a lower order band combination, the requirements on this capability are no longer needed for any higher order band combination built upon this lower order “fall-back” combination.

Observation 2:  Based on Observation 1 relative to the core requirements of clause 5.2A.2, the following simplifications are sufficient:

- For CA_n77-n78, CA_n77-n79. For Table 5.2A.2.1: “The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration”

- For CA_n78-n79:

  - Note 5 of Table 5.2A.2.1 could be simplified to “The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers for UEs supporting band 78 with a n77 implementation. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration”.

  - We observe a discrepancy between this Note 5 and the footnotes of tables 6.2A.x and 7.3A.x. It seems that the following wording is missing in Note 5 of Table 5.2A.2.1 and should be added: “The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability”.

Proposal 1: To simplify and clarify the requirements on the simultaneous Rx/Tx conditional mandatory capability:

- Void all related footnotes from all tables of TS 38.101-1,

- Remove all footnotes from CA combinations. Example remove Notes 1,5,7,9,13,15 from column “NR CA Band” in Table 5.2A.2.1-1,

- Capture the requirements using a new column in Table 5.2A.2.1-1 entitled “UE support for simultaneous Rx/Tx (Simultaneous RxTx)”, similar to the column that capture the “DL interruption allowed” requirements.

Proposal 2:

- Normalize the entries to new column “UE support for simultaneous Rx/Tx (SimRxTx)” in Table 5.2A.2.1-1 to only four cases:

  - “Empty or blank” for optional conditional SimRxTx support

  - “Mandatory Simultaneous RxTx” for note 1

  - Tentatively “[Mandatory Simultaneous RxTx except for band 78 supported with n77 implementation]” for CA_n78A-n79A. Companies are encouraged to resolve the discrepancies of observation 2.

  - “Simultaneous RxTx is not supported” for note 5, 7 and note 9.

- Introduce the following core requirement text in clause 5.2A.2.

“5.2A.2 Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, Table 5.2A.2.2-1, Table 5.2A.2.3-1, Table 5.2A.2.4-1 and Table 5.2A.2.5-1, where all operating bands are within FR1.

The mandatory simultaneous Rx/Tx capability is specified in Table 5.2A.2.1-1. CA configurations which do not have mandatory simultaneous Rx/Tx specified and for which the UE may signal optional support for simultaneous Rx/Tx shall be captured with an empty entry. If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.”

An illustration of the simplification scheme is presented in Table 2 and companion CRs are proposed at this meeting.

Observation 3: For inter-band CA TDD-TDD combinations with mandatory support for simultaneous Rx\TX, RAN4 considers two UL configurations:

- 1UL CC configurations and the corresponding 1UL CC MSD requirements should be always analysed and specified when applicable [1,2]. In this mode, it is assumed the UE is configured so that only single carrier transmissions occur in one band at any moment of time,

- 2UL inter-band CA configurations, a mode in which it is assumed that the UE is configured so that dual uplink transmissions occur simultaneously all the time. The corresponding 2UL MSD requirements should be systematically analysed and when applicable specified.

- It is not clear how both this UE capability and the corresponding MSD requirements are verified using RAN4 specified RMC or by RAN5 test procedures.

Proposal 3: Interested companies are invited to check how the UE mandatory support for simultaneous Rx\TX for TDD-TDD band combinations can be verified using the existing TS 38.101-1 RMC and how the corresponding 1UL CC MSD and 2UL inter-band MSD requirements are verified. Consider sending an LS to RAN5 to clarify the testing procedures.

# References

- R4-2320020 Discussion on Simultaneous Rx/Tx, 3GPP TSG-RAN WG4 Meeting #109, Athens, Greece, Nokia, Nokia Shanghai Bell.

- R4-2409192 Discussion on Simultaneous RxTx, 3GPP TSG-RAN WG4 Meeting #109, Fukuoka, Japan, Nokia.

- R4-2315377 On simultaneous Rx/Tx testing, 3GPP TSG-RAN WG4 Meeting#108-bis, Xiamen, China, Apple.

- R4-2501944 (NR_CADC_SUL_R19-Core) Cat F CR for TS 38.101-1-j00 Simultaneous RxTx clean-up, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia.