3GPP TSG-RAN WG4 Meeting #114bis 	R4-2503192

Wuhan, China,  7th April – 11th April 2025

Agenda item:	7.8.2

Source:	Apple

Title:	On the introduction of 7MHz CBW

WI/SI:	NR_FR1_7MHz_BW-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce 7MHz channel bandwidth for n26 and n5. For the initial discussion this contribution focuses on selected objectives from the WID. Before any further analysis can be made it is important to specify the spectrum utilization. Another important objective is to determine the impact on network signaling label so that the A-MPR work can be started.

# 2  Discussion

##### 2.1 Simulation setup and Tx impairments

The following assumptions and requirements for the simulations are applied:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

##### 2.2 Introduction of 7MHz CBW to NS_12

| Channel location | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| Channel at lower band edge (fc: 817.5MHz) |  |  |

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.88 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

Proposal 1: Consider Table 1 when specifying A-MPR for NS_12

##### 2.3 Introduction of 7MHz CBW to NS_15

| Channel location | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| Channel at upper band edge (fc: 845.5MHz) |  |  |
| Shifted by 5.5MHz from upper band edge (fc: 840MHz) |  |  |
| Shifted by 7MHz from upper band edge (fc: 838.5MHz) |  |  |

Table 3: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7MHz | 840 < Fc ≤ 845.5 | ≥4.68 | >0 | A1 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A2 |
|  |  | <1.8 | ≤0.36 | A3 |
|  | 837 < Fc ≤ 840 | ≥0 | ≥2.7 | A7 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |

Table 4: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK |  | ≤ 9 |  | ≤ 5 |  | ≤ 4 |  | ≤ 9 |  | ≤ 5 |  | 1 |  | 3.0 |  |
| DFT-s-OFDM QPSK |  | ≤ 9 |  | ≤ 5 |  | ≤ 4 |  | ≤ 9 |  | ≤ 5.5 |  | 1.5 |  | 3.5 |  |
| DFT-s-OFDM 16 QAM |  | ≤ 9 |  | ≤ 5 |  | ≤ 4 |  | ≤ 9 |  | ≤ 5.5 |  | 2 |  | 4.0 |  |
| DFT-s-OFDM 64 QAM |  | ≤ 9 |  | ≤ 5 |  | ≤ 4 |  | ≤ 9 |  | ≤ 5.5 |  |  |  | 4.0 |  |
| DFT-s-OFDM 256 QAM |  | ≤ 9 |  | ≤ 5 |  | ≤ 9 |  | ≤ 13.5 |  | ≤ 6 |  |  |  | 4.5 |  |
| CP-OFDM QPSK |  | ≤ 10.5 |  | ≤ 6.5 |  | ≤ 4 |  | ≤ 10.5 |  | ≤ 7 |  | 3 |  | 5.5 |  |
| CP-OFDM 16 QAM |  | ≤ 10.5 |  | ≤ 6.5 |  | ≤ 4 |  | ≤ 10.5 |  | ≤ 7 |  | 3 |  | 5.5 |  |
| CP-OFDM 64 QAM |  | ≤ 10.5 |  | ≤ 6.5 |  | ≤ 4 |  | ≤ 10.5 |  | ≤ 7 |  |  |  | 5.5 |  |
| CP-OFDM 256 QAM |  | ≤ 10.5 |  | ≤ 6.5 |  | ≤ 9 |  | ≤ 13.5 |  | ≤ 7.5 |  |  |  | 6.5 |  |

Proposed RB allocation regions:

| 840 < Fc ≤ 845.5 | 837 < Fc ≤ 840 |
| --- | --- |

Proposal 2: Consider Table 3 and Table 4 when defining A-MPR for NS_15

# 4	Conclusions

This contribution provides initial insight and discussion points for the introduction of 7MHz channel. The following proposals are made:

Proposal 1: Consider Table 1 when specifying A-MPR for NS_12

Proposal 2: Consider Table 3 and Table 4 when defining A-MPR for NS_15

# 4	References

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA, 3GPP TSG RAN Meeting #106

- R4-2502870, “WF on requirements for 7MHz channel bandwidth”, Ericsson, 3GPP TSG-RAN4 Meeting #114