3GPP TSG-RAN WG4 Meeting #114bis	R4-2503249

# Wuhan, China, 7th - 11th, April 2025

Source: 	SK Telecom

Title: 	Discussion on HPUE inter-band UL CA and EN-DC

Agenda Item:	7.1.1.1.3

Document for:	Discussion

- Introduction

This contribution presents our views on several remaining issues related to inter-band UL CA and EN-DC with 2Tx and/or 3Tx, based on the agreements reached during the RAN#113 meeting [1].

- Configurations Agreement: - To create and maintain a configuration table listing all configurations can be considered in Rel-19 in next meeting, for information purpose - FFS specific MSD calculation procedure details for the band combination including PC2 FDD as a constituent band (details in section 4.1) - No need to enter such combination into the WID - Re-organize 2Tx/3Tx Notes in clause 5 and clause 6 Agreement: - Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings

- 2. Discussion

2.1 Configurations Table

We think it is meaningful to provide a table listing all configurations discussed in Release 19 for information purposes. However, including detailed requirements could make it difficult to structure the table properly. Therefore, it would be more appropriate to include general requirements for band combinations as much as possible, which will require further discussion. The configuration tables proposed in [2] and [3] could serve as a good starting point.

Proposal 1: It is necessary to maintain the configuration table at a general level to ensure clarity and manageability.

2.2 Re-organize 2Tx/3Tx Notes

We agree with the motivation for re-organizing the 2Tx/3Tx notes, as there are redundancies in the current descriptions with the evolution of the specification. During the reorganization, it should be ensured that operators’ requests and the WI scope are properly reflected. For example, the WI “UE RF enhancements for NR FR1/FR2 and EN-DC” includes 3Tx handheld UE, which should be clearly specified during the reorganization. Removing Note 7 in Table 6.2H.1.3-1 and Table 6.2L.1.3-1 of TS 38.101-3 could be one way to address this.

Table 6.2H.1.3-1: Maximum output power for inter-band EN-DC with UL MIMO (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_7A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_20A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A | 26 | +2/-3 | 23 | +2/-3 |
| DC_41A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO in NR band NOTE 6: VoidNOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

Table 6.2L.1.3-1: Maximum output power for inter-band EN-DC with Tx Diversity (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_7A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_20A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| DC_41A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with Tx Diversity in NR band NOTE 6: Void NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

Proposal 2: 3Tx for handheld UE should be properly reflected in the note reorganization.

- 3. Conclusion

Proposal 1: It is necessary to maintain the configuration table at a general level to ensure clarity and manageability.

Proposal 2: 3Tx for handheld UE should be properly reflected in the note reorganization.

- 4. Reference

[1] R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113

[2] R4-2500317, On Rel-19 HPUE inter-band UL CA DC configurations, Apple, RAN4#114

[3] R4-2501686, HPUE for inter-band UL CA and EN-DC, LG Electronics, RAN4#114

[4] RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), RAN#103