3GPP TSG-RAN4 Meeting #114bis					          	       R4-2503275

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Tejas Networks

Title: 	Discussion on BS demodulation performance requirement for MMSE-IRC

Agenda Item:	7.18.3

Document for: 	Discussion

# Introduction

In the RAN4 #114 meeting, RAN4 discussed interference modelling, test parameters, and simulation assumptions for the MMSE-IRC receiver, and the WF document [1] was agreed upon. In this contribution, we provide our views on the remaining open issues according to [1] and simulation results for MMSE-IRC performance analysis.

# Discussion

## Interference Modelling

Number of interferers for 2Rx

  - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx.

  - For the number of interferers for 2Rx under HetNet, further decide between the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

  - For HomNet scenario, further decide among the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

- Option 3: Not to consider 2Rx requirements for HomNet scenario.

  - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

The WF document [1] suggests the following options for defining the number of interferers for the 2Rx scenario:

To highlight the impact of the number of interferers on 2Rx, simulations were performed for a 1-layer, 2-Rx scenario under single and dual interference circumstances. Table 1 illustrates the performance gain of MMSE-IRC over MMSE in the HetNet scenario, while Table 2 demonstrates the performance gain for the HomNet scenario.

Table 1: PUSCH performance gain for HetNet scenario

| Case | SCS | BW | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz | 5MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 17.91 | 14.61 | 3.3 |
| 2 | 15KHz | 5MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 19.02 | 15.61 | 3.41 |
| 3 | 15KHz | 5MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 21.97 | 16.32 | 5.65 |
| 4 | 15KHz | 5MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.88 | 18.03 | 4.85 |
| 5 | 15KHz | 20MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.29 | 14.61 | 3.68 |
| 6 | 15KHz | 20MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.63 | 15.14 | 3.49 |
| 7 | 15KHz | 20MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.27 | 16.21 | 6.06 |
| 8 | 15KHz | 20MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.66 | 16.84 | 5.82 |
| 9 | 30KHz | 10MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 17.42 | 14.66 | 2.76 |
| 10 | 30KHz | 10MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.24 | 14.4 | 3.84 |
| 11 | 30KHz | 10MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 21.31 | 16.21 | 5.1 |
| 12 | 30KHz | 10MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.23 | 16.4 | 5.83 |
| 13 | 30KHz | 100MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.51 | 14.76 | 3.75 |
| 14 | 30KHz | 100MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 19.05 | 15.39 | 3.66 |
| 15 | 30KHz | 100MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.32 | 16.62 | 5.7 |
| 16 | 30KHz | 100MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 23.01 | 17.61 | 5.4 |

Table 2: PUSCH performance gain for HomNet scenario

| Case | SCS | BW | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz | 5MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.06 | 13.05 | 1.01 |
| 2 | 15KHz | 5MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 15.22 | 13.75 | 1.47 |
| 3 | 15KHz | 5MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 17.76 | 14.47 | 3.29 |
| 4 | 15KHz | 5MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.86 | 15.53 | 3.33 |
| 5 | 15KHz | 20MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.43 | 12.67 | 1.76 |
| 6 | 15KHz | 20MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.91 | 13.06 | 1.85 |
| 7 | 15KHz | 20MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.21 | 14.49 | 3.72 |
| 8 | 15KHz | 20MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.55 | 15.07 | 3.48 |
| 9 | 30KHz | 10MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.04 | 13.12 | 0.92 |
| 10 | 30KHz | 10MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.48 | 12.61 | 1.87 |
| 11 | 30KHz | 10MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 17.41 | 14.65 | 2.76 |
| 12 | 30KHz | 10MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.14 | 14.41 | 3.73 |
| 13 | 30KHz | 100MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.82 | 13.05 | 1.77 |
| 14 | 30KHz | 100MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 15.36 | 13.65 | 1.71 |
| 15 | 30KHz | 100MHz | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.39 | 14.7 | 3.69 |
| 16 | 30KHz | 100MHz | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.99 | 15.36 | 3.63 |

Proposal 1: For HetNet scenario, we prefer option 1, which utilizes the two-interferer case to specify performance requirements for 2Rx, but are open to consider option 2 based on consensus.

Proposal 2: For HomNet scenario, we support option 1, incorporating the two-interferer case to define performance requirements for 2Rx, however open to consider option 2 based on consensus.

## Test parameters and simulation assumptions

Propagation Condition for serving cell

•	Select between the following options:

–	Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet

–	Option 2: TDLC300-100 for all cases

–	Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2)

In the WF document [1], the following candidate options are proposed for the propagation condition of serving cells:

As TDL-C models severe multi-path channel environment, it is not well-suited for practical HetNet deployments, which typically require lower delay spreads. Therefore, we support TDLA30-10 for HetNet scenario. In contrast, HomNet scenario requires severe multi-path channel with higher delay-spread, making TDL-C a suitable choice. Thus, we support TDLC300-100 for the HomNet scenario.

Proposal 3: For Propagation condition of serving cell, we support option 3 to consider TDLA30-10 for HetNet and TDLC300-100 for HomNet.

# Simulation Results

In this section, simulation results are provided to analyse the performance gain of the MMSE-IRC receiver over MMSE based on the parameters agreement made in the WF document [1].

Table 3: PUSCH Performance Gain for SCS:15KHz and BW:5MHz

| Case | Tx | Rx | Mapping Type | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 17.91 | 14.61 | 3.3 |
| 2 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 19.02 | 15.61 | 3.41 |
| 3 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 21.97 | 16.32 | 5.65 |
| 4 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.88 | 18.03 | 4.85 |
| 5 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.06 | 13.05 | 1.01 |
| 6 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 15.22 | 13.75 | 1.47 |
| 7 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 17.76 | 14.47 | 3.29 |
| 8 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.86 | 15.53 | 3.33 |
| 9 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 14.5 | 10.83 | 3.67 |
| 10 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 15.87 | 11.09 | 4.78 |
| 11 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 18.49 | 14.09 | 4.4 |
| 12 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 19.9 | 14.84 | 5.06 |
| 13 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 10.81 | 8.51 | 2.3 |
| 14 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 11.8 | 8.56 | 3.24 |
| 15 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 14.43 | 10.82 | 3.61 |
| 16 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 15.77 | 11 | 4.77 |
| 17 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 12.83 | 9.07 | 3.76 |
| 18 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.03 | 9.1 | 3.93 |
| 19 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.04 | 13.09 | 3.95 |
| 20 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.3 | 13.08 | 4.22 |
| 21 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 8.75 | 5.32 | 3.43 |
| 22 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 8.78 | 5.38 | 3.4 |
| 23 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 12.81 | 8.97 | 3.84 |
| 24 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 12.91 | 9.02 | 3.89 |

Table 4: PUSCH Performance Gain for SCS:15KHz and BW:20MHz

| Case | Tx | Rx | Mapping Type | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.29 | 14.61 | 3.68 |
| 2 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.63 | 15.14 | 3.49 |
| 3 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.27 | 16.21 | 6.06 |
| 4 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.66 | 16.84 | 5.82 |
| 5 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.43 | 12.67 | 1.76 |
| 6 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.91 | 13.06 | 1.85 |
| 7 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.21 | 14.49 | 3.72 |
| 8 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.55 | 15.07 | 3.48 |
| 9 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 14.54 | 10.3 | 4.24 |
| 10 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 15.49 | 10.55 | 4.94 |
| 11 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 18.52 | 13.77 | 4.75 |
| 12 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 19.57 | 13.97 | 5.6 |
| 13 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 10.76 | 8.8 | 1.96 |
| 14 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 11.62 | 8.22 | 3.4 |
| 15 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 14.43 | 10.2 | 4.23 |
| 16 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 16.4 | 10.48 | 5.92 |
| 17 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.4 | 8.21 | 5.19 |
| 18 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.21 | 8.3 | 4.91 |
| 19 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.52 | 11.97 | 5.55 |
| 20 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.37 | 12.08 | 5.29 |
| 21 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.22 | 4.67 | 4.55 |
| 22 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.08 | 4.96 | 4.12 |
| 23 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 13.28 | 8.07 | 5.21 |
| 24 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 13.09 | 8.21 | 4.88 |

Table 5: PUSCH Performance Gain for SCS:30KHz and BW:10MHz

| Case | Tx | Rx | Mapping Type | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 17.42 | 14.66 | 2.76 |
| 2 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.24 | 14.4 | 3.84 |
| 3 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 21.31 | 16.21 | 5.1 |
| 4 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.23 | 16.4 | 5.83 |
| 5 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.04 | 13.12 | 0.92 |
| 6 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.48 | 12.61 | 1.87 |
| 7 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 17.41 | 14.65 | 2.76 |
| 8 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.14 | 14.41 | 3.73 |
| 9 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 14.37 | 9.23 | 5.14 |
| 10 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 15.68 | 10.16 | 5.52 |
| 11 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 18.34 | 12.14 | 6.2 |
| 12 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 19.67 | 12.91 | 6.76 |
| 13 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 10.5 | 7.57 | 2.93 |
| 14 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 11.66 | 8.15 | 3.51 |
| 15 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 14.27 | 9.21 | 5.06 |
| 16 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 15.58 | 10.1 | 5.48 |
| 17 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.04 | 6.93 | 6.11 |
| 18 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.28 | 6.93 | 6.35 |
| 19 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.16 | 8.23 | 8.93 |
| 20 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.43 | 8.34 | 9.09 |
| 21 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.01 | 4.94 | 4.07 |
| 22 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.13 | 4.41 | 4.72 |
| 23 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 12.91 | 6.83 | 6.08 |
| 24 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 13.2 | 6.83 | 6.37 |

Table 6: PUSCH Performance gain for SCS:30KHz and BW:100MHz

| Case | Tx | Rx | Mapping Type | MCS for target cell | MCS for interference cell | Propagation Condition for target cell | Propagation Condition for interference cell | Number of Interference | DIP Set | MMSE (SNR) | MMSE-IRC (SNR) | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 18.51 | 14.76 | 3.75 |
| 2 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 19.05 | 15.39 | 3.66 |
| 3 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 22.32 | 16.62 | 5.7 |
| 4 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 23.01 | 17.61 | 5.4 |
| 5 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 14.82 | 13.05 | 1.77 |
| 6 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 15.36 | 13.65 | 1.71 |
| 7 | 1 | 2 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.39 | 14.7 | 3.69 |
| 8 | 1 | 2 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 18.99 | 15.36 | 3.63 |
| 9 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 15.39 | 10.05 | 5.34 |
| 10 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 15.81 | 10.17 | 5.64 |
| 11 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 19.41 | 12.54 | 6.87 |
| 12 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 19.86 | 12.96 | 6.9 |
| 13 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 11.55 | 8.55 | 3 |
| 14 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 11.91 | 8.43 | 3.48 |
| 15 | 1 | 4 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 15.27 | 10.02 | 5.25 |
| 16 | 1 | 4 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 15.72 | 10.08 | 5.64 |
| 17 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.65 | 5.88 | 7.77 |
| 18 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HetNet Set2: (-0.43, NA/) | 13.65 | 6.09 | 7.56 |
| 19 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.81 | 8.01 | 9.8 |
| 20 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HetNet Set2: (-0.43, -13.78) | 17.75 | 8.25 | 9.5 |
| 21 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.54 | 4.53 | 5.01 |
| 22 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 1 | HomNet Set1: (-1.11, NA/) | 9.57 | 4.68 | 4.89 |
| 23 | 1 | 8 | A | 16 | 16QAM | TDL-A 30 10 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 13.56 | 5.82 | 7.74 |
| 24 | 1 | 8 | A | 16 | 16QAM | TDL-C 300 100 | TDL-C 300 100 | 2 | HomNet Set1: (-1.11, -10.91) | 13.56 | 6.09 | 7.47 |

These simulation results demonstrate the superior interference suppression capability of the MMSE-IRC receiver over the conventional MMSE in NR scenarios.

# Conclusion

In this contribution, we have given our views and proposals on the remaining issues of MMSE-IRC performance requirement for BS demodulation along with simulation results.

Proposal 1: For HetNet scenario, we prefer option 1, which utilizes the two-interferer case to specify performance requirements for 2Rx, but are open to consider option 2 based on consensus.

Proposal 2: For HomNet scenario, we support option 1, incorporating the two-interferer case to define performance requirements for 2Rx, however open to consider option 2 based on consensus.

Proposal 3: For Propagation condition of serving cell, we support option 3 to consider TDLA30-10 for HetNet and TDLC300-100 for HomNet.

# Reference

- R4-2502272, “Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN#114, Athens, Greece.

- RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.