3GPP RAN WG4 Meeting #114bis	R4-2503413

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on testability and interoperability issues for AIML mobility

Source: 	CATT

Agenda item:	7.21.3

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 continued to discuss the AI/ML mobility SI and the agreements were captured in [1]. In this contribution, we will further discuss the impacts of testability and interoperability of AI/ML mobility.

Discussion

Testing goals

In last meeting, RAN4 agreed a high-level principle for testing goal:

RAN4#113: - Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

However, the testing goal needs to be further discussed for each scenario based on the above principle. Actually RAN2 has clarified study goals for all use cases and in simulation assumption the evaluation metrics are also given:

TR 38.744 The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption). … Measurement reduction rate for intra-frequency scenario is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS): MRRT = skipped measurement time instances / total measurement time instances MRRS = skipped beams to be measured/ total beams to be measured

Apparently, the motivation of this feature is to save measurement efforts or to improve HO performance. Therefore for scenario 2 and 3, the aspect that matters is the measurement efforts that UE reduces. However, given that the MRRT or MRRS may be a fixed value when measurement configuration signalling is indicated to UE, RAN4 can test whether the prediction results satisfy the existing measurement accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

For scenario 4, RAN4 needs to verify the HO performance as it is highlighted above. The issue is the metrics for HO performance evaluation is not defined in RAN4. The only requirement that is related to handover is the handover delay requirement. Thus, if RAN2’s purpose is to shorten the handover delay/interruption, then current RAN4 handover delay requirement can be used as a starting point. But if RAN2 try to reduce other aspects, such as Ping-pong HO, HOF/RLF, then RAN4 needs to further study how to evaluate HO performance in RAN4 test and the feasibility of defining metrics based on ping-pong HO, etc.

Proposal 2: For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss how to define HO performance metric.

Testing in FR1

In RAN#112bis meeting, it was agreed to adopt conducted testing method for AI/ML mobility test in FR1 and consider at least the following aspects for this SI:

	-	Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3)

	-	Testing cell-level L3-RSRP measurements

	-	Consideration of serving and neighbor cells

First, we would like to discuss how to make DUT obtain cell-level L3-RSRPs at model input in FR1 conducted test. In general, there are two methods: (1) configure only one SSB resource for each serving/neighbor cell and DUT performs RSRP measurements, and the results are treated as cell-level RSRPs which then can be used as model input for further predictions. (2) configure multiple SSB resources for each serving/neighbor cell and the RSRP results are regarded as beam-level RSRPs, and then DUT calculates cell-level RSRPs for all cells based on these beam-level RSRPs. A figure illustrating these two methods is provided in Figure 1. Considering the test will be carried out in a conducted manner and RAN4 agrees to prioritize scenario 2, 3 and 4 which focus on saving measurement efforts in temporal or frequency domain, method (1), in our opinion, is sufficient to test DUT’s capability of AI/ML-enabled RRM measurement prediction.

Second, a more important issue is how to model a time-variant RSRPs to reflect the changes in temporal, frequency or spatial domain. Based on method (1), TE can randomly adjust the transmitting power of SSBs in all cells, DUT should measure these SSBs at time instance (t1) and predict cell-level RSRPs of all cells at time instance (t2) and report them to TE for verification. Since the transmitting power of SSBs is controlled by TE, the ground truth should be known to TE. Note that the adjusted transmitting power should lead to a reasonable range of SSB receiving power at DUT for which the AI/ML models can still work, otherwise the models may perform badly.

Regarding channel model, since the prioritized scenarios in RAN4 do not involve predictions in spatial domain, AWGN or TDL channel can serve the purpose, which do not require upgrade on existing test equipment.

Figure 1. Setting up cell-level RSRP in FR1 conducted test

Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 4: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS.

Proposal 5: AWGN or TDL channel can be used in FR1 conducted test.

For number of cells configured in FR1 test, the values for UE capability in current RRM spec can be used as a starting point, i.e., for 8 cells for intra-frequency temporal domain cases and 4 cell for inter-frequency temporal domain cases, when HO performance based on predictions is tested. For measurement effort reduction purpose, less cells can be set up in test. And one extra serving cell should also be included. In addition, the values of carrier frequencies for inter-frequency predictions can be determined in a later stage.

Proposal 6: The following values can be a baseline for the number of configured cells in FR1 HO performance test:

- [9] cells for intra-frequency predictions;

- [5] cells for inter-frequency predictions;

- And values of carrier frequencies can be determined later.

Proposal 7: For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

Testing in FR2

Some proposals about testing in FR2 were also brought up in last meeting and a summarized list that RAN4 should discuss for this SI was provided and it is copied below:

- The following new aspects for testing in FR2 can be considered by interested companies: - Number of AoAs in AI/ML based mobility tests, i.e., if more than 3 AoA are needed - Relationship between SSB signals and AoAs when a single AoA is multiplexed by multiple cells - Maximum numbers of Tx beam that has to be configured - Which fading channel should be used - Number of cells to measurement - Whether to consider one probe to emulate different Tx beams by applying different power - Other aspects are not precluded

Apparently, many aspects are quite similar to the issues being discussed in AI/ML-based beam management. Hence, in order to avoid parallel discussions, test set up in FR2 for this SI should be postponed until sufficient progress in AI/ML BM is made and then the corresponding agreements can be reused.

Proposal 8: RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM.

Testability of inter-frequency scenario

Inter-frequency prediction is a slightly special case since UE predicts the RSRP of carrier 1 based on the measurement result of carrier 2. Some conditions need to be met if a good performance of this mechanism is expected. One of the key aspects is these two cells needs to be co-located. How to reflect their colocation in test is a main problem. Since test up in FR2 is preferred to be deprioritized, the following discussion focuses on FR1 only.

First, both carriers belong to the same frequency range, which means they experience a similar propagation condition over the air, e.g., they probably have a similar pathloss. If reference signals carried by these two carriers are transmitted with the same power, the RSRP at the DUT may be similar as well. Hence, a straightforward method is to reflect the co-location relationship by a minor RSRP difference between reference signals carried by co-located carriers.

Proposal 9: Colocation can be reflected by a minor RSRP difference between two carriers.

Conclusions

This contribution discussed the testability and interoperability issues in AI/ML mobility and the following proposals are provided:

Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

Proposal 2: For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss how to define HO performance metric.

Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 4: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS.

Proposal 5: AWGN or TDL channel can be used in FR1 conducted test.

Proposal 6: The following values can be a baseline for the number of configured cells in FR1 HO performance test:

- [9] cells for intra-frequency predictions;

- [5] cells for inter-frequency predictions;

- And values of carrier frequencies can be determined later.

Proposal 7: For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

Proposal 8: RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM.

Proposal 9: Colocation can be reflected by a minor RSRP difference between two carriers.

References

[1] R4-2502594, WF on AI/ML Mobility, Nokia, RAN4#114.