3GPP TSG-RAN4 Meeting #114-bis	R4- 2503719

# Wuhan, China, 6th Apr 2025 – 11th Apr 2025

Source:	LG Electronics

Title:	Discussion for UE RF Tx requirements of NTN_NR_HPUE

Agenda item:	7.10.2.2.1

Document for:	Discussion

Introduction

In RAN4#114 meeting, WF on NR NTN HPUE requirements was approved. Based on the WF, we would like to share our  view for the requirements.

Discussion

Table 1 : Way forward for UE RF requirements for NTN_NR_HPUE [1]

Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19. Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202 Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

Tx configuration according to PC

Generally, NTN UE is assumed to be not different from TN UE. Accordingly, most of the NTN UE requirements defined in Rel-17 reuse the TN UE RF requirements. This can also be applied to NTN HPUE. Therefore, the HPUE RF requirements defined in TN can be used similarly for NTN.

Talbe 2 shows the Tx configurations of NR TN HPUE, and Table 3 shows the Tx configuration candidates of NR NTN HPUE.

HH: handheld, NH: non-handheld

Table 2 : Tx configurations of current TN spec according to the power class

|  | 1Tx |  | 2Tx |  | 4Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD | FDD | TDD |
| PC3 | O (NH and HH) | O (NH and HH) | O (NH and HH) | O (NH and HH) |  |  |
| PC2 | O (NH and HH) | O (NH and HH) | O (NH and HH) | O (NH and HH) |  |  |
| PC1.5 |  |  |  | O (NH and HH) |  | O (NH) |
| PC1 | O (NH) | O (NH) |  |  |  |  |

Table 3 Tx configuration candidates of NTN according to the power class

|  | 1Tx |  | 2Tx |  | 4Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD | FDD | TDD |
| PC3 | O (NH and HH) |  |  |  |  |  |
| PC2 | O (NH and HH) |  | O (NH and HH) |  |  |  |
| PC1.5 | O our proposal |  | O (NH) Other cases are not precluded |  |  |  |
| PC1 | O (NH) |  |  |  | [O] (NH) |  |

Our view on the NTN HPUE Tx configurations is presented below.

Power Class 2

Observation 1: The PC2 1Tx and 2Tx UE RF requirements have already been defined in TN, and it seems that most of the TN RF requirements can be reused. Therefore, it is necessary to define both 1Tx (non-handheld and handheld) and 2Tx (non-handheld and handheld).

Proposal 1: Specify the NTN PC2 RF requirements, for both 1Tx (both non-handheld and handheld) and 2Tx (both non-handheld and handheld).

power class 1.5

According to the WF [1], it was agreed to define the requirements for 2Tx non-handheld, and other configurations are not restricted. Unlike TN, NTN considers LOS environment, in which the benefits of TxD may be limited. Therefore, if only 2Tx PC1.5 is defined , it could mean that the link budget benefit of PC1.5 NTN HPUE might be minimal. The most effective way to maximize the benefit of HPUE NTN is to define 1Tx, which can help mitigate the lack of link budget.

Observation 2: Unlike TN, NTN considers LOS environment, in which the benefits of TxD may be limited. Therefore, if only 2Tx PC1.5 is defined, it could mean that the link budget benefit of PC1.5 NTN HPUE might be minimal. The most effective way to maximize the benefit of HPUE NTN is to define 1Tx, which can help mitigate the lack of link budget.

Proposal 2: Specify the the PC1.5 RF requirements, considering both 1Tx (non-handheld) and 2Tx(non-handheld).

power class 1

1Tx PC1 is already defined in TN, and it seems that it can also be applied to NTN PC1 UE. For 4Tx PC1, there are questions . Firstly, 4Tx PC1 is not defined in TN, which could result in a significant workload. Additionally, the benefit of NTN 4Tx PC1 may be minimal due to the LoS environment. Therefore, it is appropriate to consider only the 1Tx PC1 configuration.

Observation 3: There are questions regarding 4Tx PC1. Firstly, 4Tx PC1 is not defined in TN, which could result in a significant workload. Additionally, the benefit of NTN 4Tx PC1 may be minimal due to the LoS environment.

Proposal 3: specify the the PC1 RF requirements,considering 1Tx (non-handheld) only.

ACLR

ACLR is a critical performance metric in wireless communication systems, indicating the extent to which a transmitted signal leaks into adjacent channels. While NTN UE and TN UE may not differ significantly, the operational environment for NTN UE can be distinct from that of TN UE. Consequently, the ACLR values for NTN UE may need to be relaxed based on the co-existence study.

In the NTN environment, there may be less interference compared to TN enviroment. Therefore, it is reasonable to relax the ACLR values for NTN UE. This is because adjacent channel interference might be less critical in the NTN operation. Additionally, using lower ACLR values can allow for the relaxation of MPR requirements compared to TN. This relaxation can provide a link budget benefit, enhancing the overall performance and efficiency of the NTN-TN communication system.

Observation 4: In the NTN environment, there may be less interference compared to TN enviroment. Therefore, it is reasonable to relax the ACLR values for NTN UE. This is because adjacent channel interference might be less critical in the NTN operation. Additionally, using lower ACLR values can allow for the relaxation of MPR requirements compared to TN. This relaxation can provide a link budget benefit, enhancing the overall performance and efficiency of the NTN-TN communication system.

Proposal 4: Specify the smaller ACLR value for NTN HPUE compared to TN HPUE.

  - PC2: 30 dBc

  - PC1.5: 30 dBc

  - PC 1: 34 dBc

MPR

TN and NTN UEs are not separately distinguished and have a similar form factor. Therefore, if the emission requirements are the same, they can have the same MPR values, like PC3 NTN and TN UEs. However, since the ACLR requirement for NTN UEs can be more relaxed than that for TN UEs, the MPR values can be defined differently from TN MPR. Nevertheless, the relaxation of ACLR does not mean that the MPR values for all RB allocations can be relaxed. For example, the limiting factor for the outer RB allocation MPR could be ACLR, but the inner RB allocation MPR could be determined by in-band emission. Therefore, the MPR values for inner RB allocation can reuse the MPR values of TN UEs regardless of the ACLR values. Additionally, considering the current communication link budget between NTN and NTN UEs, it is difficult to operate at high-order modulation such as 64 and 256 QAM. Therefore, there is no need to study MPR specifications related to high-order modulation.

Observation 5: TN and NTN UE are not separately distinguished and have a similar form factor. Therefore, if the emission requirements are the same, they can have the same MPR values like PC3 NTN and TN UEs. However, since the ACLR requirement for NTN UE can be more relaxed than that for TN UE, the MPR values can be defined differently from TN MPR.

Observation 6: The MPR values for inner RB allocation can reuse the MPR values of TN UE regardless of the ACLR values.

Observation 7: Considering the current communication link budget between NTN and NTN UEs, it is difficult to operate at high-order modulation such as 64 and 256 QAM. Therefore, there is no need to study MPR specifications related to high-order modulation.

The currently considered MPR values for NTN HPUE can be defined according to the Power Class and Tx configuration as shown in Tables 4 to 7. The MPR values for Edge RB allocation and Outer RB allocation may vary depending on the ACLR values, and can be discussed once the ACLR values are determined. However, the MPR values for the Inner RB allocation area are not affected by ACLR and can reuse the existing TN MPR values.

Additionally, if the NTN HPUE 1Tx configuration is considered, Table 8 can be added. The MPR values for 1Tx tend to be smaller than those for 2Tx, as there is no additional leakage due to Tx signal leakage between antennas. In the case of TN PC2, the difference between the 1Tx MPR value and the 2Tx MPR value appears to be 0.5 dB, and this can be applied to the NTN HPUE PC1.5 1Tx MPR value as well.

Proposal 6: Use the MPR tables 4~8 as a starting point.

Table 4 For PC2 1Tx for both non-handheld and handheld UE

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS(3.5) | FFS(0.5) | 01 |
|  | QPSK | FFS(3.5) | FFS(1) | 02 |
|  | 16 QAM | FFS(3.5) | FFS(2) | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | FFS(3.5) | FFS(3) | ≤ 1.5 |
|  | 16 QAM | FFS(3.5) | FFS(3) | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 5 For PC2 2Tx for both non-handheld and handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | 0 |
|  | QPSK | FFS | FFS | 0.5 |
|  | 16 QAM | FFS | FFS | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | FFS | FFS | ≤ 2 |
|  | 16 QAM | FFS | FFS | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6 For PC1.5 2Tx non-handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | ≤ 0 |
|  | QPSK | FFS | FFS | ≤ 0 |
|  | 16 QAM | FFS | FFS | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | FFS | FFS | ≤ 1.5 |
|  | 16 QAM | FFS | FFS | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 7 For PC1 1Tx non-handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | FFS | FFS | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | FFS | FFS | 0 |
|  | QPSK | FFS | FFS | 0 |
|  | 16 QAM | FFS | FFS | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | FFS | FFS | ≤ 1.5 |
|  | 16 QAM | FFS | FFS | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |

Table 8 For PC1.5 1Tx non-handheld (if the 1Tx configuration for PC1.5 is considered)

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | ≤ [0] |
|  | QPSK | FFS | FFS | ≤ [0] |
|  | 16 QAM | FFS | FFS | ≤ [1] |
| CP-OFDM | QPSK | FFS | FFS | ≤ [1] |
|  | 16 QAM | FFS | FFS | ≤ [1.5] |

Conclusion

Observation 1: The relevant RF requirements have already been defined in TN, and it seems that most of the TN RF requirements can be reused. Therefore, it is necessary to define both 1Tx (non-handheld and handheld) and 2Tx (non-handheld and handheld).

Observation 3: There are questions regarding PC1 4Tx. Firstly, PC1 4Tx is not defined in TN, which could result in a significant workload. Additionally, the benefit of NTN PC1 4Tx diversity may be minimal due to the enviroment of NTN communication operating in a LoS environment.

Observation 4: In the NTN environment, there may be less interference compared to TN enviroment. Therefore, it is reasonable to relax the ACLR values for NTN UE. This is because adjacent channel interference might be less critical in the NTN operation. Additionally, using lower ACLR values can allow for the relaxation of MPR requirements compared to TN. This relaxation can provide a link budget benefit, enhancing the overall performance and efficiency of the NTN-TN communication system.

Observation 2: Unlike TN, NTN communication with UE occurs in a LOS environment, which means the benefits of TxD may be limited. Therefore, if PC1.5 is defined only for 2Tx, it could mean that the link budget benefit of PC1.5 NTN HPUE might be minimal. The most effective way to maximize the impact of HPUE NTN is to define 1Tx, which can help mitigate the lack of link budget.

Observation 5: TN and NTN UE are not separately distinguished and have a similar form factor. Therefore, if the emission requirements are the same, they can have the same MPR values like PC3 NTN and TN UEs. However, since the ACLR requirement for NTN UE can be more relaxed than that for TN UE, the MPR values can be defined differently from TN MPR.

Observation 6: The MPR values for inner RB allocation can reuse the MPR values of TN UE regardless of the ACLR values.

Observation 7: Considering the current communication link budget between NTN and NTN UE, it is hard to operate at the high order modulation such as 64 and 256 QAM. Therefore, there is no need to define MPR specifications related to high order modulation.

Proposal 1: Specify the NTN PC2 RF requirements, considering 1Tx (both non-handheld and handheld) and 2Tx (both non-handheld and handheld).

Proposal 2: specify the the PC1.5 RF requirements, considering both 1Tx (non-handheld) and 2Tx(non-handheld).

Proposal 3: specify the the PC1 RF requirements,considering 1Tx (non-handheld) only.

Proposal 4: Specify the smaller ACLR value for NTN HPUE compared to TN HPUE.

  - PC2: 30 dBc

  - PC1.5: 30 dBc

  - PC 1: 34 dBc

Proposal 6: Use the MPR tables 4~8 as a starting point.

Table 4 For PC2 1Tx for both non-handheld and handheld UE

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | 01 |
|  | QPSK | FFS | FFS | 02 |
|  | 16 QAM | FFS | FFS | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | FFS | FFS | ≤ 1.5 |
|  | 16 QAM | FFS | FFS | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 5 For PC2 2Tx for both non-handheld and handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | 0 |
|  | QPSK | FFS | FFS | 0.5 |
|  | 16 QAM | FFS | FFS | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | FFS | FFS | ≤ 2 |
|  | 16 QAM | FFS | FFS | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6 For PC1.5 2Tx non-handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | ≤ 0 |
|  | QPSK | FFS | FFS | ≤ 0 |
|  | 16 QAM | FFS | FFS | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | FFS | FFS | ≤ 1.5 |
|  | 16 QAM | FFS | FFS | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 7 For PC1 1Tx non-handheld

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | FFS | FFS | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | FFS | FFS | 0 |
|  | QPSK | FFS | FFS | 0 |
|  | 16 QAM | FFS | FFS | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | FFS | FFS | ≤ 1.5 |
|  | 16 QAM | FFS | FFS | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |

Table 8 For PC1.5 1Tx non-handheld (if the 1Tx configuration for PC1.5 is considered)

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | FFS | FFS | ≤ [0] |
|  | QPSK | FFS | FFS | ≤ [0] |
|  | 16 QAM | FFS | FFS | ≤ [1] |
| CP-OFDM | QPSK | FFS | FFS | ≤ [1] |
|  | 16 QAM | FFS | FFS | ≤ [1.5] |

- References

# [1] R4-2502954, WF on HPUE requirments for NR-NTN (Samsung)